3GPP TSG-RAN WG2 Meeting #132	R2-2508771

# Dallas, USA, 13th – 17th October 2025

Agenda Item:	8.7.2

Source:	Ericsson

Title:	Solution to Address RLC-H02

Document for:	Discussion, Decision

# 1	Introduction

The following agreement was made in the last meeting for one of the remaining RLC issues:

(RLC-H02) Confirm that when t-RxDiscard expires, RX_Next might be larger or equal than RX_Highest_Status. Companies are invited to bring contributions on the solutions to address it.

Thus, in this contribution, we provide a solution to address this issue.

# 2	Discussion

## 2.1 	RLC-H02

As pointed out in [1] and well illustrated in [2], there are scenarios when upon expiry of t-RxDiscard, the state variable RX_Next is higher than RX_Highest_Status thus violating the status reporting rule such that only SNs between RX_Next and RX_Highest_Status (i.e., RX_Next <= SN < RX_Highest_Status) that have not been completely received yet starting from RX_Next can be included in the status report. Then an agreement was made to confirm this behaviour and companies were invited to bring contributions to address this issue.

[1] also proposed a TP to update the status reporting mechanism text by incorporating t-RxDiscard. That is, if t-RxDiscard expiry did not trigger the status report, legacy procedures are followed. Else, only the ACK_SN is included in the status report. However, the proposed mechanism is a departure from the legacy procedure. No agreements were made to modify the legacy status reporting mechanism during the Rel-19 WI phase . In fact, one of the advantages in the Rx-based mechanism was that it is possible to largely reuse such legacy procedures. As a result, we believe solutions updating the legacy status reporting mechanisms should be avoided, particularly during the maintenance phase.

- We should avoid modifications to the legacy status reporting procedure as such enhancements were not agreed during the Rel-19 WI phase and must not be introduced in the maintenance phase.

As an alternative, we can draw inspiration from the polling mechanism as specified:

Polling from its peer AM RLC entity: - When an AMD PDU with SN = x and the P field set to "1" is received from lower layer, the receiving side of an AM RLC entity shall: - if the AMD PDU is to be discarded as specified in clause 5.2.3.2.2; or - if x < RX_Highest_Status or x >= RX_Next + AM_Window_Size: - trigger a STATUS report. - else: - delay triggering the STATUS report until x < RX_Highest_Status or x >= RX_Next + AM_Window_Size.

As seen from the highlighted text, when the received SN = x is higher than RX_Highest_Status, the status report is simply delayed until the condition is satisfied again.

Then, in the case of when t-RxDiscard expires, it should be possible to simply delay the status report until the state variables are updated to the conditions that satisfy the transmission of a status report. And in this case, it would be for RX_Next to be less than RX_Highest_Status i.e., RX_Next < RX_Highest_Status. Adopt the TP as in the Annex.

(RLC-H02) When RX_Next >= RX_Highest_Status, delay the triggering of the status report in the receiving side of an RLC AM entity until otherwise. Adopt the TP in the Annex.

# 3 	Conclusion

Based on the discussion in the previous sections, we have the following observations:

Observation 1	We should avoid modifications to the legacy status reporting procedure as such enhancements were not agreed during the Rel-19 WI phase and must not be introduced in the maintenance phase.

Based on the discussion in the previous sections, we propose the following:

Proposal 1	(RLC-H02) When RX_Next >= RX_Highest_Status, delay the triggering of the status report in the receiving side of an RLC AM entity until otherwise. Adopt the TP in the Annex.

# 4	Annex, TP

### 5.3.4 Status reporting An AM RLC entity sends STATUS PDUs to its peer AM RLC entity in order to provide positive and/or negative acknowledgements of RLC SDUs (or portions of them). Triggers to initiate STATUS reporting include: - Polling from its peer AM RLC entity: - When an AMD PDU with SN = x and the P field set to "1" is received from lower layer, the receiving side of an AM RLC entity shall: - if the AMD PDU is to be discarded as specified in clause 5.2.3.2.2; or - if x < RX_Highest_Status or x >= RX_Next + AM_Window_Size: - trigger a STATUS report. - else: - delay triggering the STATUS report until x < RX_Highest_Status or x >= RX_Next + AM_Window_Size. NOTE 1: This ensures that the RLC Status report is transmitted after HARQ reordering. - Detection of reception failure of an AMD PDU - The receiving side of an AM RLC entity shall trigger a STATUS report when t-Reassembly expires. - Detection of discard of an AMD PDU, when t-RxDiscard expires: - If RX_Next < RX_Next_Highest - The receiving side of an AM RLC entity shall trigger a STATUS report - else: - The receiving side of an AM RLC entity shall delay triggering the STATUS report until RX_Next < RX_Next_Highest. NOTE 2: The expiry of t-Reassembly triggers both RX_Highest_Status to be updated and a STATUS report to be triggered, but the STATUS report shall be triggered after RX_Highest_Status is updated. NOTE 3: The expiry of t-RxDiscard triggers both RX_Next to be updated and a STATUS report to be triggered, but the STATUS report shall be triggered after RX_Next is updated. When STATUS reporting has been triggered, the receiving side of an AM RLC entity shall: - if t-StatusProhibit is not running: - at the first transmission opportunity indicated by lower layer, construct a STATUS PDU and submit it to lower layer. - else: - at the first transmission opportunity indicated by lower layer after t-StatusProhibit expires, construct a single STATUS PDU even if status reporting was triggered several times while t-StatusProhibit was running and submit it to lower layer. When a STATUS PDU has been submitted to lower layer, the receiving side of an AM RLC entity shall: - start t-StatusProhibit. When constructing a STATUS PDU, the AM RLC entity shall: - for the RLC SDUs with SN such that RX_Next <= SN < RX_Highest_Status that has not been completely received yet, in increasing SN order of RLC SDUs and increasing byte segment order within RLC SDUs, starting with SN = RX_Next up to the point where the resulting STATUS PDU still fits to the total size of RLC PDU(s) indicated by lower layer: - for an RLC SDU for which no byte segments have been received yet: - include in the STATUS PDU a NACK_SN which is set to the SN of the RLC SDU. - for a continuous sequence of byte segments of a partly received RLC SDU that have not been received yet: - include in the STATUS PDU a set of NACK_SN, SOstart and SOend. - for a continuous sequence of RLC SDUs that have not been received yet: - include in the STATUS PDU a set of NACK_SN and NACK range; - include in the STATUS PDU, if required, a pair of SOstart and SOend. - set the ACK_SN to the SN of the next not received RLC SDU which is not indicated as missing and is not discarded due to the expiry of t-RxDiscard in the resulting STATUS PDU.

# 5	Reference

[1] R2-2507056, Discussion on remaining issues for RLC in R19 XR, Huawei, HiSilicon, RAN2#131bis, Prague, Czech.

[2] R2-2507903 Discussion on the RLC STATUS PDU at t-RxDiscard expiry, Huawei, HiSilicon, RAN2#131bis, Prague, Czech.