3GPP TSG-RAN WG2 Meeting #132	R2-2508795

Dallas, USA, November 17th – 21st, 2025

Agenda Item:	10.4

Source:	Turkcell, Nokia

Title:	Mobility

WID/SID:	FS_6G_Radio - Release 20

Document for:	Discussion and Decision

# 1	Introduction

The motivation for developing a 6G Radio Access Technology (6GR) comes from anticipated growth in service demands and ongoing wireless technology advancements over the next decade. At RAN #108, a new Study Item Description (SID) titled ‘Study on 6G Radio’ was approved, aiming to define a 6G radio system that can address a broad range of use cases and requirements identified during the RAN requirements study phase. Regarding mobility in 6GR, the SID [1] specifies RAN2-led objectives for further studies at RAN #109.

The detailed objectives of the RAN2 related mobility items are:

(4) Mobility for 6GR (for all RRC states), including related RRM [RAN2, RAN1, RAN4, RAN3]

(7) Migration from 5G NR to 6GR as well as interworking and mobility between 5G NR and 6GR:

a) 5G-6G Multi-RAT Spectrum Sharing for migration [RAN1, RAN2, RAN4, RAN3]

- Study if any additional migration option(s) is needed (other than standalone, MRSS, and inter-RAT mobility between NR-6G). [RAN] [RAN2, RAN1, RAN3, RAN4]
RAN plenary starts this study in March 2026 and will make a decision by September 2026 whether to expand WG SI scope to cover additional migration option(s).

- Mobility between 5G NR and 6GR [RAN2, RAN3, RAN4]

Note: Inclusion of LTE/6G interworking/coexistence aspects may be further discussed based on the requirement from RAN plenary

This contribution presents our perspective on the mobility aspects of the 6G radio interface, analyzes limitations identified in 5G NR, and proposes high-level principles to prevent recurrence of such issues in 6G. In this contribution, we discuss further improvements for the mobility based on the conclusions from the 3GPP RAN2 #131bis meeting [2].

# 2	Discussion

The following items were agreed upon in the 3GPP RAN2 #131bis meeting [2] related to the 10.4. Mobility:

Agreements

1	Study mobility with the following requirements in 6G mobility design:

-	Minimize interruption time and ensure service continuity (i.e. minimize throughput degradation during mobility). Consider complexity and gains when discussing solutions.

-	Robustness of mobility procedures

-	Energy efficiency for both UE and NW

2	Study aspects related to mobility (e.g. early DL/UL synchronization, UE configuration processing, pre-configurations, conditional handover, early CSI acquisition)

Across NR Releases 15 to 19, a wide variety of mobility mechanisms have been standardized. Release 15 introduced the initial Layer-3 handover as part of the first NR specification. Release 16 added Conditional Handover (CHO) to enhance robustness and Dual Active Protocol Stack (DAPS) to reduce interruption time. Subsequent releases further refined these concepts, including conditional PSCell change and improvements to CHO with candidate SCG support.

Release 18 introduced Layer-1/Layer-2 Triggered Mobility (LTM) to reduce latency compared to DAPS, while Release 19 standardized Conditional LTM (C-LTM) for better reliability. However, procedures like CHO and C-LTM have overlapping goals, causing implementation fragmentation among vendors due to similar features.

All these mechanisms, along with beam-management schemes defined by RAN1, serve a common purpose: enabling the UE to switch to a new synchronization reference signal for uplink and downlink transmission. Therefore, 6G should aim for a unified mobility framework that combines these approaches into a single architecture to ensure consistent operation and easier deployment.

The number of mobility-related features has increased significantly across NR releases, but integration with the baseline mobility framework is still incomplete. While each feature focuses on specific use cases or scenarios, many end up addressing similar needs.

- The growing number of mobility features across NR releases has not been fully aligned with baseline procedures, leading to overlap and increased configuration complexity.

- 6G should adopt a unified and integrated mobility framework to ensure consistency and simplify configuration and implementation for both UE and the network.

Mobility control in NR is divided between the network and the UE based on the RRC state. In connected mode, the network manages mobility, while in idle and inactive states, UEs handle cell reselection autonomously.

- Since UEs lack visibility into network conditions, they cannot make independent mobility decisions in connected mode without risking overall network performance.

From a network perspective, connected-mode UEs directly influence load distribution. Therefore, network-controlled mobility is crucial for effective resource use. UEs are unaware of factors like network congestion, energy-saving modes, disabled cells, or operator-specific policies that reserve resources for high-priority users.

- Mobility control and traffic steering should remain under the mobile network operator’s policy and management.

RACH-less handover is a low-complexity process that reduces interruption time by skipping the random-access procedure. NR currently defines it for limited intra- and inter-gNB cases under strict coordination. For 6G, a more comprehensive framework should expand RACH-less mobility to various scenarios, including non-ideal backhaul, inter-PLMN, inter-RAT, and other heterogeneous environments, utilizing early TA acquisition and pre-configuration.

- RACH-less handover and early downlink (DL) synchronization effectively reduce interruption times, though current support remains limited.

Early DL synchronization—introduced in Release 18—further decreases cell-switch delays by utilizing pre-activated TCI states, thereby reducing service interruption.

- RAN2 should establish a unified 6G mobility framework that encompasses RACH-less handover and early DL synchronization across all deployment types to ensure seamless, low-latency mobility.

Beyond minimizing interruption time, reducing throughput degradation during mobility is equally crucial since it directly impacts user experience. Early CSI acquisition introduced in Release 19 helps mitigate throughput loss and should be explored further within a unified 6G framework. Throughput degradation can also occur during carrier-aggregation handovers, where direct secondary-cell activation via RRC configuration and early secondary-cell scheduling may improve performance.

- RAN2 should research methods to reduce throughput degradation during mobility, utilizing Release 19 early CSI acquisition and exploring new 6G advancements.

# 3	Conclusion

In this contribution, we made the following observations in the previous section for the mobility, which are summarized:

- The growing number of mobility features across NR releases has not been fully aligned with baseline procedures, leading to overlap and increased configuration complexity.

- Since UEs lack visibility into network conditions, they cannot make independent mobility decisions in connected mode without risking overall network performance.

- RACH-less handover and early downlink (DL) synchronization effectively reduce interruption times, though current support remains limited.

We propose the following based on the discussion in the previous section:

- 6G should adopt a unified and integrated mobility framework to ensure consistency and simplify configuration and implementation for both UE and the network.

- Mobility control and traffic steering should remain under the mobile network operator’s policy and management.

- RAN2 should establish a unified 6G mobility framework that encompasses RACH-less handover and early DL synchronization across all deployment types to ensure seamless, low-latency mobility.

- RAN2 should research methods to reduce throughput degradation during mobility, utilizing Release 19 early CSI acquisition and exploring new 6G advancements.

# 4	References

RP-252912 – SID: Revised SID on Study on 6G Radio, 3GPP TSG RAN Meeting #109, Beijing, China, September 15-18, 2025

3GPP RAN2 #131bis Meeting: R2_131bis_ChairNotes_25-10-17_17-00_final

# 5	Annex