3GPP TSG-RAN WG2 #132	R2-2508797

Dallas, United States, 17th – 21st November 2025

Source:	NTT DOCOMO INC.

Title:	On 6G QoS framework

Document for:	Discussion

Agenda Item:	10.3.1.2

- Introduction

This contribution presents our views on application awareness in the 6G user plane, as well as the challenges operators faced in the 5G QoS framework and the solutions to those challenges.

- Discussion

  - Application awareness considering congestion control algorithm

In the previous meeting, it was agreed to conduct a study on application/service awareness as outlined below [1].

4.	Study potential benefits and standardized mechanisms for applications/service-awareness in the RAN (e.g. immersive communications, AI mobile traffic).  Understand the issues and shortcoming with the current NR QoS.   Aim to look at a general framework.

This section explains why RAN2 should study awareness of application congestion control protocols in the 6G era.

In both NR and pre-NR systems, the air interface has fundamentally been designed to deliver packets in a lossless manner by utilizing both ARQ and HARQ. One major reason for this design choice is that most user data relied on transport layers, where loss-based congestion control algorithms were predominantly used.

Loss-based congestion control algorithm is one type of congestion control protocols that exist in the transport layer. This algorithm interprets packet loss occurring in the transport layer as an indication that congestion has occurred somewhere among the network nodes between the application server and the user terminal, causing buffer overflow within a node and resulting in packet loss. To reduce congestion, it controls the congestion window by reducing its size.

When it comes to communication including 3GPP air interface, the loss-based congestion control algorithm interprets packet loss over the air interface as a sign of congestion caused by buffer overflow at network nodes and responds by reducing the congestion window. This behavior leads to a significant drop in throughput, negatively impacting user experience. This is one of the key reasons why the air interface up to NR has been designed to be lossless.

Observation 1.	Before NR was standardized, loss-based algorithms dominated congestion control in the transport layer. Consequently, the 3GPP air interface up to NR was designed on the principle of lossless transmission.

Figure 1. How network congestion is mistakenly detected even when packet loss happens in air interface.

Now we describe the congestion control algorithms at the transport layer in the table. (To be very correct, it may be noted that the L4S algorithm, introduced as a derivative of delay-based congestion control, is not limited to the transport layer — it can be implemented at various layers that terminate between nodes.)

| Congestion control algorithm | Examples | Description |
| --- | --- | --- |
| Loss-based congestion control | CUBIC, NewReno | This algorithm is at transport layer and determines that network congestion is worsening as the number of packets discarded during data transmission increases and reduces the congestion window size, i.e., lowers throughput. |
| Delay-based congestion control | BBR, TCP RACK | This algorithm is at transport layer and determines that network congestion is worsening when the amount of delay caused by packet transmission increases and reduces the congestion window size, i.e., lowers throughput. BBR is a delay-based algorithm that is already used in the latest popular applications, e.g., YouTube [2], and expected to be used for more applications. |
| Derivative of delay-based congestion control | L4S [3][4] | This algorithm estimates congestion between each node by calculating the proportion of ACKs received every RTT (Round-Trip Time) that are flagged with ECN (Explicit Congestion Notification). L4S is rapidly gaining adoption as a technology for enabling end-to-end ultra-low latency communication. |

As shown in the table, there is a growing trend among servers providing today’s major applications to adopt delay-based rather than loss-based congestion control algorithms. Specifically, examples include the BBR algorithm, invented in 2016 and currently used by heavy-traffic services like YouTube, and the L4S algorithm, which is rapidly gaining traction in mobile networks following the specification of its required technologies in 3GPP Release 18.

Observation 2.	Although NR was standardized during the dominance of loss-based congestion control, application servers are now increasingly adopting delay-based algorithms like BBR and L4S.

In delay-based congestion control algorithms, packet loss over the air interface does not lead to a reduction in throughput, so it may not have to ensure loss-less delivery by utilizing both ARQ and HARQ. Instead, congestion is detected based on transmission delays, thus it may be preferable to reduce delay jitters in the air interface.

Therefore, Layer 2 optimizations may be possible, such as disabling ARQ retransmission functions in the RLC layer, which could otherwise contribute to delay jitter as already mentioned in section 2.3 when using delay-based congestion control algorithms.

We believe that these optimized Layer 2 functions must be configurable by the base station. The reason is to ensure operational flexibility. Operators have the freedom to build a distributed network by deploying CDNs (Content Distribution Networks) near specific base stations. If any of the deployed CDNs adopt a delay-based congestion control algorithm, the corresponding base station should be able to configure its Layer 2 functions accordingly.

Observation 3.	Unlike loss-based algorithms, delay-based congestion control favors reducing delay jitters rather than ensuring lossless transmission on the air interface.

Proposal 1.	RAN2 studies awareness of congestion control protocols in the transport layer and investigates methods to reduce delay jitter on the AS interface.

  - QoS mapping and data transfer

It is specified in NR [5], when transferring data from the target gNB to the source gNB during handover, the mapping from QoS Flow to DRB applied at the target gNB must match the mapping applied at the source gNB.

TS 38.300 9.2.3.2.3 Data Forwarding If "lossless handover" is required and the QoS flows to DRB mapping applied at the target NG-RAN node allows applying for data forwarding the same QoS flows to DRB mapping as applied at the source NG-RAN node for a DRB and if all QoS flows mapped to that DRB are accepted for data forwarding, the target NG-RAN node establishes a downlink forwarding tunnel for that DRB.

How QoS Flow is mapped to DRB is determined by operational decisions and is communicated to each UE connected to a gNB via RRC reconfiguration. Therefore, if an MNO wants to design a service that ensures loss-less handover, it is necessary to unify the mapping rule between QoS Flow to DRB between the source gNB and the target gNB. But in some cases, this requirement imposes design and operational costs on MNOs.

For example, data forwarding cannot be performed between two gNBs that apply different configurations for a given PDU session as shown below:

- Configuration 1: No mapping between QoS Flow and DRB; in other words, no SDAP header is added to DL data.

- Configuration 2: Only one DRB is configured for the UE, and all QoS Flows are mapped to this DRB using RRC parameters.

Note that, in this case, data forwarding cannot be performed even though both configurations achieve the same result, i.e., transmitting all DL data through a single DRB.

In general, which configuration (1 or 2) is adopted by default varies depending on the vendors designing the gNB, or depending on gNB products. Therefore, to enable data forwarding, there is a design cost associated with unifying the configuration (either 1 or 2) between neighboring gNBs.

Observation 4.	In NR, the requirement to align the mapping between QoS Flows and DRBs across the source gNB and target gNB in order to enable data forwarding is a factor that increases operational costs.

Proposal 2.	Study methods to enable data forwarding regardless of the configuration for mapping between QoS Flows and DRBs across neighboring cells.

- Conclusions

Application awareness considering congestion control algorithm

Observation 1.	Before NR was standardized, loss-based algorithms dominated congestion control in the transport layer. Consequently, the 3GPP air interface up to NR was designed on the principle of lossless transmission.

Observation 2.	Although NR was standardized during the dominance of loss-based congestion control, application servers are now increasingly adopting delay-based algorithms like BBR and L4S.

Observation 3.	Unlike loss-based algorithms, delay-based congestion control favors reducing delay jitters rather than ensuring lossless transmission on the air interface.

Proposal 1.	RAN2 studies awareness of congestion control protocols in the transport layer and investigates methods to reduce delay jitter on the AS interface.

QoS mapping and data forwarding

Observation 4.	In NR, the requirement to align the mapping between QoS Flows and DRBs across the source gNB and target gNB in order to enable data forwarding is a factor that increases operational costs.

Proposal 2.	Study methods to enable data forwarding regardless of the configuration for mapping between QoS Flows and DRBs across neighboring cells.

- References

[1]	Chair notes, RAN2#131bis.

[2]	“TCP BBR congestion control comes to GCP – your Internet just got faster,” Google Cloud Blog.

[3]	RFC 9330, “Low Latency, Low Loss, and Scalable Throughput (L4S) Internet Service: Architecture.”

[4]	RFC 9331, “The Explicit Congestion Notification (ECN) Protocol for Low Latency, Low Loss, and Scalable Throughput (L4S).”

[5]	TS 38.300, “NR; NR and NG-RAN Overall description; Stage-2,” V19.0.0.