3GPP TSG-RAN WG2 Meeting #132                                    	R2-2508811

Dallas, USA, November 17th-21st, 2025

Agenda item:	8.17

Source:	Qualcomm Incorporated

Title:	SPS for NB-IoT NTN TDD mode

Document for:	Discussion and Decision

- Introduction

In RAN2#131bis, SPS collision issue was not resolved and following was captured.

Come back to the SPS collision issue (due to SPS postponement) in the next meeting.

In this document, we provide details to resolve the issues.

- Discussion

In NB-IoT TDD mode, it is agreed that when UL SPS occasion falls on an invalid frame, it will be postponed to the next valid frame. When SPS occasion is postponed to the next valid frame, multiple UE’s UL transmission may collide. This would lead to interference and failure of the transmission. We think the simple solution is to interpret current values of the SPS periodicities as new values that are multiple of 90ms.

For example, the value sf128 of semiPersistSchedIntervalUL-r15 can be interpreted as value sf180 in NB-IoT TDD mode without changing ASN.1 signaling.

There is also concern that with such periodicity, it may still be possible that repetitions may spill over next UL occasion. Similar to handling paging occasion and paging repetitions, network can make sure the gap between two UL SPS occasions is larger enough (i.e., enough number of valid frames) to accommodate the repetitions of PUSCH transmission. The other solution is to define UE behavior that the UE continues the transmission and in case of overlapping with the start of the new UL SPS occasion, the UL grant in the SPS occasion is skipped. We think such behavior is not necessary with proper network configuration.

For SPS interval, the values of semiPersistSchedIntervalUL-r15 are interpreted as new values that are multiple of 90ms.

It is up to network to configure the periodicity larger enough to have enough number of valid frames to accommodate PUSCH repetitions.

- Conclusion

Following proposals are made:

Proposal 1	For SPS interval, the values of semiPersistSchedIntervalUL-r15 are interpreted as new values that are multiple of 90ms.

Proposal 2	It is up to network to configure the periodicity larger enough to have enough number of valid frames to accommodate PUSCH repetitions.