3GPP TSG-RAN2#132 	R2-2508826

Dallas, USA, Nov 17th –21th, 2025

Agenda item:		8.4.2 (NR_LPWUS-Core)

Source:	LG Electronics Inc.

Title: 	Discussion on UAI for enabling and disabling LP-WUS

Document for:	Discussion and Decision

1.	Introduction

In the last meeting, RAN2 discussed whether to allow UAI for enabling/disabling LP-WUS, but there was no conclusion.

In this contribution, we present our view on UAI for enabling/disabling LP-WUS.

2.	Discussion

In the last meeting, it was proposed to allow the UE to send UAI based on LR measurement to indicate enabling/disabling LP-WUS. The motivation is that the network does not know the accurate LR quality even though the measurement report is received. This is because the measurement report is based on MR measurement and the MR measurement result may differ from actual LR quality due to the weaker tolerance for adjacent-channel interference for LR. Thus, companies thought that the UAI based on LR measurement is needed.

However, we have different understanding on this.

Firstly, in order to support the UAI based on the LR measurement result, LR measurement needs to be performed, but is not currently supported RRC_CONNECTED.

Agreement (in RAN1#112bis-e) - For RRC connected mode, the following is assumed for LP-WUS study in RAN1 - RLM/BFD/CSI are performed by UE Main Radio (MR) - RRM measurements are performed by UE Main Radio (MR) - Ultra-deep sleep state is not allowed for MR. - Study additional support of RRM measurement by LP-WUR for RRC connected mode - Study RRC connected mode LP-WUS functionality/purpose/procedures - Study RRC connected mode LP-WUS activation/deactivation procedures. - Study RRC connected mode LP-WUS BW, whether same as IDLE/Inactive mode or different - In RRC connected, study the relationship between LP-WUS and legacy UE power saving techniques.

During LP-WUS study phase, RAN1 agreed to assume that RRM measurements are performed by MR in RRC_CONNECTED, and there was no further agreement on RRM measurement by LR, i.e. RRM measurement in RRC_CONNECTED is allowed in MR only. RAN1 designed LP-WUS in RRC_CONNECTED with this assumption during work item phase, so there may be a big impact on RAN1 if RAN2 concludes to allow the RRM measurement by LR.

In addition, in our understanding, MR and LR reception are correlated, so the MR measurement result may not significantly differ from the actual LR quality. This is because LP-WUS resource should be within the same carrier as the active DL BWP of PCell according to RAN1 agreement. Thus, the MR measurement result is sufficient to help NW decide whether to enable or disable LP-WUS.

Agreement (in RAN1#120) For RRC connected, LP-WUS frequency resource can be outside of active DL BWP but has to be within the same carrier as the active DL BWP - Basic capability is LP-WUS, if present, frequency resource within active DL BWP. LP-WUS frequency resource outside of active DL BWP is subject to separate UE capability - No RAN1 optimization specific to the case where LP-WUS frequency resource is outside of active DL BWP

In addition, even though there may be a gap between the MR measurement result and the actual LR quality, it would be marginal and the smart network can consider the marginal gap when the network determines enabling/disable LP-WUS.

Meanwhlie, some companies argued to support UAI based on MR measurement result. However, based on MR measurement is reported, the network can decide whether to enable/disable LP-WUS. Thus, UAI based on MR measurement result is redundant information. If the network configure the measurement configuration well to cover LR coverage, the network can obtain sufficient information to decide to enable/disable LP-WUS.

Therefore, we think that UAI for enabling/disabling LP-WUS is not needed.

Proposal 1. UAI for enabling/disabling LP-WUS in RRC_CONNECTED is not supported.

3.	Conclusion

In this document, we discuss UAI for enabling/disabling LP-WUS in RRC_CONNECTED, and make proposals as follows.

Proposal 1. UAI for enabling/disabling LP-WUS is not supported.

4.	Reference