3GPP TSG-RAN WG2 Meeting #132	R2-2508839

Dallas, USA, Nov. 17th-21st, 2025

Agenda Item:	10.2.1

Source:	CMCC

Title:		Considerations on UE capability framework for 6GR

WID/SID:		FS_6G_Radio

Document for:	Discussion

# 1 Introduction

In RAN #108, the New SID on 6G Radio was agreed, and the principle for 6GR UE capability signaling framework is as following [1]:

d) Radio signalling framework for UE capabilities, aiming at improvements and simplification compared to 5G NR [RAN2, RAN1, RAN4]

And in last RAN2 meeting, there were some discussion on UE capabilities for 6GR, and the following agreements are achieved [2]:

- Agreements on UE capabilities for next meeting

- For next meeting companies should focus on identifying the critical problems with UE capability framework and how to address them.  Identify what actions to be triggered with other WGs.

- Companies can identify use cases to justify dynamic capability change in connected mode and analyse impacts to network side and network behaviour upon reception of updated capabilities.   Understand if there is impact to other WGs.

- Companies can focus on the practical IODT problems in the field.   Possible ways to address them.

In this contribution, we would like to provide some considerations on the UE capability for 6GR.

# 2 Discussion

## 2.1 Issues of NR UE capability

Issues of UE capability signaling structure

Compared to LTE, the main enhancements introduced for NR UE capability size reduction were gNB requested band combination signaling, baseband capability extraction and UL/DL bands decoupling.

However, it is proved that a large number of frequency band combinations and complex signaling structures require higher development and maintenance costs for terminals and network. And there is overly redundant UE capability information in multiple band combinations and feature sets. Further there are some new features introduced in the subsequent discussions of different NR releases leading to exploding UE capabilities. And most new features of different NR release are defined as optional, resulting that increasingly more overhead on reporting more and complexity to both UE and network are introduced, and some beneficial features are not supported by most UEs for some cost and information leakage considerations.

Table 1 UE capability specification content comparison

|  | LTE | NR | 6GR |
| --- | --- | --- | --- |
| Size | TS 36.306: ~160 pages Per UE/Band/BC | TS 38.306: ~400 pages Per UE/Band/BC/FS/FSPC FR1/FR2/FR2-2 | Simplification! |

Observation 1: The existing UE capability framework is overly complex with exploding size, complex band combinations and feature sets increase development and maintenance costs, and explosive growth in UE capability size due to optional features.

Definition of Device type categories with future proof

To enable the network to efficiently support diverse 6G use cases, meet performance requirements, and enhance user experience, it is beneficial for RAN to identify and study representative device types. Device categorization can be based on characteristics such as power class, supported bandwidth, and number of antennas. For each device type, a baseline UE capability profile can be defined to represent mandatory capabilities, while UEs would only report deviations from the baseline.

At least three representative UE types can be identified for 6G:

- Type I: Low-power devices, e.g., LPWA, RedCap, wearables, and 2Rx UEs.

- Type II: Devices with moderate radio capability, similar to traditional eMBB UEs.

- Type III: Devices with high radio capability, e.g., CPE, FWA, and high-end XR devices.

In addition to these categories, 6G is expected to support emerging device types driven by new services and features. The introduction of AI-native functionalities, immersive or multi-modal services, NTN, aerial, and vehicular deployments indicates that categorization based solely on radio capability may be insufficient. These service-specific features may define new or sub-categories of device types that warrant further study.

This means 6G device categorization should be adaptable for revolving demands of future devices. Each device type should define the minimum set of supported UE capabilities and the allowed basic RRC configuration; Knowing the default or minimum set of capabilities associated with each device type can facilitate smooth enable the configuration profile manner with additional delta configuration. And optional UE capability need on top of device type.

Observation 2: To reduce the capability signaling, it is proposed to study representative 6G device types and define categories based on both radio capability (e.g., power class, bandwidth, number of antennas) and emerging service-specific features, including AI-native functions, multi-modal support, NTN, vehicular, and aerial deployments. This will be an inter-WG issue, with RAN, RAN1 and RAN4 involved.

Per the RAN #109 discussion, 6GR aims to define a set of mandatory baseline functionalities for all UEs, however, regarding the detailed device types and related parameters/factors, further discussion is needed. Then in LTE, per UE Category uplink and downlink capability, including combined UL/DL UE capabilities for Category 1~Category 12 are introduced. However, such fixed definition mechanism is not friendly and can not achieve forward compatibility. Therefore, 6GR needs to consider a more flexible mandatory baseline functionalities based on diverse device types. Further, such mandatory UE Capability need to consider forward compatibility for more possible new device types introduced whereafter, which could improve flexibility compared to LTE per category UE Capability mechanism.

Observation 3: UE categories defined in LTE can facilitate the capabilities' reporting signaling reduction, however, per fixed definition of UE Category uplink and downlink capability in LTE is not friendly and can not cover foreseeable use cases, 6GR needs to consider a definition of Device type categories with future proof.

Issues of UE capability update mechanism

For UE capability update, the current mechanism is based on UAI (UE assistance information), including the cases overheating, power saving, and MUSIM, etc. However, such kind of UE capability update is mainly a temporary capability change made for some reasons, which may not be maintained. Further, from the network side, it might not adopt UAI due to that the UAI is merely a suggestion or hope from the UE side.

Observation 4: UAI-based updates are temporary and not guaranteed to be adopted..

## 2.2 Considerations on UE capability for 6GR

UE capability signaling structure optimization

Different from the existing too complex UE capability signaling structure including per BC, per FS, per FSPC, 6GR system could consider UE capability signaling structure optimization for 6GR. Specifically, define mandatory UE capability per device type could be considered due to that diverse device types would be introduced in 6GR as mentioned above. And different device types have different related parameters and factors needed to be taken into account for the corresponding Mandatory UE capability. For example, define as more fundamental and comprehensive UE capability sets as possible for all possible device types at 6G initial stage (as shown in the following figure). Then if new types are introduced later, the mandatory baseline UE capability set will be extended in the form of "add on" and based on network configuration to reach forward compatibility. Further in addition to per device type UE capability set, per service UE capability set may be also considered, such as QoS based mandatory baseline UE capability set, etc..

Figure 1: Device type based UE capability framework

Then to further optimize the UE capability structure, index based UE capability (set) could be discussed. As shown in the following table, different UE capability (set) associated with one index, and the detail capability information is pre-defined at network (e.g. RAN, CN node) or the third-party server. When network sends UE capability request signaling, UE could only report corresponding capability index information, which could help to reduce the signaling overhead. Then to further reduce the signaling overhead, UE could only report UE capability information based on network specific request, such as AI related capability, sensing capability, etc..

Table 2: An example for index based UE capability (set)

| Index 1 | UE capability (set) 1 |
| --- | --- |
|  | UE capability (set) 2 |
|  | UE capability (set) 3 |
| Index 2 | UE capability (set) 4 |
|  | UE capability (set) 5 |

To address the interoperability issues due to different understanding on complicated BB/RF features between UE and NW, exploding UE capability size of band combination defines in the specification. Then if the UE is enabled to report explicit RF capability parameters, i.e., RF number supported by UE and corresponding band and bandwidth parameters, it will be beneficial to simplify the UE capability signaling structure.

Observation 5: Enabling the UE to report explicit RF capability parameters will facilitate the simplification of the UE capability signaling structure, i.e., RF number supported by UE and corresponding band and bandwidth parameters.

Proposal 1: RAN2 should study a simplified and future-proof UE capability framework for 6GR, including per device type mandatory UE capabilities, index-based reporting, network-provided filter based UE capability reporting, explicit RF capability parameters, etc., in coordination with RAN, RAN1 and RAN4.

UE capability updating mechanism with forward compatibility

More flexible UE capability update mechanism different from the exiting capability restriction based on UAI is also needed to be considered, especially for various UE operation manners in some cases, for example, RF modification at UE, (e.g., foldable screen devices switching between foldable screen and unfoldable, NeedForGap), UE status change(e.g.,low battery status, overheating status), etc.. In such cases, dynamic update of UE capability can facilitate the RAN node to provide more appropriate configurations to UE based on UE’s situation. Then regarding the updatable parameters, it may be better to keep them within a limited range (e.g. maximum channel bandwidth, CC number, supported MIMO layer, etc.). Further the updated UE capability information is better to only report to the RAN node to minimize the impact on the core network as much as possible.

Proposal 2: Enable flexible UE capability update mechanism for some use case, for example, RF modification at UE, (e.g., foldable screen devices switching between foldable screen and unfoldable, NeedForGap), UE status change(e.g.,low battery status, overheating status).

Proposal 3: It may be better to keep the updatable parameters within a limited range (e.g. maximum channel bandwidth, CC number, supported MIMO layer, etc.)

Proposal 4: It is proposed to only report the updated UE capability information to the RAN node.

To support flexible UE capability update, the following two solutions could be considered.

Network control based UE capability update reporting solution, UE needs to perform the capability update reporting procedure based on network indication/request. And network could configure UE a not-allowed update indication for UE capability or network does not explicitly indicate the UE to report capability update to the UE if network does not expect the UE to change its capabilities in some cases. Conversely UE could perform the capability update reporting procedure when network explicitly indicate the UE to report capability update. Further if network only needs to obtain some certain specific UE capability update information, it sends UE capability update request information including the specific request to UE, and then UE report the UE capability update information based the specific request from network. With this solution, UE could only report the specific capability update information instead of whole UE capability update information, which is beneficial for signaling overhead reduction.

Event-based UE capability reporting solution, in order to further reduce the signaling overhead due to multiple UE capability update indication/requests from network, an event based UE capability update mechanism could be discussed. Network configure the UE capability update trigger condition in advance, once the condition is fulfilled, UE report capability update information directly without UE capability update request signaling from network.

Proposal 5: RAN2 could consider more flexible UE capability update mechanisms, including network-control based and event-based approaches.

IODT problem

Per the discussion in last meeting, IODT problem is raised, [3] proposed that RAN2 to study solutions allowing the UE to activate a feature capability after IODT with a given set of network vendor, while avoiding interoperability problems due to the lack of IODT with other network vendors.

Firstly, we provide the whole IODT phase applied in CMCC as shown in figure 2, which approximately lasts two years from specification release to E2E commercialization. Actually, CMCC only requires UE/chipset vendors to verify the features through IODT test with some given vendors before the UE/chipset activate the features in operator’s deployed network. Hence, it seems the above IODT issue is not so serious. On the other hand, considering that per the current implementation, the IODT issue may just occur in the first version of each generation and then get alleviated gradually via software version updating by OTT. Further in subsequent versions, there will also be regression testing for upgrade. Therefore further enhancements are not needed to resolve the IODT issue.

Figure 2: IODT test phase of CMCC

Observation 6: Per the current implementation, to verify the features through IODT test with some given vendors before the UE/chipset activate the features in operator’s deployed network is already supported in CMCC.

Proposal 6: Regarding the IODT issue, further enhancements are not needed.

However, the problem of inconsistency between UE capabilities and the network indeed exists. For instance, some UEs do not support BWP dynamic configuration via DCI, the RF capabilities do not match the 3GPP specification, etc.. To achieve better performance, the mismatch issue needs to be discussed.

Proposal 7: Inconsistency issue (e.g. RF capabilities do not match the 3GPP specification, etc.) between UE capabilities and the network needs further discussion.

# 3 Conclusion

Based on the discussions mentioned above, in this contribution we provide some discussions on 6GR UE capability and have the following observations and proposals:

Observation 1: The existing UE capability framework is overly complex with exploding size, complex band combinations and feature sets increase development and maintenance costs, and explosive growth in UE capability size due to optional features.

Observation 2: To reduce the capability signaling, it is proposed to study representative 6G device types and define categories based on both radio capability (e.g., power class, bandwidth, number of antennas) and emerging service-specific features, including AI-native functions, multi-modal support, NTN, vehicular, and aerial deployments. This will be an inter-WG issue, with RAN, RAN1 and RAN4 involved.

Observation 3: UE categories defined in LTE can facilitate the capabilities' reporting signaling reduction, however, per fixed definition of UE Category uplink and downlink capability in LTE is not friendly and can not cover foreseeable use cases, 6GR needs to consider a definition of Device type categories with future proof.

Observation 4: UAI-based updates are temporary and not guaranteed to be adopted.

Observation 5: Enabling the UE to report explicit RF capability parameters will facilitate the simplification of the UE capability signaling structure, i.e., RF number supported by UE and corresponding band and bandwidth parameters.

Proposal 1: RAN2 should study a simplified and future-proof UE capability framework for 6GR, including per device type mandatory UE capabilities, index-based reporting, network-provided filter based UE capability reporting, explicit RF capability parameters, etc., in coordination with RAN, RAN1 and RAN4.

Proposal 2: Enable flexible UE capability update mechanism for some use case, for example, RF modification at UE, (e.g., foldable screen devices switching between foldable screen and unfoldable, NeedForGap), UE status change(e.g.,low battery status, overheating status).

Proposal 3: It may be better to keep the updatable parameters within a limited range (e.g. maximum channel bandwidth, CC number, supported MIMO layer, etc.)

Proposal 4: It is proposed to only report the updated UE capability information to the RAN node.

Proposal 5: RAN2 could consider more flexible UE capability update mechanisms, including network-control based and event-based approaches.

Observation 6: Per the current implementation, to verify the features through IODT test with some given vendors before the UE/chipset activate the features in operator’s deployed network is already supported in CMCC.

Proposal 6: Regarding the IODT issue, further enhancements are not needed.

Proposal 7: Inconsistency issue (e.g. RF capabilities do not match the 3GPP specification, etc.) between UE capabilities and the network needs further discussion.

# 4 Reference

- RP-251881 New SID: Study on 6G Radio

- R2_131bis_ChairNotes_25-10-17_17-00_final

- R2-2506988 Considerations on UE capability signalling in 6G	Qualcomm Incorporated	discussion	Rel-20	FS_6G_Radio