3GPP TSG-RAN WG2 Meeting #132		R2-2508851

Dallas, USA, 17th – 21st Nov. 2025

Source:	TCL

Title:	Discussion on initial and system access for 6GR

Agenda Item:	10.3.2.3

Document for:	Discussion and Decision

- Introduction

In the RAN#109 meeting, the revised SID [1] for 6G radio was agreed, in which some 6G requirements for energy efficiency, system simplification, diverse device types, and latency were included. In the RAN2#131bis meeting [2], some agreements were reached as follows.

Agreements on system information: - System information design should consider energy efficiency and low complexity for both network and UE. - System information design should ensure that the public warning requirements are met. - Study RAN2 aspects of system information designs (e.g. on-demand SIB and SSB, periodicity of SSBs) - Study how to improve flexibility and extensibility of scheduling SI design - Study mechanisms to improve SI update mechanism - Study aspects related to SIB1 size. Understand the issues and desired content/size from RAN2 point of view. - Study areas specific SIB design. Understand 5G pain points and what improvements can be considered. NOTE: this is a starting list Agreements on paging: - Paging design should consider energy efficiency and simplity for both UE and Network. Agreements on energy efficiency: - For next meeting companies are encouraged to identify what UE/NW power saving features can be considered and improved in 6G while avoiding specifying multiple features for the same purpose. Focus on solutions that have RAN2 impacts.

In this contribution, we will discuss the system information, paging, and access control.

- Discussion

  - System information

SI content

In NR, some system information blocks are broadcast in a unified manner for all UEs with different device types or capabilities. For example, SIB1 is delivered regardless of whether the UE is a high-performance smartphone or an IoT device, even though many fields are irrelevant for IoT UEs. Similarly, SIB2 and SIB3 carry frequency reselection information for bands that certain UEs do not support but must nonetheless decode. Besides, the system information in the NTN cell and TN cell is different. When device types are supported over a unified air interface in 6G,  the current approach lacks sufficient granularity. Undifferentiated broadcasting may result in some information being redundant for certain UEs, leading to higher UE power consumption and increased latency in system information acquisition.

Observation 1: Some system information blocks uniformly broadcasting to all device types result in redundant information for certain devices, leading to UE power consumption and longer system information acquisition delay.

In the cell selection and reselection procedures, determining whether a candidate cell is a suitable cell is a key step. If the candidate cell does not meet the criteria for a suitable cell, the UE continues to evaluate other candidate cells. In the current NR system, the criteria for determining a suitable cell are included in SIB1 and occupy only a small portion of it. This design requires the UE to receive and decode the entire SIB1 to obtain the suitable cell criteria, even though only a small subset of the information is needed. As a result, the UE experiences increased latency and higher power consumption during cell selection and reselection, particularly when multiple candidate cells are evaluated before finding a suitable cell to camp on. This issue becomes more pronounced for UEs with stringent power or latency requirements, such as IoT devices.

Observation 2: The criteria for the suitable cell are included in SIB1 and occupy only a small portion of SIB1. The UE needs to receive the entire SIB1 to obtain the criteria for the suitable cell during the cell selection/reselection processes, resulting in more latency and UE power consumption.

In NR, the cell bar and cell reservation indications are used by the UE to determine whether it can (continue to) camp on a given cell. To support different functionalities, NR defines a lot of Information Elements for various types of cell bar and reservation, as summarized in the table below. However, the current design of these IEs introduces signaling overhead and complexity.

Table 1: The NR cell bar and cell reservation IEs

| Cell bar IE | For RedCap: cellBarredRedCap1Rx, cellBarredRedCap2Rx, halfDuplexRedCapAllowed, intraFreqReselectionRedCap For eRedCap: cellBarred-eRedCap1Rx, cellBarred-eRedCap2Rx, intraFreqReselection-eRedCap For XR, NES: cellBarred2RxXR，cellBarredNES For IAB, NCR: iab-Support, mobileIAB-Support, ncr-Support For NTN, ATG: cellBarredNTN, cellBarredFixedVSAT, cellBarredMobileVSAT, cellBarredATG |
| --- | --- |
| Cell reservation IE | cellReservedForOperatorUse cellReservedForOtherUse cellReservedForFutureUse |

Firstly, defining each IE separately increases the signaling size, as ASN.1 encoding requires one bit for every optional IE to indicate whether the optional IE exists or not. Secondly, the cell bar and reservation IEs are complex, resulting in complex procedures and configurations in TS 38.304 and TS 38.331. With the anticipated increase in diversity of 6G service scenarios compared with 5G, the signaling overhead and structural complexity issues are expected to become more significant. This situation is not aligned with the RAN simplification objectives pursued in 6G.

Observation 3: The cell bar/reservation information in the system information is complex.

Based on the analysis for Observation 1 to Observation 3 above, undifferentiated broadcasting, the current mechanism for suitable cell determination, and the cell bar and cell reservation indications may result in higher UE power consumption, increased latency in system information acquisition, and RAN complexity. Therefore, the current mechanisms are not fully aligned with the 6G goals of low latency, sustainability, and RAN simplification. It is necessary to explore more efficient mechanisms to address these problems.

In NR, the physical layer imposes a limit to the maximum size a SIB can take. The maximum SIB1 or SI message size is 2976 bits. Besides, in RAN2#131bis meeting [2], we discussed whether the size limitation of SIB1 can be extended to support more content for 6G. In our view, the current size limitation of SIB1 is enough as long as we address the problems proposed in Observation 1 to Observation 3 and adopt other simplification mechanisms (e.g., the simplification mechanism for paging in clause 2.2).

Proposal 1: The system information should be enhanced to address the problems proposed in Observation 1 to Observation 3. The current SIB1 size will be enough with the enhancement and simplification mechanisms.

Area-specific SI

In NR, the area-specific system information mechanism allows certain SIBs to be shared across multiple cells within the same area, aiming to reduce UE power consumption. However, the current mechanism has not been widely adopted. Firstly, only a small subset of SIBs can be the same within an area. Secondly, for area-specific SIBs, the required coordination and interaction between gNBs is complex. This complexity increases the signaling burden between gNBs and complicates network management and configuration.

Observation 4: 5G pain points of area-specific system information are as follows,

- Only a small number of system information blocks can be shared across cells.

- The interaction and coordination between gNBs is complex.

To solve the pain points above, some improvements can be considered. For example, the information elements that are likely to be the same in multiple cells can be centrally included in one or several system information blocks. The area-specific SIB is uniformly deployed by operators without interaction between gNBs.

Proposal 2: One or several system information blocks are defined to include the information elements that are likely to be the same in multiple cells. The area-specific SIB is uniformly deployed without interaction between gNBs.

SI scheduling and update

In NR, other system information and positioning system information blocks are carried in multiple system information messages, each containing one or more SIBs. These SI messages are transmitted periodically within their respective transmission windows, and the windows do not overlap with each other. This design results in longer latency for UEs to obtain all the necessary system information. For example, when there are multiple SI messages—such as eight SI messages—the UE, after receiving the MIB and SIB1, may still need to wait through eight separate windows to complete the reception of all required SI messages. Such sequential scheduling increases both the overall system information acquisition delay and the power consumption of the UE. Similarly, the network also incurs additional power consumption to maintain periodic transmissions across multiple windows.

Observation 5: Different system information messages are sent within their respective windows, and they do not overlap with each other. It increases the delay for the UE to obtain all necessary system information, and the power consumption for both network and UE.

In NR, the network transmits the system information update notification to UEs to indicate that the system information (will) change. The updated SI includes both the changed and unchanged parts. For UEs that have already received the previous version of the SI, the unchanged part is redundant. When the UEs requiring the SI are all those that have previously received the older version, transmitting the unchanged part also becomes unnecessary from the network perspective. The transmission and reception of such redundant information lead to increased signaling overhead, longer system information acquisition delay, and higher power consumption for both network and UE. This issue becomes more pronounced when the size of the SI is large.

Observation 6: The updated system information includes both the changed part and the unchanged part. The unchanged part results in increased power consumption for both network and UE, and longer system information acquisition delay.

The current mechanisms for the  SI windows and updated system information are particularly unfavorable for UEs with latency-sensitive services or stringent low-power requirements, and network energy saving. It is desirable to explore more efficient mechanisms to reduce delay and improve power efficiency in system information transmission and acquisition. For example, the windows of system information messages can overlap with each other. Different UEs only decode the required system information message. The updated system information only includes the changed content.

Proposal 3: The windows of system information messages can overlap with each other. The updated system information only includes the changed content.

On-demand SI/SSB

In NR, the on-demand SIB1 mechanism is specified to achieve network energy saving, in which the SIB1 is only transmitted when the network receives a request from UE. However,  the application scope of this mechanism is limited. Because the UE relies on cell A to obtain the uplink WUS configuration that is used to request SIB1 transmission from the NES cell. In other words, the UE needs to first camp on cell A, and the NES cell can only be the candidate cell for cell reselection.

Observation 7: In NR, only cell A-assisted NES cell with on-demand SIB1 is specified.

To enable more widespread realization of network energy saving, the applicable scope for on-demand SIB1 can be extended. Specifically, the following scenarios can be considered.  The standalone NES cell with only on-demand SIB1, in which the UL WUS configuration can be obtained from the NES cell itself or predefined protocol parameters. The standalone NES cell with on-demand SSB and on-demand SIB1, in which the UL WUS configuration can be obtained from the predefined protocol parameters. The cell A-assisted NES cell with on-demand SSB and on-demand SIB1, in which the UL WUS configuration can be obtained from cell A. Compared with cell A-assisted NES cell, the standalone NES cell can be the candidate cell for both cell selection and cell reselection. Compared with only on-demand SIB1, on-demand SSB and on-demand SIB1 can achieve more network energy saving. RAN2 is expected to discuss which scenarios are supported in 6G.

Proposal 4: RAN2 to discuss which scenarios below are supported for on-demand SIB/SSB for NON-RRCCONNECTED UEs,

- Cell A-assisted NES cell with only on-demand SIB1;

- Cell A-assisted NES cell with on-demand SSB and on-demand SIB1;

- Standalone NES cell with only on-demand SIB1;

- Standalone NES cell with on-demand SSB and on-demand SIB1.

  - Paging

In LTE and NR, several power-saving mechanisms have been defined—such as DRX, eDRX, GWUS, PEI, and LP-WUS—that operate on paging monitoring for NON-CONNECTED UEs. When DRX is enabled, the UE does not continuously monitor the paging. Instead, the UE monitors the paging only during a PO within each DRX cycle and remains in low-power sleep mode at all other times, thereby reducing the overall active time of the radio receiver. Extended Discontinuous Reception (eDRX) is an enhancement to the DRX mechanism that allows the NON-CONNECTED UE to adopt significantly longer sleep cycles for paging monitoring. Thus, it achieves even lower power consumption as the following Figure in TR38.875 [3], showing the relationship between eDRX and the UE battery lifetime.

Figure 1: RedCap UE battery lifetime in RRC_INACTIVE and RRC_IDLE state

Regarding the PEI/LP-WUS/GWUS mechanisms, before each PO, the network transmits a short indication signal to inform the UE whether it needs to monitor the PO. If the indication shows that no paging for that UE is expected, the UE may skip monitoring of the PO. This reduces the probability of unnecessary wake-up for the UE, thereby avoiding superfluous energy consumption. In practice, the network groups UEs into different groups, with each group being associated with a specific indication or wake-up signal. Based on the text below illustrated from TR38.869 [4], LP-WUS achieves paging latency reduction and UE power saving gain compared with eDRX.

- Compared with existing eDRX operation, significant paging latency reduction and moderate UE power saving gain is observed, if LP-WUS monitoring and the corresponding paging monitoring after MR wake-up is performed not restricted within existing PTW of eDRX. Significant UE power gain and moderate paging latency increase is observed if LP-WUS monitoring is restricted within existing PTW of eDRX and existing paging occasion determination is reused.

PEI and LP-WUS each present distinct advantages and limitations. LP-WUS, which relies on a low-power receiver to monitor a low-complexity physical wake-up signal, achieves greater power saving but introduces additional paging latency. Conversely, PEI uses the primary receiver to monitor DCI, resulting in lower paging latency but reduced power-saving efficiency, since the primary receiver is active and the wake-up signal itself is a DCI. LTE GWUS provides a compromise between the energy efficiency of LP-WUS and the latency of PEI. Specifically, LTE GWUS still employs the primary receiver to monitor the wake-up signal, but the signal is realized as a low-complexity physical layer signal rather than a DCI. This yields better power saving compared to PEI, while retaining the low-latency advantage characteristic of PEI.

The network needs to periodically send the paging messages to UEs in the fixed paging frames and paging occasions, which requires regular wake-up and transmission. To achieve network power saving, paging adaptation is introduced in Rel-19, in which the interval of paging transmission is longer.

Observation 8: The DRX/eDRX, GWUS/PEI/LP-WUS, and paging adaptation mechanisms in LTE or NR are beneficial for UE or network power saving.

In NR, DRX and eDRX mechanisms were introduced in different releases to reduce UE power consumption. At present, the two mechanisms are largely overlapping: both reduce UE paging monitoring time within a cycle to achieve power saving, with the only distinction being that eDRX supports longer cycles with an additional PTW. Similarly, PEI and LP-WUS in different releases are also overlapping. The difference lies only in the type of indication signal and the receiver used by the UE to detect it. Such duplication results in management complexity and signalling overhead for both UE and network. Concretely, separate procedures and parameters are required in the NAS and RRC layers.

Observation 9: The DRX and eDRX provide similar functionalities, which results in system complexity. The same problem exists between the PEI and LP-WUS mechanisms.

In 6G, mechanisms similar to DRX/eDRX, PEI/LP-WUS, and paging adaptation should continue to be supported to ensure efficient power control for paging monitoring and transmission. These mechanisms have demonstrated clear benefits in LTE or NR for extending UE battery life and achieving network power saving. However, the system complexity proposed above should be avoided to achieve both simplified protocol implementation and power saving.

Proposal 5: The DRX/eDRX, PEI/LP-WUS, and paging adaptation mechanisms can be considered for 6G. The DRX and eDRX,  also PEI and LP-WUS, can be uniformly designed to avoid duplicated functions.

  - Access control

In LTE and NR, in order to ensure network stability and optimize access efficiency, a set of overload and access control mechanisms were introduced, such as RACH back off, RRC Connection Reject, RRC Connection Release, and UE-based access barring mechanisms.

- RACH back off: When the network detects congestion on the random access channel, the RAR message may include a backoff indication. Upon receiving the backoff indication, the UE selects a random backoff time between 0 and the backoff indication, and shall not initiate a new random access attempt until the expiry of the random backoff time.

- RRC Connection Reject: When the network load is high, the gNB may reject an RRC connection establishment/resume attempt by sending an RRC Connection Reject message. The reject message may include a wait time, during which the UE shall not initiate a new RRC connection establishment/resume request.

- RRC Connection Release: When the network load is elevated, or when no valid user data transfer has been detected for a prolonged period, the network may actively release the UE connection by sending an RRC Connection Release message, thereby conserving radio resources.

- UE-based access barring: Each UE access attempt is associated with the specific Access Category and Access Identities. The network broadcasts the barring information for all Access Categories and Access Identities. Based on the received barring information, the UE autonomously determines whether its access attempt is permitted.

Observation 10: In LTE and NR, access control mechanisms are beneficial to ensure network stability and optimize access efficiency.

In 6G, a unified air interface is envisioned to simultaneously support diverse device types, including eMBB and IoT devices, TN and NTN devices, etc. However, the NR access control parameters do not vary across different device types. For example, the UE-based access barring defines the Access Categories and Access Identities only per service (e.g., MO data, MO signalling, MT access, emergency, MO MMTel voice) in TS24.501 [5] as the Table below.

Table 2: Mapping table for access identities/access categories and RRC establishment cause when establishing N1 NAS signalling connection via E-UTRA connected to 5GCN

| Rule # | Access identities | Access categories | RRC establishment cause is set to |
| --- | --- | --- | --- |
| 1 | 1 | Any category | highPriorityAccess |
| 2 | 2 | Any category | highPriorityAccess |
| 3 | 11, 15 | Any category | highPriorityAccess |
| 4 | 12,13,14, | Any category | highPriorityAccess |
| 5 | 0, 3 | 0 (= MT_acc) | mt-Access |
|  |  | 1 (= delay tolerant) | Not applicable (NOTE 1) |
|  |  | 2 (= emergency) | emergency |
|  |  | 3 (= MO_sig) | mo-Signalling |
|  |  | 4 (= MO MMTel voice) | mo-VoiceCall |
|  |  | 5 (= MO MMTel video) | mo-VoiceCall |
|  |  | 6 (= MO SMS and SMSoIP) | mo-Data |
|  |  | 7 (= MO_data) | mo-Data |
|  |  | 9 (= MO IMS registration related signalling) | mo-Data |
|  |  | 10 (= MO exception data) | mo-ExceptionData (NOTE 3) |
| NOTE 1: A UE using access category 1 for the access barring check will determine a second access category in the range 3 to 7 that is to be used for determination of the RRC establishment cause. See subclause 4.5.2, table 4.5.2.2, NOTE 6. NOTE 2: See subclause 4.5.2, table 4.5.2.1 for use of the access identities of 0, 1, 2, and 11-15. NOTE 3: This applies to the UE in NB-N1 mode. |  |  |  |

Different device types have different service characteristics. For example, the number of IoT devices is excessive, and a lot of IoT devices may request RRC connection establishment simultaneously in order to report water meter readings.

Observation 11: Different device types have different service characteristics. NR access control mechanisms do not differentiate between diverse device types.

While existing overload and access control mechanisms provide certain protection against network overload, they were not designed to address the diverse device types in 6G. To better support and manage different devices in 6G, the access control mechanism should consider diverse device types.

Proposal 6: Access control mechanisms considering diverse device types should be studied.

- Conclusion

In this contribution, we discuss the initial and system access for 6GR. Based on the discussion, we have the following observations and proposals.

System information

Observation 1: Some system information blocks uniformly broadcasting to all device types result in redundant information for certain devices, leading to UE power consumption and longer system information acquisition delay.

Observation 2: The criteria for the suitable cell are included in SIB1 and occupy only a small portion of SIB1. The UE needs to receive the entire SIB1 to obtain the criteria for the suitable cell during the cell selection/reselection processes, resulting in more latency and UE power consumption.

Observation 3: The cell bar/reservation information in the system information is complex.

Observation 4: 5G pain points of area-specific system information are as follows,

- Only a small number of system information blocks can be shared across cells.

- The interaction and coordination between gNBs is complex.

Observation 5: Different system information messages are sent within their respective windows, and they do not overlap with each other. It increases the delay for the UE to obtain all necessary system information, and the power consumption for both network and UE.

Observation 6: The updated system information includes both the changed part and the unchanged part. The unchanged part results in increased power consumption for both network and UE, and longer system information acquisition delay.

Observation 7: In NR, only cell A-assisted NES cell with on-demand SIB1 is specified.

Proposal 1: The system information should be enhanced to address the problems proposed in Observation 1 to Observation 3. The current SIB1 size will be enough with the enhancement and simplification mechanisms.

Proposal 2: One or several system information blocks are defined to include the information elements that are likely to be the same in multiple cells. The area-specific SIB is uniformly deployed without interaction between gNBs.

Proposal 3: The windows of system information messages can overlap with each other. The updated system information only includes the changed content.

Proposal 4: RAN2 to discuss which scenarios below are supported for on-demand SIB/SSB for NON-RRCCONNECTED UEs,

- Cell A-assisted NES cell with only on-demand SIB1;

- Cell A-assisted NES cell with on-demand SSB and on-demand SIB1;

- Standalone NES cell with only on-demand SIB1;

- Standalone NES cell with on-demand SSB and on-demand SIB1.

Paging

Observation 8: The DRX/eDRX, GWUS/PEI/LP-WUS, and paging adaptation mechanisms in LTE or NR are beneficial for UE or network power saving.

Observation 9: The DRX and eDRX provide similar functionalities, which results in system complexity. The same problem exists between the PEI and LP-WUS mechanisms.

Proposal 5: The DRX/eDRX, PEI/LP-WUS, and paging adaptation mechanisms can be considered for 6G. The DRX and eDRX,  also PEI and LP-WUS, can be uniformly designed to avoid duplicated functions.

Access control

Observation 10: In LTE and NR, access control mechanisms are beneficial to ensure network stability and optimize access efficiency.

Observation 11: Different device types have different service characteristics. NR access control mechanisms do not differentiate between diverse device types.

Proposal 6: Access control mechanisms considering diverse device types should be studied.

- References

- RP-252912, Revised SID: Study on 6G Radio

- Draft_RAN2_131bis_MeetingReport_v1

- TR38.875 v17.0.0

- TR38.869 v18.0.0

- TS24.501 v19.1.1