3GPP TSG-RAN WG2 Meeting #132																						         R2-2508911

Dallas, USA, Nov 17 - 21, 2025

Source: 	      ZTE Corporation, Sanechips

Title:                 Consideration on 6G TN-NTN integration

Agenda item:   10.2.2

Document for: Discussion and Decision

# Introduction

In this contribution, a brief review of NR NTN is given. Based on which, essential requirement for 6G NTN are discussed. And the minimum features that need to be considered in the integrated TN-NTN discussion and the NTN specific functionalities, are proposed.

# Overview of 5G NTN

Support of NTN is introduced since R17. After three releases of standardization, 5G NTN is now a communication system supports both NR/IoT devices, various satellite orbits, transparent/regenerative payloads. Below table summarizes the NTN specific enhancements introduced in NR.

Table 1 5G NTN feature overview

| Categories | Function | Introduction |
| --- | --- | --- |
| Common | UL pre-compensation | Mandatory feature in NTN. UE with GNSS capability is assumed, where UE needs to perform UL pre-compensation based on NTN provided assisting information (e.g., ephemeris, common TAs, and etc.) and its GNSS. This includes following: - Support of UE specific TA calculation based on its GNSS-acquired position and the serving satellite ephemeris. - Support of common TA calculation according to the parameters provided by the network (UE considers common TA as 0 if the parameters are not provided) - For TA update in RRC_CONNECTED state, support of combination of both open (i.e. UE autonomous TA estimation, and common TA estimation) and closed (i.e., received TA commands) control loops - Support of pre-compensation of the calculated TA in its uplink transmissions - Support of estimating UE-gNB RTT and delaying the start of RAR window by UE-gNB RTT - Support of frequency pre-compensation to counter shift the Doppler experienced on the service link - Support of determining timing of the scheduling of PUSCH, PUCCH and PDCCH ordered PRACH, CSI reference resource, transmission of aperiodic SRS activation of TA command, first PUSCH transmission in CG Type 2 with cell-specific K_offset if indicated - Support of determining timing of the UE action and assumption on a downlink configuration carried by MAC CE command by K_mac if it is indicated and determining the timing of PDCCH monitoring in recovery search space using K-mac during beam failure recovery procedure - Support of UE receiving cell-specific K_offset/K_mac in system information |
|  | Extended SSB periodicity | Mandatory for R19 NTN UEs. Due to hardware limitation in NTN, additional default 160 ms SSB periodicity is supported for initial cell selection to allow power sharing among satellite beams. |
| CP (exclude mobility) | SIB19 | Mandatory feature in NTN. Dedicated system information (i.e., SIB19) is used to deliver NTN assisting information, including satellite ephemeris information, common TA parameters, scheduling offset (e.g., K-offset) and etc., which is used to assist UE to perform pre-compensation, TA update, measurements and so on. |
|  | NTN specific barred mechanism | Mandatory feature in NTN. Dedicated bar bit (i.e., cellBarredNTN) is introduced to determine whether the cell is barred for NTN access or not. NTN capable UEs ignore the legacy bar bit when cellBarredNTN presents. |
| UP enhancements | UP timer adaption | Mandatory feature in NTN. Below enhancements are considered to cover the long RTT in the system: - The PDCP/RLC timer (e.g., t-reassembly, discardTimer) value ranges are extended to cover transmission delays of long RTT - the start of RAR/contentionReolutionTimer/HARQ-RTT timer are delayed by UE-gNB RTT |
|  | HARQ adaption | Optional features in NTN. To deal with the HARQ stalling issue caused due to long RTT below enhancements are introduced: - Maximum HARQ process number is increased to 32. - DL HARQ feedback disabling and UL HARQ mode is introduced |
| Mobility Enhancements, including measurements | Location based RRM/ CHO | Optional feature in NTN. (Cond)EventD1/D2 is introduced for (quasi)earth fixed/moving cell respectively, which allows measurement report/CHO based on UE location and NTN cell coverage. |
|  | Time based CHO | Optional feature in NTN. CondEventT1 is introduced for earth moving cell to allow CHO based on NTN cell service time. |
|  | PCI unchanged | Optional feature in NTN. Allows group switching UEs to target satellite with reduced signaling overhead. Applicable for satellite switch for earth fixed cell with transparent payload, located in the same gNB. |
|  | SMTC enhancements | Optional feature in NTN. Account for NTN specific characteristics, e.g., extensive coverage, beam hopping mechanism, below SMTC enhancements are introduced respectively for different RRC states: - For connected mode, maximum 4 SMTCs are supported, which is to allow neighbor cell measurements across different satellites. Additionally, propagation delay report, closest reference location report features are used to report UE assistance information to assist SMTC configuration. - For idle/inactive mode, maximum 7 SMTCs are supported, where UE can select up to 4 SMTCs to be used, based on its capability and reference location information provided. |
|  | Location/time-based measurement | Optional feature in NTN. Time/Location based measurement initiation are introduced for measurement relaxation, which allows UE to initiate neighbor cell measurements only when leaving NTN serving cell coverage, or before serving cell stops service. This allows some energy saving as well as smooth cell selection based on predictable NTN cell coverage/service time. |
|  | TN measurement relaxation | Optional feature in NTN. Allows UE to skip TN measurement when it is not within TN coverage for power saving purpose. |
| Others | Geo fencing for broadcast service | Optional feature in NTN. Service area coordinates are introduced in NTN to provide UE with the service areas of broadcast service, including PWS/MBS. UE can use the service area information to assist reception of broadcast service. |

# Common NTN/TN

This section intends to give detailed analysis of features identified in table 1, and give proposals on the minimum set to be considered in TN/NTN design, focusing of RAN2 responsible topics.

## 3.1 Common aspects

Random Access

Both 4step RACH and 2step RACH with contention based or contention free resource are still needed in 6G. Especially for NTN where the propagation delay is large, 2step RACH can significantly reduce the access delays by compressing the 4step access procedure with only 2steps. Therefore, 2step RACH in NTN is a necessary feature to be supported, to help improve the system performance.

Observation 1: 2step RACH can significantly reduce the access delays in NTN, which is necessary to support in 6G day 1 to help improving the NTN performance.

UE GNSS capability

UE with GNSS is a must for 5G NTN access, which is used for pre-compensation for UL transmission, TA handling, measurements, mobility management and etc. Similar to 5G, GNSS based operation is still needed in 6G, especially for a more robust mobility management. Because the RRM measurement cannot provide sufficient distinguish of UEs in NTN cell center or NTN cell edge due to near-far effect. Taking 5G NTN as baseline also allows reusing existing industry chain and its economies of scale to reduce costs. This also ensures a smoother and more commercially mature transition to 6G NTN. Considering RAN1 has not yet studied on the lower layer impacts of GNSS resilience support in NTN (e.g., synchronization, TA, etc.), it is proposed to postpone RAN2 discussion on this until RAN1 makes sufficient progress. Therefore, RAN2 6G discussion for NTN can assume GNSS-based UE as a baseline. While discussion on GNSS resilience can wait for further RAN1 input.

Observation 2: Due to the near-far effect in NTN, location-based mobility with GNSS capability is still needed in 6G for robust mobility management. While RAN2 discussion on GNSS resilience in NTN shall wait for further RAN1 input.

Extended SSB periodicity

In R19 further DL coverage enhancements are discussed. Due to power limitation at satellite side, it is impossible for satellite to activate all beams simultaneously to provide full coverage. Beam hopping mechanism is used to enable power sharing among satellite beams in a time division manner. Therefore, an additional default 160ms SSB periodicity is introduced in initial cell selection, which is a mandatory feature for UE supporting R19 NTN. The extended SSB periodicity is introduced due to real hardware limitations, which shall also be considered in integrated TN-NTN discussion. The extended SSB periodicity has impacts on both RAN2 and RAN1, which can be discussed simultaneously in both groups where RAN2 can focus on RAN2 specific topics, e.g., SI, paging, energy saving in UE/NW side, and etc.

UE power saving/NES

NW energy saving and UE power saving is a key component for both TN and NTN. In NR, different solutions are defined to allow UE power saving and/or NW energy saving, e.g., LP-WUS, Cell DTX/DRX, cDRX, measurement relaxation in NTN and etc. However, companies become to realize that different solutions can increase implementation complexity and compromise the overall gain can be achieved from UE power saving and NES features. For example, if NW and UE support different NES/power saving features, then both UE/NW will not be able to enjoy the power saving gain. Therefore, to allow UE/NW to be benefited from the power saving features defined with mobility between TN-NTN, a unified design to allow both features in TN and NTN is desired to maximum the power saving gain in 6G. To achieve this purpose, when designing the 6G NES/UE power saving solution, the NTN specific characteristics shall be considered. For example, the value range defined for DRX/DTX periodicity, or the timer used to control UE PDCCH monitoring behavior shall be able to comply with the larger SSB periodicity/long RTT in NTN.

Observation 3: 6G common TN-NTN design shall take NTN specific characteristics (e.g., long RTT, large SSB periodicity) into account, and aims for a unified UE power saving/NES design applicable in both NTN-TN to maximum power saving gain.

## 3.2 Control plane

Satellite assisting information over SI

SIB19 is an essential SIB for NTN access, which contains minimum set of satellite assisting information for initial access including satellite ephemeris, epochTime, ntn-UlSyncValidityDuration, common TA parameters, cell specific K-offset and Kmac. Based on these parameters provided, UE can perform pre-compensation, TA adjustment, scheduling offset adjustment and RACH timer (e.g.,RAR/contentionResolutionTimer) handling accordingly. Similarly, 6G NTN system information design shall allow provision of the same amount of minimum satellite assisting information via system information so that UE can perform initial access.

NTN specific barred

NTN specific barred bit is introduced for two purposes, identifying NTN network type and barred UEs from NTN access, e.g., for load balancing. Because some special handling, e.g., GNSS requirement, is still needed for 6G NTN access, the same requirement stands for 6G for system access. That is, in the common TN-NTN design, ways to allow NW type identification (NTN, TN) when accessing the NW and/or bar UE from access is still needed. How to achieve this purpose, e.g., by barred bit in system information, access control, or other methods, can be further discussed at later stage.

Observation 4: For TN-NTN integration, the 6G CP design shall allow providing the minimum satellite assisting information required for initial access in system information, NTN network identification and barring UE from access.

## 3.3 User plane

UP Timer adaption

UP timer adaption as in 5G NTN is also one of the necessary functions for TN-NTN integration since it is used to accommodate the inevitable propagation delays in NTN. For example, the start of RAR/contentionResolutionTimer/HARQ-RTT timers are delayed by UE-gNB RTT so that UE doesn’t wait power when waiting for feedback/scheduling from NW. Or the t-reassembly timer or PDCP discard timer value range is extended to accommodate the propagation delays of transmissions. Considering the detailed UP protocol stack is still under-discussion, which timers are needed in UP are still pending. The detailed handling on how to handle each timer. E.g., whether by extended value range or by delaying the start of the timer can be further discussed in later stage. But the 6G common TN-NTN design shall at least consider similar timer adaptions to fit with NTN characteristics.

HARQ adaption

There are two directions of NTN HARQ enhancements aiming to deal with HARQ stalling caused due to long RTT: one is increased NTN HARQ process number (up to 32), another is to allow reuse of HARQ before one RTT elapses. For the first one, increased HARQ process also means increased UE memory cost, which will depend on UE capability. For the later one, it is done via DL HARQ feedback disabling or UL HARQ mode. When DL HARQ feedback is disabled, or UL HARQ mode B is used, then NW can schedule the same HARQ process before receiving the previously scheduled transmission, this allows fast reuse of HARQ processes.  Although 6G has not yet discussed the detailed HARQ design, it is assumed that similar solution will be needed to deal HARQ stalling issue in NTN. Although there is no HARQ stalling issue in TN, some TN service with delay sensitive but error tolerant characteristics, e.g., real-time interactive services, can also be benefited from DL HARQ feedback disabling or UL HARQ mode, which allows fast scheduling with reused HP processes. Considering HARQ is a quite fundamental function, it is proposed to design a HARQ mechanism common for TN-NTN that supports DL HARQ feedback disabling and UL HARQ mode.

## 3.3 Mobility

Unlike 5G NTN mobility discussion where intra-NTN mobility is prioritized, 6G mobility design shall be discussed considering a unified design for intra-TN, intra-NTN and TN-NTN mobility management to allow seamless mobility between TN and NTN. To support TN-NTN mobility in 6G Day 1, both intra-RAT scenario and inter-RAT scenario between 5G and 6G needs to be studied. The mobility involving NTN shall consider both transparent and regenerative payload. As aforementioned, measurement based mobility may not be reliable in NTN due to the near-far effect. Considering the predictability of satellite coverage, location based as well as time based mobility management is introduced to allow a more robust mobility management mechanism without dependency on RRM measurements. Moreover, a series of SMTC enhancements are introduced to comply with NTN specific characteristics, e.g., extensive cell coverage, beam hopping pattern, which is also needed to be considered when designing 6G measurements mechanism. Similar to TN, NTN also requires an efficient solution that allows efficient L1 cell/carrier switch. For instance, an LTM-like mechanism is useful for frequent satellite beam switching when satellite moves. Therefore, the unified design to support intra TN, intra-NTN, NTN-TN L1/L3 mobility is needed in 6G. And the unified solution needs to consider both NTN and TN characteristics into account to support configurable measurement/time/location based mobility.

It can be observed that the features related to NTN specific characteristics (e.g., long RTT, large coverage, frequent movement) are inevitable factors to be considered in the integrated TN-NTN discussion. Based on above observations and analysis, below proposal is made to identify the minimum sets to be considered in a scalable TN/NTN design. Each aspect is also categorized into different classes for future RAN2 discussion.

Proposal 1: The minimum set to be considered in TN/NTN scalable design includes below aspects:

- (Common) Supports 2-step RACH in Day 1.

- (Common) GNSS-based UEs as baseline for NTN, while discussion on GNSS-resilience can be postponed until RAN1 makes sufficient progress.

- (Common) Extended SSB periodicity needs to be considered in RAN2-centric discussion, e.g., SI, paging.

- (Common) A unified UE power saving/NES solution applicable for TN and NTN, taking NTN specific characteristics, e.g., long RTT, into account.

- (CP) SI includes minimum information for NTN initial access, including ephemeris, epochTime, validity duration, common TA parameters, cell specific Koffset and Kmac.

- (CP) Network type, e.g. TN or NTN, aware cell selection/reselection, access control and system access.

- (UP) UP timer design (e.g., value range or start timing) needs to comply with long RTT in NTN.

- (UP) UP native supports DL HARQ feedback disabling/UL HARQ mode.

- (Mobility) A unified mechanism that allows location/time/measurement based L1/L3 mobility to support intra-RAT or inter RAT mobility between 5G and 6G for intra- TN/intra-NTN/TN-NTN scenarios, considering both transparent and regenerative payload.

# NTN specific feature

In this section, NTN specific features that are worth studying in the first release of 6G are discussed.

- Geo fencing in NTN

With its wide coverage and independence from ground infrastructure, NTN provides a resilient communication for disaster warning and post-disaster relief.  Disaster communication is one of the major components to improve resilience of 6G communication system, where NTN is a key enabler. In NTN, geo fencing is introduced to allow more accurate disaster warning within NTN coverage, and it is based on the same approach defined for CMAS in TN. There are no technical obstacles to support common Geo fencing design for PWS for TN/NTN from RAN2’s perspective, but the decision may be left to SA2 and CT1.

- Multi-orbit mobility

Multi-orbit NTN communication can overcome the fundamental physics-based limitations of individual orbits. For example, GSO cell with its wide earth fixed coverage can provide coverage with robust accessibility with less mobility.  But the long RTT in GSO is not suitable for service with delay requirement. NGSO cell (e.g., LEO) has smaller RTT which is suitable to enable service with delay requirement but the frequent movement of LEO poses challenge on mobility management. With deployment of NTN, NTN communications over different satellite orbits will be common in 6G design, therefore mobility between different orbits is beneficial to be studied in 6G NTN for initial access, cell re-selection and handover, taking 6G cell/carrier design into account. The goal is to maximize usage of multi-orbit resource to enhance coverage, bust capability, while providing service with diverse QoS requirements.

- Mobility due to satellite switch

Another NTN specific mobility for study is mobility caused due to satellite switch, and the typical scenario is earth fixed cell, where partial or complete cell coverage will be switched immediately for all UEs under the same region. Therefore, a group of UEs will need to be switched simultaneously to a cell/carrier served by a different satellite. In 5G, satellite switch with re-sync is introduced for this use case, but the solution is limited to transparent load within the same gNB. The challenge facing when discussing mobility caused due to satellite switch includes: how to group UE, efficient delivery of RRC configurations without causing signaling storm, security upon change of gNB, as well as compatibility of legacy UEs. Now that RAN2 in 6G starts to study a new framework for modular RRC configuration, lower layer security, UE types, we see an opportunity to study again how to improve mobility efficiency upon change of satellite for earth fixed cell, for both transparent and regenerative payload.

- TN coverage based measurement relaxation

Due to larger NTN coverage, the TN coverage overlapped within NTN coverage could be discontinuous or only occasionally presents. In order to save UE power, NR NTN supports UE measurement relaxation based on TN coverage. Based on this approach NW can provide TN coverage in SIB25, and UE can decide to skip measurement on TN frequency when it determines it is not within the corresponding TN coverage, which efficiently saves UE power. The same approach is beneficial in 6G design to allow further UE power saving.

Based on above, below proposal is made:

Proposal 2: Critical NTN specific features to be considered in 6G study includes following:

- Geo fencing of PWS, but confirmation from SA2/CT1 is needed

- Multi-orbit mobility, including cell re-selection and handover

- Mobility enhancement for satellite switch, considering earth fixed cell with regenerative or transparent payload

- TN coverage based measurement relaxation

# Conclusion

Observation 1: 2step RACH can significantly reduce the access delays in NTN, which is necessary to support in 6G day 1 to help improving the NTN performance.

Observation 2: Due to the near-far effect in NTN, location-based mobility with GNSS capability is still needed in 6G for robust mobility management. While RAN2 discussion on GNSS resilience in NTN shall wait for further RAN1 input.

Observation 3: 6G common TN-NTN design shall take NTN specific characteristics (e.g., long RTT, large SSB periodicity) into account, and aims for a unified UE power saving/NES design applicable in both NTN-TN to maximum power saving gain.

Observation 4: For TN-NTN integration, the 6G CP design shall allow providing the minimum satellite assisting information required for initial access in system information, NTN network identification and barring UE from access.

Proposal 1: The minimum set to be considered in TN/NTN scalable design includes below aspects:

- (Common) Supports 2-step RACH in Day 1.

- (Common) GNSS-based UEs as baseline for NTN, while discussion on GNSS-resilience can be postponed until RAN1 makes sufficient progress.

- (Common) Extended SSB periodicity needs to be considered in RAN2-centric discussion, e.g., SI, paging.

- (Common) A unified UE power saving/NES solution applicable for TN and NTN, taking NTN specific characteristics, e.g., long RTT, into account.

- (CP) SI includes minimum information for NTN initial access, including ephemeris, epochTime, validity duration, common TA parameters, cell specific Koffset and Kmac.

- (CP) Network type, e.g. TN or NTN, aware cell selection/reselection, access control and system access.

- (UP) UP timer design (e.g., value range or start timing) needs to comply with long RTT in NTN.

- (UP) UP native supports DL HARQ feedback disabling/UL HARQ mode.

- (Mobility) A unified mechanism that allows location/time/measurement based L1/L3 mobility to support intra-RAT or inter RAT mobility between 5G and 6G for intra- TN/intra-NTN/TN-NTN scenarios, considering both transparent and regenerative payload.

Proposal 2: Critical NTN specific features to be considered in 6G study includes following:

- Geo fencing of PWS, but confirmation from SA2/CT1 is needed

- Multi-orbit mobility, including cell re-selection and handover

- Mobility enhancement for satellite switch, considering earth fixed cell with regenerative or transparent payload

- TN coverage based measurement relaxation

# References

- Draft_RAN2_131bis_Meeting Report_v1

- R2-2507069	Consideration on 6G control plane	ZTE Corporation, Sanechips	discussion	  Rel-20	FS_6G_Radio

- R2-2507169	Consideration on 6G Mobility	ZTE Corporation	discussion 	Rel-20	FS_6G_Radio