3GPP TSG-RAN WG2 Meeting #132                                                               R2-2508912

Dallas, USA, Nov. 17th-21st, 2025

Agenda Item:	8.17

Source:	         ZTE Corporation, Sanechips

Title:	         Discussion on remaining issue for IoT NTN TDD mode

Document for:	Discussion, Agreement

Introduction

In this contribution, we will discuss some remaining issues on IoT NTN TDD mode and present our view.

Discussion

SPS collision

In RAN2#129bis, the following agreement has been made:

=>  When the UL SPS overlaps with non-U NB-IoT subframes UE postpones the UL SPS resource to the next valid UL subframe.

To avoid possible SPS collision between different UEs after the postponement, the following solutions have been provided in RAN2#131bis:

- UE is configured with a (starting) subframe id within the 8 UL NB-IoT subframes for all UL SPS transmissions.

- the values of semiPersistSchedIntervalUL-r15 are interpreted as new values that are multiple of 90ms.

For the solution 1), it can only avoid the start collision of SPS resource between some UEs. For NB-IoT, transmission with repetition is usually used for coverage enhancement, since the value range of  semiPersistSchedIntervalUL-r15 in SR-SPS-BSR-Config-NB-r15 is ENUMERATED {sf128, sf256, sf512, sf1024, sf1280, sf2048, sf2560, sf5120}, which are not multiple of 90ms, even the start subframes of the two UEs SPS are not overlapped, after several periodicity, the subframes of the two UEs SPS may overlapped. For example, the the following figure:

- Figure 1 SPS overlapped example

In which:

- The Radio frame 0/9/18/27/36/45/54 includes uplink subframes

- The SPS resource of UE 1 is started from subframe 3 of radio frame 0,  and the SPS resource of UE 2 is started from subframe 7 of radio frame 9, the semiPersistSchedIntervalUL of both UEs is set to sf128, and the repetition of both UEs are set to 8.

Then the SPS resource of UE1 in radio frame 52 and the SPS resource of UE2 in radio frame 47 will both be postponed to radio frame 54 (e.g. after some SPS periods, the SPS resource of two UEs will inevitably be postponed to the same radio frame including U-subframes), in this case since both UE1/2 need to 8 U frames for SPS transmission, the collision will occur.

So, it cannot  avoid SPS transmission overlap completely by configuring a (starting) subframe id within the 8 UL NB-IoT subframe when the repetition number is larger than 4.

- Observation 1: It cannot avoid SPS transmission overlap completely by configuring a (starting) subframe id within the 8 UL NB-IoT subframe when the repetition number is larger than 4.

For the solution 2), since postpone mechanism is used for UL transmission, e.g

- (RAN2#129bis)When PUR resource start subframe does not align with the UL subframes in the H-SFN, UE postpones the PUR resource start subframe to the next valid UL subframe.

- (RAN1#120bis)NPRACH transmissions are postponed in non-U NB-IoT subframes until the next U NB-IoT subframe(s). The unit of postponement is one PRU

- (RAN1#120)The non-U NB-IoT subframes are not considered by the UE as “NB-IoT UL subframes” from the specifications

  - NPUSCH is mapped as per current specifications.

Thus, even with new values of multiple of 90ms are used for semiPersistSchedIntervalUL-r15, it can only avoid some SPS collisions between different UEs, it cannot avoid collisions between SPS and other resources (e.g. between SPS resource and PUR resource).

Observation 2: Even with new values that are multiple of 90ms are used for semiPersistSchedIntervalUL-r15, the resource collisions(e.g. between SPS and other UL resource) can not be avoided completely.

It can be observed that the UL resource collisions cannot be avoided easily based on the current specification assumption. Considering the objectives of the work item is specify a new NB-IoT TDD NTN mode based on minimum necessary changes to the NB-IoT NTN FDD frame structure and procedures and the SPS in NB-IoT is only used for BSR which is not an essential feature, especially in NTN case. It is suggested to leave it  to network implementation to handle the UL resource collision, e.g. NW proactively (de)active the SPS resource when UL collision occurs.

Observation 3: The SPS is only used for BSR, which is not an essential feature, especially in NTN case.

Proposal 1: No enhancement is needed for SPS configuration for IoT NTN TDD.

Restriction on DL and UL transmissions

In TS36.321, a note on scheduling restriction is given as follow.

NOTE 4: For NB-IoT, for operation in FDD mode, and for operation in TDD mode with a single HARQ process when the associated HARQ RTT timer has been started, DL and UL transmissions will not be scheduled in parallel, i.e. if a DL transmission has been scheduled an UL transmission will not be scheduled until HARQ RTT Timer of the DL HARQ process has expired (and vice versa).

In RAN2#131bis, based on this restriction, the following proposal has been provided[6]

- In NTN TDD mode, UE doesn’t start onDurationTimer within the UL segmented transmission gap.

The intention of this proposal is “during PUSCH transmission procedure, UE does not monitor PDCCH”. But during the PUSCH transmission, drx-InactivityTimer may has already been started. So, it cannot avoid UE to monitor PDCCH only doesn’t start onDurationTimer

Observation 4: During PUSCH transmission, PDCCH monitoring cannot be avoided if only restricting that UE doesn’t start onDurationTimer.

And before v18.3.0, the note is as following:

NOTE 4: For NB-IoT, for operation in FDD mode, and for operation in TDD mode with a single HARQ process, DL and UL transmissions will not be scheduled in parallel, i.e. if a DL transmission has been scheduled an UL transmission will not be scheduled until HARQ RTT Timer of the DL HARQ process has expired (and vice versa).

In v18.3.0, “when the associated HARQ RTT timer has been started” is added for the HARQ feedback disable case in NTN, this does not matter for this discussion.

That means for NB-IoT:

- For operation in FDD mode, DL and UL transmissions will not be scheduled in parallel.

- For operation in TDD mode with a single HARQ process, DL and UL transmissions will not be scheduled in parallel.

- For operation in TDD mode with two HARQ processes, DL and UL transmissions can be scheduled in parallel.

The first restriction(i.e. For operation in FDD mode) is mainly because the half-duplex FDD restriction, UL transmission and DL reception cannot be performed simultaneously.

The second restriction(i.e. For operation in TDD mode with a single HARQ process) is mainly because the time gap is necessary between UL and DL switch for NB-IoT UE with one single Tx/Rx chain.

There is no restriction for operation in TDD mode with two HARQ processes, because UL transmission and DL reception will not be performed simultaneously in TDD(no half-duplex FDD issue), and no time gap limitation between UL and DL switch for NB-IoT UE with two Tx/Rx chain. With same situation, for IoT NTN TDD mode, UL transmission and DL reception will not be performed simultaneously(no half-duplex FDD issue), and no time gap limitation issue between UL and DL switch because the gap between U-subframe and D-subframe is large enough. Thus, from technique point of view, the scheduling restriction on DL and UL transmissions need not apply to IoT NTN TDD mode.

Observation 5: From technique point of view, the scheduling restriction on DL and UL transmissions need not apply to IoT NTN TDD mode.

But considering the objectives of the work item is specify a new NB-IoT TDD NTN mode based on minimum necessary changes to the NB-IoT NTN FDD frame structure and procedures, both RAN2 and RAN1 assumed that NB-IoT TDD NTN mode UE follows FDD behaviour unless specified otherwise. Considering that DL and UL transmissions will not be scheduled in parallel for FDD, there is no critical issue if this restriction applies to IoT NTN TDD mode, the only shortcoming is that the service delay maybe larger than not applying, which is tolerable in IoT NTN TDD system with sparse valid subframes.

Observation 6: For scheduling restriction on DL and UL transmissions, there is no critical issue if IoT NTN TDD follows FDD behaviour.

Currently, the PDCCH monitoring behaviour during PUSCH/PDSCH transmission procedure is specified in TS 36.213 as follows. If RAN2 decides that the scheduling restriction on DL and UL transmissions does not apply to IoT NTN TDD mode, UE will need to monitor PDCCH during PUSCH/PDSCH transmission procedure, which has not been considered in RAN1’s discussion. It is preferred not to introduce enhancements that will impact specification in the maintenance stage, unless it will broke the system.

For a NPDCCH UE-specific search space, if a NB-IoT UE is configured with higher layer parameter twoHARQ-ProcessesConfig or npusch-MultiTB-Config and if the NB-IoT UE detects NPDCCH with DCI Format N0 ending in subframe n, and if the corresponding NPUSCH format 1 transmission starts from n+k or in a NTN serving cell, from an uplink subframe which, after accounting for uplink transmission timing, overlaps with downlink subframe n+k, - if the corresponding NPDCCH with DCI format N0 with CRC scrambled by C-RNTI schedules two transport blocks as determined by the Number of scheduled TB for Unicast field if present, the UE is not required to monitor an NPDCCH candidate in any subframe starting from subframe n+1 to subframe n+k-1, otherwise the UE is not required to monitor an NPDCCH candidate in any subframe starting from subframe n+k-2 to subframe n+k-1; and - the UE does not expect to receive a DCI Format N0 before subframe n+k-2 for which the corresponding NPUSCH format 1 transmission ends later than subframe n+k+255 if the corresponding NPDCCH with DCI format N0 schedules one transport block. - for TDD, and if the corresponding NPUSCH format1 transmission ends in subframe n+m, the UE is not required to monitor NPDCCH in any subframe starting from subframe n+ k to subframe n+m-1. otherwise - if the NB-IoT UE detects NPDCCH with DCI Format N0 ending in subframe n or receives a NPDSCH carrying a random access response grant ending in subframe n, and if the corresponding NPUSCH format 1 transmission starts from n+k or in a NTN serving cell, from an uplink subframe which, after accounting for uplink transmission timing, overlaps with downlink subframe n+k, the UE is not required to monitor NPDCCH in any subframe starting from subframe n+1 to subframe n+k-1. - for TDD, if the NB-IoT UE detects NPDCCH with DCI Format N0 ending in subframe n or receives a NPDSCH carrying a random access response grant ending in subframe n, and if the corresponding NPUSCH format 1 transmission ends in n+k, the UE is not required to monitor NPDCCH in any subframe starting from subframe n+1 to subframe n+k.

Observation 7: Current RAN1 specs assumes the PDCCH monitoring behaviour during PUSCH/PDSCH transmission procedure for IoT NTN TDD follows FDD behaviour.

Proposal 2: To avoid impacting specification much, RAN2 confirms that IoT NTN TDD follows FDD behaviour for the scheduling restriction on DL and UL transmissions.

RIL H600

RIL Id; WI; Class; Title; Tdoc; Delegate; Misc; File version; Status. H600; IoTTDD; 2; Enabling CB-Msg3-EDT configuration in IoT TDD; xxx; Huawei (Xubin); V008; Todo. [Description]: During R2#131 meeting, we made the following agreement: - We aim at introducing the needed changes (if any) to support CB-Msg3 and CB-Msg4 for IoT NTN TDD mode during the ASN.1 review, including at least - Adding configurations that are aligned with the 90 ms periodicity for npusch-Periodicity and windowPeriodicity-NB (FFS other needed enhancements based on the progress of the CB-Msg3-EDT discussion this week) However, it has not been thoroughly discussed what the configuration looks like in IoT NTN TDD. For IoT NTN, we assume one window covers some consecutive PUSCH occasions, which are not too far from each other. In IoT NTN TDD, the frame structure looks like below: If we make PUSCH occasion periodic, it means we can only configure one PUSCH occasion within one 8.28ms UL period. In that case, the window size would be incredibly large to cover multiple PUSCH occasions. This makes CB-Msg3 useless as the replicas are very far from each other; If we make PUSCH occasion periodic only within one 8.28ms UL period, it makes some sense. In this case, the window size is reasonable since it only needs to cover one 8.28ms UL period at most, and it can still be periodic with the 90ms periodicity. Then we need to configure a set of PUSCH occasions which repeat every 90ms. We also need to decide whether the PUSCH occasions are configured at every UL period or only some (which requires more complex signalling) We need to confirm whether this is the intention and whether there is anything more to consider and then discuss what parameters are required for this. [Comments]: Rapporteur’s comment: Contributions are invited on this so that we can discuss during the meeting.

For this RIL, we can discuss based on the IoT NTN configuration.

In IoT NTN, the maximal PUSCH periodicity is sf256 (e.g. 256ms), and the maximal cb-Msg3-TxWindow is 128ms, both of them are larger than 90ms, which can work.

cb-Msg3-TimeResource-r19 SEQUENCE { pusch-Periodicity-r19 ENUMERATED {sf2, sf4, sf8, sf16, sf32, sf64, sf128, sf256}, npusch-StartSFN-r19 INTEGER (0..1023), npusch-StartSubframe-r19 INTEGER (0..9) ... }, ... cb-Msg3-TxWindow-r19 SEQUENCE { windowSize-r19 ENUMERATED {n1, n2, n4, n8, n16, n32, n64, n128}, windowPeriodicity-r19 ENUMERATED {n1, n2, n4, n8, n16, n32, n64, n128} OPTIONAL --Need OP } CB-Msg3-ConfigSIB-NB field descriptions. .... cb-Msg3-TxWindow-NB CB-Msg3 transmission window configuration. The start time of the CB-Msg3 transmission window is aligned with the NPUSCH start time indicated by npusch-StartSFN-r19 and npusch-StartSubframe-r19. When cb-Msg3-NumOfReplicas-NB-r19 equals 1, this field is absent. When windowPeriodicity-NB is absent, the window periodicity uses the same value as windowSize-NB. Value n8 corresponds to 80ms, n16 corresponds to 160ms and so on..

For IoT NTN TDD, the valid subframes are less than that in IoT NTN, it is reasonable to configure larger periodicity and larger window size in IoT NTN TDD than that in IoT NTN. So, the agreement of “Adding configurations that are aligned with the 90 ms periodicity for npusch-Periodicity and windowPeriodicity-NB” can be followed.

Further more, in IoT NTN, absolute SFN and subframe is used to indicate the NPUSCH start time. Since 90ms cannot align with the 1024 SFN period, how to align the NPUSCH start time with the U-subframe should be considered. For simple, the postpone mechanism can be used, e.g. if the subframe of the NPUSCH start time is not a U-subframe, the NPUSCH start time is postponed to the nearest U-subframe.

Proposal 3: Stick to the RAN2 agreement of “Adding configurations that are aligned with the 90 ms periodicity for npusch-Periodicity and windowPeriodicity-NB”.

Proposal 4:  If the subframe of the NPUSCH start time is not a U-subframe, the NPUSCH start time is postponed to the nearest U-subframe.

- Conclusion

Based on the discussion we propose the following:

- Observation 1: It cannot avoid SPS transmission overlap completely by configuring a (starting) subframe id within the 8 UL NB-IoT subframe when the repetition number is larger than 4.

Observation 2: Even with new values that are multiple of 90ms are used for semiPersistSchedIntervalUL-r15, the resource collisions(e.g. between SPS and other UL resource) can not be avoided completely.

Observation 3: The SPS is only used for BSR, which is not an essential feature, especially in NTN case.

Observation 4: During PUSCH transmission, PDCCH monitoring cannot be avoided if only restricting that UE doesn’t start onDurationTimer.

Observation 5: From technique point of view, the scheduling restriction on DL and UL transmissions need not apply to IoT NTN TDD mode.

Observation 6: For scheduling restriction on DL and UL transmissions, there is no critical issue if IoT NTN TDD follows FDD behaviour.

Observation 7: Current RAN1 specs assumes the PDCCH monitoring behaviour during PUSCH/PDSCH transmission procedure for IoT NTN TDD follows FDD behaviour.

Proposal 1: No enhancement is needed for SPS configuration for IoT NTN TDD.

Proposal 2: To avoid impacting specification much, RAN2 confirms that IoT NTN TDD follows FDD behaviour for the scheduling restriction on DL and UL transmissions.

Proposal 3: Stick to the RAN2 agreement of “Adding configurations that are aligned with the 90 ms periodicity for npusch-Periodicity and windowPeriodicity-NB”.

Proposal 4:  If the subframe of the NPUSCH start time is not a U-subframe, the NPUSCH start time is postponed to the nearest U-subframe.

- Reference

- RP-243293, Revised WID on introduction of IoT-NTN TDD mode

- R2-131bis_NES_NR-NTN_IoT-NTN_(Sergio)_ EOM

- Draft_RAN2_129bis_Meeting_Report_v2

- Draft_Minutes_report_RAN1#120b_v010

- Draft_Minutes_report_RAN1#120_v030

- R2-2507048 - Discussion on IoT NTN TDD mode, OPPO, Oct, 2025

- R4-2511349 [R19][NTN IoT TDD] (IoT_NTN_TDD) Big CR 36.133 R19 UE RRM (Cat-B), August, 2025