3GPP TSG-RAN WG2 Meeting #132	R2-2508919

# Dallas, USA, 17 - 21 November, 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.306 | CR | 1392 | rev | - | Current version: | 17.14.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction on UE capabilities on RLF report |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R2 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_ENDC_SON_MDT_enh-Core |  |  |  |  |  | Date: |  |  | 2025-11-03 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Radio Link Failure report feature was introduced in NR Rel-16, and it is mainly for MRO (Mobility Robustness Optimization) purpose. In Rel-16, RLF reporting is mandatory supported without UE capability. In Rel-17, RLF report enhancements were introduced, and relevant UE capabilities were also introduced, such as rlfReportCHO-r17, rlfReportDAPS-r17. For Rel-17 RLF report enhancements, according to TS 38.331, if the UE supports one of handover types (such as CHO or DAPS) and there is a handover failure related to the handover type, the UE should log RLF report with Rel-16 defined content, and the inclusion of the handover type related content depends on whether the UE supports handover type related UE capabilities or not. However, for the following definitions in this specification, they may be interpreted as follows: this capability means the UE supports to log RLF report only when a specific handover type happens. In this case, the definitions are inaccurate and they are not aligned with UE behaviours in TS 38.331. rlfReportCHO-r17 Indicates whether the UE supports RLF-Report for conditional handover. rlfReportDAPS-r17 Indicates whether the UE supports RLF-Report for DAPS handover. |  |  |  |  |  |  |  |  |
| Summary of change: |  | For the UE capability rlfReportCHO-r17, the definition is changed into: Indicates whether the UE supports the delivery of conditional handover related information in the RLF-Report. For the UE capability rlfReportDAPS-r17, the definition is changed into: Indicates whether the UE supports the delivery of DAPS handover related information in the RLF-Report. Impact Analysis Impacted functionality: RLF report Inter-operability: There is no inter-operability issue since this CR only impacts UE side. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | If this CR is not agreed, there may be a misalignment regarding UE RLF report behaviours between TS 38.331 and TS 38.306. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.2.17 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | X | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

4.2.17	SON parameters

| Definitions for parameters | Per | M | FDD-TDD DIFF | FR1-FR2 DIFF |
| --- | --- | --- | --- | --- |
| onDemandSI-Report-r17 Indicates whether the UE supports delivery of on-Demand SI information upon request from the network as specified in TS 38.331 [9]. | UE | No | No | No |
| pscell-MHI-Report-r17 Indicates whether the UE supports the storage of PSCell mobility history information and the reporting in UEInformationResponse message as specified in TS 38.331 [9]. | UE | No | No | No |
| rach-Report-r16 Indicates whether the UE supports delivery of RA report upon request from the network. | UE | No | No | No |
| rlfReportCHO-r17 Indicates whether the UE supports the delivery of conditional handover related information in the RLF-Report. | UE | No | No | No |
| rlfReportDAPS-r17 Indicates whether the UE supports the delivery of DAPS handover related information in the RLF-Report. | UE | No | No | No |
| success-HO-Report-r17 Indicates whether the UE supports the storage and delivery of Successful Handover Report upon request from the network as specified in TS 38.331 [9]. | UE | No | No | No |
| twoStepRACH-Report-r17 Indicates whether the UE supports the storage and delivery of 2-step RACH related information upon request from the network as specified in TS 38.331 [9]. | UE | No | No | No |