3GPP TSG-RAN WG2 Meeting # 132	R2-2508972

# Dallas, USA, 17th – 21st November 2025

Source:	Google Inc.

Title:	Discussion on RRC Structure and (re)configuration

Agenda Item:	10.3.2.2

Document for:	Discussion and Decision

- Introduction

According to previous RAN2 meeting agreements, serval key study items were initiated to address 6G RRC design:

  - ASN.1 is used for encoding of RRC signaling for 6G air interface (same as NR).

  - Study overall RRC structure, configuration improvements, improve readability of ASN.1 for RRC signalling

  - Study how to efficiently, reliably and unambiguously configure UEs while keeping signalling size small (e.g. improvements to delta signaling or no delta signaling).

  - RAN2 will study modular design of RRC for 6G.  Further study how to modularize RRC e.g. based on features, functions, verticals, etc.

Based on agreement 3 and 4, this contribution provides further discussion.

- Discussion

2.1 RRC Configuration Model

In the 5G system, a fundamental trade-off exists. Full RRC configurations are robust and simple to implement but result in very large message sizes. Delta configurations are small but have introduced implementation complexity and ambiguity. Therefore, delta configurations are almost not supported between different vendors’ gNBs.

According to previous RAN2 meeting discussions, 5G delta signaling introduces ambiguities and implementation robustness issues, which were identified as a source of potential security vulnerabilities. Given these issues, the 6G RRC configuration model study should first perform a root cause analysis of 5G delta signaling ambiguities. This analysis should investigate issues at multiple levels:

- Specification-Level Ambiguity: The 5G RRC specification (using ASN.1) uses need codes (e.g., need-S, need-R) to manage the lifecycle of optional parameters. The logical rules for these codes, especially when nested, are extremely complex and can be ambiguous. This leads to different vendor interpretations and interoperability failures, resulting in a state mismatch.

- Misalignment between UE and target gNB: During handover, a target gNB may misinterpret parts of the UE's context received from the source gNB over the Xn interface (e.g., due to different vendor implementations or software versions). The target gNB then builds delta configurations based on its own incorrect understanding of the UE's context. When the UE applies these delta configurations, a mismatch occurs.

- Ambiguous Failure Handling: The 5G "reconfiguration failure" procedure, where a UE reverts to its last known good configuration, creates ambiguity. The network cannot be certain if the UE successfully reverted, leading to desynchronization.

Given these deep-rooted issues at multiple levels, we think 6G should not adopt delta signaling. Instead, the 6G should adopt a "replace" path, supported by non-delta signaling. We believe this approach will significantly reduce the number and complexity of issues that need to be discussed and resolved compared to supporting delta signaling.

Proposal 1: To ensure a robust and unambiguous design, 6G RRC shall adopt non-delta signaling as the baseline configuration mechanism.

2.2 RRC modular design for 6G.

If Proposal 1 is agreed, adopting a configuration model based on the full replacement of a single module (e.g., MeasConfig) is the next logical step for RAN2 to discuss. This approach eliminates ambiguity while maintaining a small signaling size.

In the previous meeting, RAN2 had following agreement - “RAN2 will study modular design of RRC for 6G.  Further study how to modularize RRC e.g. based on features, functions, verticals, etc.”. We propose using these three grouping methodologies as a starting point for discussion.

- Grouping by Function/Layer

This methodology (e.g., Module_PHY, Module_MAC) organizes RRC parameters horizontally, aligning with the 3GPP protocol stack.

- Advantage: This approach provides a clean specification structure and is a logical way to manage the inter-WG dependencies with RAN1 and RAN4.

- Challenge: This structure is poorly aligned with functional capabilities. A single feature (e.g., Sidelink) requires configuration across multiple modules (PHY, MAC, RLC). This may also challenge 6G goals for scalability, as a simple IoT device might be forced to parse large, complex modules for functions it does not support.

- Grouping by Feature

This methodology (e.g., Module_Mobility, Module_Sidelink) groups all parameters related to a specific technical capability, regardless of their protocol layer.

- Advantage: This method is intuitive for device implementation and supports a scalable "opt-in" model. A device only needs to support the modules for the features it implements.

- Challenge: This approach introduces a significant risk of inter-dependencies and parameter duplication. For example, Module_Mobility and Module_NTN may both need to configure similar MAC or PHY parameters, and the management of this overlap must be studied.

- Grouping by Verticals

This methodology (e.g., Module_V2X, Module_URLLC) groups all parameters required to support a specific service category or use case.

- Advantage: This approach aligns directly with the 6G service-oriented architecture and goals, such as network slicing.

- Challenge: This presents the highest risk of parameter duplication, as multiple verticals (e.g., eMBB, URLLC) all require common functions like mobility, but with different performance parameters. As 3GPP specifies functions and features, this approach may be difficult to specify. It also creates complexity for UEs that must support multiple verticals simultaneously.

| Grouping module | Primary logical | pros | cons |
| --- | --- | --- | --- |
| Function/Layer | Protocol Stack (Horizontal) | Clean specification, manages WG dependencies. | Poorly aligned with features and use cases. |
| Feature | Technical Capability | Intuitive for device builds, very scalable. | High risk of inter-dependencies and duplication. |
| Verticals | Service/use case | Aligns well with business/service requirements. | Highest risk of duplication, hard to specify. |

Table 2.2-1 Pros and cons of different grouping modules

In a non-delta signaling model, the standard practice for an update is the full replacement of a single module. Therefore, the most suitable grouping methodology should minimizes the number of modules that must be replaced for a typical configuration change.

Base on above analysis, we think the Grouping by Feature is the most suitable baseline for the following two reasons:

- Efficient Updates: Configuration updates are often driven by specific technical capabilities or features. If parameters are grouped by feature, updating that specific feature likely requires replacing only that single corresponding module.

- Scalability: This grouping supports a scalable model where devices only need to support and parse modules for the specific features they actually implement.

In contrast, other grouping methods have significant drawbacks for this use case:

- Grouping by Function/Layer: This method is poorly aligned with functional capabilities. Because a single feature (e.g., Sidelink) often requires parameters across multiple layers (e.g., PHY, MAC, and RLC), enabling or updating that feature would require simultaneously replacing multiple modules, which is less efficient.

- Grouping by Verticals: This method carries the highest risk of parameter duplication. If a common parameter used by multiple verticals changes, it might require replacing multiple vertical modules simultaneously to effect that single change.

Proposal 2: Grouping by Feature shall be the baseline for the 6G RRC modular design study.

- Conclusion

In conclusion, we propose the followings for 6G:

Proposal 1: To ensure a robust and unambiguous design, 6G RRC shall adopt non-delta signaling as the baseline configuration mechanism.

Proposal 2: Grouping by Feature shall be the baseline for the 6G RRC modular design study.

- References

[1] 3GPP TR 38.914 V0.2.0, "Study on 6G Scenarios and Requirements

[2] RAN2#131bis Chair note