3GPP TSG-RAN WG2 Meeting #132	R2-2508975

# Dallas, USA, Nov. 17th – 21st, 2025

Source: KDDI Corporation

Title:　 Considerations on RRC Modelling and connection management for 6GR

Agenda Item:	10.3.2.1

Document for:	Discussion

- Introduction

The new SID for “study on 6G Radio” was approved in RAN#108 meeting and was revised in RAN#109 meeting, with the objective to develop 6GR to meet the requirements of the next generation of RAN, including those related to the control plane. Based on that, RAN2#131bis had initial discussion and the following agreements for the future study. In this contribution, we would like to share our considerations regarding RRC state modelling.

Agreements

Continue study

-	Idle mode –

-	Connected mode –

-	Inactive mode or sub-state: Should support at least on these functions: UE based mobility (e.g. cell selection/reselection), UE context and identification and energy efficient for both UE and NW.  Further study fast transitions, data transfer, further improvements to UE identification, and RNA management and paging/wake-up simplifications/enhancements.   We can consider the pros/cons of different modelling, after having some discussions on the functionality.

- Discussion

  - Design Principle for 6GR RRC state modelling

Before discussing the design of 6G RRC state modeling, we would like to briefly revisit the evolution of RRC states across generations.

In 3G, two main RRC states were specified: RRC_CONNECTED and RRC_IDLE. Within RRC_CONNECTED, several substates were defined, including CELL_DCH, CELL_FACH, CELL_PCH, and URA_PCH. Furthermore, within CELL_FACH, CELL_PCH, and URA_PCH, there were additional conditions such as in-service and out-of-service, which can be considered sub-substates. These states were introduced to optimize resource usage and power consumption. However, their complexity posed significant challenges for implementation and operational efficiency.

In 4G, the RRC state model was simplified to only two states: RRC_CONNECTED and RRC_IDLE, which greatly improved system manageability and facilitated efficient commercial deployment.

In 5G, the RRC_INACTIVE state was introduced to enhance UE power efficiency by enabling faster transitions between RRC_IDLE and RRC_CONNECTED. However, due to implementation complexity—particularly on the core network side—and limited functional gains, RRC_INACTIVE has not seen widespread commercial deployment.

As we move toward 6G, we are open to studying additional RRC states beyond RRC_CONNECTED and RRC_IDLE. However, we believe that simplicity must be a key design principle to ensure practical implementation and broad adoption.

Proposal 1: The design principle for 6G RRC state, including additional RRC states, if any, should prioritize functional distinctiveness and simplicity.

  - Initial analysis of RRC_CONNECTED sub-state and RRC_INACTIVE

Energy efficiency for both UE and NW

In RRC_INACTIVE, the gNB releases the radio connection with the UE and retains only the 5GC-RAN context, enabling lower energy consumption and reduced radio resource usage. If the purpose of introducing a RRC_CONNECTED sub-state in 6GR is to eliminate the need for RACH and RRC Resume Request procedures to enable faster state transitions, then the UE may need to maintain some level of connection with the gNB.

Proposal 2: RAN2 clarify whether the purpose of introducing a RRC_CONNECTED sub-state is to enable faster state transitions, in which case it may not be expected to achieve the same level of energy efficiency as RRC_INACTIVE.

Synchronization for fast state transition

When a UE in RRC_INACTIVE wants to resume its connection, it performs RACH procedure followed by sending an RRC Resume Request, which helps the UE to re-establish synchronization and timing alignment with gNB.

If a RRC_CONNECTED sub-state is introduced in 6GR to enable faster state transitions compared to RRC_INACTIVE, the UE would likely need to maintain some level of radio connection or synchronization with the gNB. This introduces a technical challenge:

How can the UE maintain uplink and downlink synchronization without an active radio connection?

One possible approach could be that the UE in the RRC_CONNECTED sub-state periodically wakes up to transmit or receive synchronization signals or lightweight control signaling, allowing it to retain timing alignment without a full connection. However, such mechanisms would require careful design and may introduce specification and implementation complexity.

Proposal 3: RAN2 clarify that if a RRC_CONNECTED sub-state is introduced for faster state transitions, mechanisms to maintain some level of uplink and downlink synchronization will be needed, which may introduce specification and implementation complexity.

Paging and DRX related

In RRC_INACTIVE, the UE is required to monitor paging from both RAN and CN. This introduces additional complexity in terms of configuration, paging cycles, and wake-up procedures, especially when DRX and eDRX are involved.

If a RRC_CONNECTED sub-state is introduced in 6GR, it will be helpful that RAN2 clarify whether a paging mechanism is needed for this state. If paging is required, it may be beneficial to study the feasibility of a simplified paging mechanism that can be commonly applied to both RRC_IDLE and RRC_CONNECTED sub-state, in order to reduce complexity and improve energy efficiency.

Proposal 4: RAN2 study whether a paging mechanism is needed for RRC_CONNECTED sub-state. If yes, then a simplified paging mechanism that can be applied to both RRC_IDLE and RRC_CONNECTED sub-state would be beneficial.

UE identity

For RRC_INACTIVE, an I-RNTI is assigned to the UE to identity its context for NR connected to 5GC. In contrast, for a UE in RRC_CONNECTED, a C_RNTI is used as a unique identifier of the RRC connection and for scheduling purposes.

If a RRC_CONNECTED sub-state is defined in 6G, a new UE identifier would likely be required to differentiate between the sub-state and the normal RRC_CONNECTED state. However, as both the RRC_INACTIVE and RRC_CONNECTED sub-state are not needed simultaneously, an I-RNTI like identifier can be used or reused for the RRC_CONNECTED sub-state.

Proposal 5: RAN2 clarify that if a RRC_CONNECTED sub-state is defined in 6GR, a new UE identifier may be required to differentiate it from the normal RRC_CONNECTED state. However, this approach may not introduce significant differences compared to RRC_INACTIVE in terms of UE context handling.

UE Context and UE based mobility

If a RRC_CONNECTED sub-state is introduced in 6GR and is expected to support UE-based mobility, such as cell selection and cell reselection, then it implies that the UE would not maintain an active radio connection with the gNB. In this case, the UE would autonomously select and reselect cells, similar to its behavior in RRC_INACTIVE.

Furthermore, when the UE moves to a new cell, it would need to report its location, for example via RNA update, just as in RRC_INACTIVE. This means the UE context would still be retained by the network, but without active radio signaling.

As a result, the functional behavior of the RRC_CONNECTED sub-state under UE-based mobility would be very similar to RRC_INACTIVE, raising the question of whether a new state is necessary, or if enhancements to RRC_INACTIVE would be more efficient.

Proposal 6: RAN2 clarify that a RRC_CONNECTED sub-state may not differ significantly from RRC_INACTIVE in terms of UE context management and UE-based mobility behavior.

  - RRC services and functions

For RRC services and functions,

At minimum, the following basic functionalities can be considered

- Broadcast of system information,

- Paging initiated by 5GC or NG-RAN

- Establishment, maintenance and release of an RRC connection between UE and NG-RAN, including add/mod/rel of carrier aggregation

- Security function including key management

-Establishment, configuration, maintenance and release of signaling Radio Bearers(SRBs) and data radio beers(DRBs)

- Qos management, as Qos configuration is delivered to the UE via RRC

- UE measurement reporting and control of the reporting, due to the L3HO will still be required as a fallback mechanism, the RRC based UE measurement-related procedures will be needed.

-Detection and recovery from radio link failure due to the supportive of L3HO, as well as the unpredictable radio condition of the spectrum

-NAS message transfer to/from NAS from/to UE due to the fact that RRC serves as the signalling interface between the UE and the NW, enabling the delivery of messages such as, registration, authentication, etc.

Regarding to other functions, such as add/mod/release of Dual connectivity, these can be wait until further progress is made in the dual connectivity/dual stack-related discussion. Mobility-related functions will be discussed in section 10.4, there it can be revisited once a clearer picture of the 6G mobility mechanism.

Proposal 7 : The following functionalities can be considered as a start point for the RRC services and functions:

Broadcast of system information;Paging;support add/mod/rel of carrier aggregation;support add/mod/rel of SRBs and DRBs;Qos management;UE measurement-related procedures;radio link failure;NAS message delivery

- 3. Conclusion

Proposal 1: The design principle for 6G RRC state, including additional RRC states, if any, should prioritize functional distinctiveness and simplicity.

Proposal 2: RAN2 clarify whether the purpose of introducing a RRC_CONNECTED sub-state is to enable faster state transitions, in which case it may not be expected to achieve the same level of energy efficiency as RRC_INACTIVE.

Proposal 3: RAN2 clarify that if a RRC_CONNECTED sub-state is introduced for faster state transitions, mechanisms to maintain some level of uplink and downlink synchronization will be needed, which may introduce specification and implementation complexity.

Proposal 4: RAN2 study whether a paging mechanism is needed for RRC_CONNECTED sub-state. If yes, then a simplified paging mechanism that can be applied to both RRC_IDLE and RRC_CONNECTED sub-state would be beneficial.

Proposal 5: RAN2 clarify that if a RRC_CONNECTED sub-state is defined in 6GR, a new UE identifier may be required to differentiate it from the normal RRC_CONNECTED state. However, this approach may not introduce significant differences compared to RRC_INACTIVE in terms of UE context handling.

Proposal 6: RAN2 clarify that a RRC_CONNECTED sub-state may not differ significantly from RRC_INACTIVE in terms of UE context management and UE-based mobility behavior.

Proposal 7 : The following functionalities can be considered as a start point for the RRC services and functions:

Broadcast of system information;Paging;support add/mod/rel of carrier aggregation;support add/mod/rel of SRBs and DRBs;Qos management;UE measurement-related procedures;radio link failure;NAS message delivery