3GPP TSG-RAN WG2 Meeting #132							R2-2508995

Dallas, United States, Nov. 17th-21st

Agenda item:		8.4.3

Title: 	Consideration on LP-WUS Other Issues

Source: 			ZTE Corporation, Sanechips

Document for: 	Discussion and Decision

Introduction

In this contribution, we will discuss some remaining issues on LP-WUS other Issues and present our views.

Discussion

- About 38304-1: Discuss whether and how to capture the common code point in TS 38.304

- Based on the description in TS38.213 [1] as below, the code point of LP-WUS includes subgroup index iSG and PO index iPO. And the subgroup index iSG determination in a PO and the code point determination for all subgroups in the PO have already been defined clearly.

## 10.4C PDCCH monitoring activation by WUS in RRC_IDLE/RRC_INACTIVE ... A paging occasion associated with a WUS occasion has index iPO=UE_ID mod N⋅NS+i_s mod NPOWO where NPOWO is a number of paging occasions associated with a WUS occasion, N, NS, iSG, and i_s are defined in [17, TS 38.304], and UE_ID is defined in clause 7.1 of [17, TS 38.304]. If a number of NSGPO subgroups per paging occasion, provided by subgroupNumber-PO-WUS, is NSGPO>1, the codepoint for the subgroup index iSG in a PO iPO is iPO*NSGPO+1+iSG, and the codepoint for all subgroups in the PO is (iPO+1)*NSGPO+1-1; otherwise, the codepoint for the PO iPO is iPO. - ...

- We understands “common code point associated with all subgroups in a PO” in TS38.300 corresponds the “the code point for all subgroups in the PO” in TS38.213, “reference to TS 38.213(e.g. as specified in clause 10.4C in TS 38.213) [4]” based on TS38.213)” should be added to make it clearer.

- In TS 38.304, it has already captured that “If the UE detects LP-WUS and the LP-WUS is associated with the UE as specified in clause 10.4C in TS 38.213 [4],” as follows. And based on TS38.213, LP-WUS associated with the UE includes the UE’s subgroup and all subgroups in the PO, it is clear enough.

### 7.5.0 General The UE supporting LP-WUS may monitor LP-WUS in RRC_IDLE and RRC_INACTIVE states in order to reduce power consumption. If LP-WUS configuration is provided in system information, the UE in RRC_IDLE or RRC_INACTIVE state supporting LP-WUS (except for the UEs expecting MBS group notification) may start LP-WUS monitoring using LP-WUS parameters in system information according to the procedure described below if the entry condition in clause 7.5.1 is fulfilled. The UE may stop LP-WUS monitoring and follows the paging monitoring procedure as described in clause 7.1 or 7.2 if the exit condition in clause 7.5.1 is fulfilled. If the UE detects LP-WUS and the LP-WUS is associated with the UE as specified in clause 10.4C in TS 38.213 [4], the UE monitors the associated PO as specified in clause 7.1 or monitors PEI as specified in clause 7.2 if PEI is supported and PEI configuration is provided in system information, which is up to UE implementation. If the UE does not detect an LP-WUS on the monitored LP-WUS occasion (LO) or the LP-WUS is not associated with the UE as specified in clause 10.4C in TS 38.213 [4], the UE is not required to monitor the associated PO as specified in clause 7.1. - ...

- Observation 1: “the LP-WUS is associated with the UE ” in TS38.304 has already referred to TS38.213(e.g. as specified in clause 10.4C in TS 38.213) [4],” and it has been described clearly in TS 38.213.

- Proposal 1(38304-1): In TS 38.300, add a reference to TS38.213 (e.g. as specified in clause 10.4C in TS 38.213) to “common code point associated with all subgroups in a PO”.

About 38304-2: Discuss how to capture the agreement “LR measurement based RX level and cell quality value should be derived by UE implementation in multi-beam operations. We assume this conclusion does not impact the cell reselection procedure.”

- Considering that the UE measurement behavior has been defined in section 5.1.53 and 5.1.54 of TS 38.215[2], it is clear enough.

- Proposal 2(38304-2): No RAN2 specification impact for the LR measurement based RX level and cell quality value derived by UE implementation in multi-beam operations. The UE measurement definition can be based on TS 38.415.

- About 38304-3: Discuss whether and how to capture the agreement in RAN1 LS R1-2508170.

- --------------------------------- RAN1 LS R1-2508170---------------------------------

In RAN1#122bis, the following clarification was made regarding the LO determination based on lpwus-LoFrameOffsetList:

Agreement

The LO determination based on lpwus-LoFrameOffsetList is clarified as follows:

- If the number of POs associated with a LO is less than Ns (the number of POs per PF), the UE uses the (i_sNPOWO+1)-th value in offsetForLongerWakeUpDelay (if configured) and the (i_sNPOWO+1)-th value in offsetForShorterWakeUpDelay (if configured) provided by lpwus-LoFrameOffsetList.

- For the case when both offsetForLongerWakeUpDelay and offsetForShorterWakeUpDelay are configured,

  - If the gap between the end of the last LP-WUS MO the UE would monitor in the LO associated with the value in offsetForShorterWakeUpDelay and the start of the corresponding PO is no less than the wake-up delay a UE reports, the UE monitors the LO associated with the value in offsetForShorterWakeUpDelay;

  - Otherwise if the gap between the end of the last LP-WUS MO the UE would monitor in the LO associated with the value in offsetForLongerWakeUpDelay and the start of the corresponding PO is no less than the wake-up delay a UE reports, the UE monitors the LO associated with the value in offsetForLongerWakeUpDelay;

  - Otherwise, the UE monitors legacy PO.

- For the case when only one of offsetForLongerWakeUpDelay and offsetForShorterWakeUpDelay is configured,

  - If the gap between the end of the last LP-WUS MO the UE would monitor in the LO associated with the value in offsetForLongerWakeUpDelay or offsetForShorterWakeUpDelay (whichever is configured) and the start of the corresponding PO is no less than the wake-up delay a UE reports, the UE monitors the LO;

  - Otherwise, the UE monitors legacy PO.

RAN1 respectfully asks RAN2 to capture the above agreement in TS 38.304.

- --------------------------------- RAN1 LS R1-2508170---------------------------------

- Considering the agreement in RAN1 LS R1-2508170 is about the parameter to be used among offsetForLongerWakeUpDelay and offsetForShorterWakeUpDelay provided by lpwus-LoFrameOffsetList., which is necessary for UE to select the parameter to be used among multiple parameters, it should be described clearly. There are two methods to capture it:

- Method1: Captured in field description of offsetForLongerWakeUpDelay and offsetForShorterWakeUpDelay.

- Method2: Captured in TS38.304.

- Considering that offsetForLongerWakeUpDelay and offsetForShorterWakeUpDelay need to be captured together, some redundant description cannot be avoided if we go for method 1. So, it is better to capture this agreement in TS38.304, e.g. in section of 7.5.0(General).

- Proposal 3(38304-3): Capture the agreement in RAN1 LS R1-2508170 in TS38.304, e.g. in section of 7.5.0(General).

- Based on the proposal 1, we provide the TP in Annex.

- Conclusion

Based on the discussion we propose the following:

- Proposal 1(38304-1): In TS 38.300, add a reference to TS38.213 (e.g. as specified in clause 10.4C in TS 38.213) to “common code point associated with all subgroups in a PO”.

- Proposal 2(38304-2): No RAN2 specification impact for the LR measurement based RX level and cell quality value derived by UE implementation in multi-beam operations. The UE measurement definition can be based on TS 38.415.

- Proposal 3(38304-3): Capture the agreement in RAN1 LS R1-2508170 in TS38.304, e.g. in section of 7.5.0(General).

- Based on the proposal 1, we provide the TP in Annex.

- Reference

- 3GPP TS 38.213 V19.1.0 (2025-09)，NR; Physical layer procedures for control

- 3GPP TS 38.215 V19.0.0 (2025-06)，NR; Physical layer measurements

- R1-2508170 LS on the LO determination based on lpwus-LoFrameOffsetList for LP-WUS operation in IDLE/INACTIVE mode，RAN1#122bis, Oct, 2025

- Annex-TP for TS 38.300

### 9.2.5 Paging ... UE power saving for paging monitoring: in order to reduce UE power consumption due to false paging alarms, the group of UEs monitoring the same PO can be further divided into multiple subgroups. With subgrouping, a UE shall monitor PDCCH in its PO for paging if the subgroup to which the UE belongs is paged as indicated via associated PEI and/or LP-WUS. If a UE cannot find its subgroup ID with the PEI and/or LP-WUS configurations in a cell or if the UE is unable to monitor the associated PEI and/or LP-WUS occasion corresponding to its PO, it shall monitor the paging in its PO. If the UE is configured with both LP-WUS and PEI, and it cannot find its subgroup ID with the LP-WUS or if the UE is unable to monitor the LP-WUS it monitors the following PEI or paging in its PO. In addition to monitoring a codepoint associated with its subgroup ID, a UE configured with LP-WUS monitoring also monitors a common codepoint associated with all subgroups in a PO, as specified in clause 10.4C in TS 38.213) [37]. ...