3GPP TSG-RAN WG2 Meeting #132                                                                      R2-2509003

Dallas, USA, Nov. 17th-21st

Agenda item:	10.3.3.3

Source: 	ZTE Corporation，Sanechips

Title: 	Views on AS security aspects

Document for:	Discussion and Decision

Introduction

In this paper, we discussed remaining access stratum security aspects including security of system information, security management for mobility enhancement, and security protection for UP control PDUs.

Discussion

2.1  Security protection of system information

Broadcast system information is transmitted to/acquired by all UE in RRC_IDLE and RRC_INACTIVE. Functionalities that relies on the system information include camping/cell selection/reselection, paging monitoring, access control, initial access procedure, public warning system (PWS) and others.

There has been study on 5G security enhancements against False Base Stations (FBS) in Release 18 with TR 33.809 as output but not formally specified.  We understand the main reason is the study is performed too late for 5G. Considering the security protection for system information has impact on all UEs it is too difficult to deploy such mechanism and cover all UEs in the late stage of a generation.

Following is the key issue that is identified in the Rel-18 study on the security protection of system information[5]. Two main security threats are identified due to lack of protection of SI message: DoS attack on UE and Rogue services.

## 5.2 Key Issue #2: Security protection of system information ### 5.2.1 Key issue details Broadcasting system information (SI) is one of the functions of the RRC protocol, defined in TS 38.331 [2]. A cell periodically broadcasts synchronization signals and SI. These broadcasted messages are intended for all UEs which are camping on a cell. In the idle mode or inactive mode, the UE monitors the SI of cells and choose a suitable cell to camp on. The UE typically acquires the SI from the cell and performs initial access to transition to connected state to obtain services. The system information includes information, among others, like cell (re-)selection parameters, neighbouring cell information, frequency priority, blocklisted cell, common channel configuration information, NAS common information, and public warning system (PWS) messages. In general, the system information is applicable for UEs in RRC_IDLE, RRC_INACTIVE, and RRC_CONNECTED. UE in IDLE mode performs PLMN selection, monitor paging, performs cell selection, cell re-selection, and applies access control before making an access attempt. In future releases, other services such as MBMS, proximity services, etc. are also likely to be supported by UEs in IDLE mode. This key issue is about investigating if and how a new protection mechanism could be introduced against over-the-air attackers who broadcast rogue SI messages or replay previously captured SI messages as-is (without modification). Since SI messages are broadcast messages meant for all UEs, it is not apparent that an integrity and replay protection is strictly necessary. Nevertheless, in general, an integrity and replay protected SIs could add security value by at least making it difficult for over-the-air attackers to succeed in using a rogue SI or a previously captures SI at a later time, e.g., to lure UEs using SI messages with incorrect neighbouring cells, and to send self-crafted or old PWS messages. SA3 has studied the PWS security in TR 33.969 [3], in which the security mechanisms used to protect the Warning notifications message from Public Warning System against false base station can be reference for the study in the present document. There are some distinct challenges when studying the broadcast messages protection as below: a) Key management. It is because of heterogenous trust-boundaries, and diverse regulations (or requirements) per countries (or regions); b) Time synchronization. It is because of difficulty to achieve fairly acceptable time synchronization between one gNB and other gNBs, and between UEs and gNBs; c) Signaling complexity. It is because of restrictive signaling expected from UEs in RRC_IDLE. Nevertheless, it is only prudent if the 5G system could be enabled (i.e., support) to achieve protection of SI messages in general. NOTE 1: This key issue is concerned with the "over-the-air" interface. Therefore, integrity protection of SI "within-the-network" is not in the scope of this key issue. ### 5.2.2 Security Threats Lack of protection of SI could potentially have following impacts in some cases: - DoS attack on UE - Rogue services ### 5.2.3 Potential Requirements 5G system should provide a means to ensure a UE in any RRC state is able to determine the authenticity of system information obtained from a cell.

Observation 1:There has been study on 5G security enhancement against False Base Station (FBS) in Rel-18 but not normally specified. One reason is that it is too late to adopt such enhancement for 5G.

For 6G, SA1 has studied the False Base Station (FBS) attacks and following requirements have been captured by SA1 in TR 22.870[4]. It is identified that the attacks because of a FBS are a major security flaw in the existing communication system. The potential requirement needed to support the use case that 6G system shall support a means for a UE to able to distinguish a False base Station from an authentic Base Station.

### 5.5.3 Use case on UE selecting a base station after assessing its legitimacy #### 5.5.3.1 Description The majority of the efforts in the mobile communication system security has been focused on delivering secure communication once connected, whereas security related to bootstrapping those connections are relatively less explored. A UE can interact with a Base Station without assessing its legitimacy, which opens way for diverse attacks. False Base Station (FBS) attacks are one such example and its potential to cause active and passive impacts are an alarming concern, worldwide. Addressing fundamental security issues is important and can entail a system level reappraisal of risk and expectations. This is most feasible to undertake at the beginning of a new mobile generation. Attacks mounted by a FBS are a major security flaw in the existing communication system. There is an important opportunity to address this concern in the next generation wireless systems. The sooner that security threats are addressed in the process of interaction between components of the system, the more risks can be reduced to the 3GPP system. By increasing the level of security between the radio network entities and the UE during the initial network selection procedure, FBS attacks can be prevented at earlier stages of communication establishment. One such example is the introduction of an efficient Base Station authenticity verification mechanism such that the UE can verify the authenticity of the Base Station prior to starting the 6G registration procedure. #### 5.5.3.2 Potential New Requirements needed to support the use case [PR 5.5.3.2-1] Subject to operator policy and regulatory requirements, the 6G system shall support a means for a UE to be able to distinguish a False Base Station from an authentic Base Station.

Observation 2:	For 6G, SA1 has captured a potential new requirement in TR 22.870 that 6G system shall support a means for a UE to be able to distinguish a FBS from an authentic base station. This should be adopted as a day 1 feature in 6G to ensure all 6G UEs are protected.

The security requirements for broadcast system information should be determined in SA3 first. RAN2 needs to wait for SA3’s conclusion on the details of the solution for Base Station authenticity verification. However, there are RAN2 considerations on system information broadcast and the general structure of SIBs for 6G that should be taken into account when the framework is designed. Thus, the overall solution for secure system information broadcast should be developed jointly by SA3 and RAN2..

In TR 33.809, several mechanisms are captured as potential solutions secure system information broadcast. One option is that the base station digitally signs the broadcast system information with asymmetric key algorithm or performs HASH on the system information with symmetric algorithm. The digital signature or the HASH value is carried in broadcast message for UE to verify. Depending on the specific algorithm, the length of the carried HASH or the digital signature ranges from 32 bits to 256 bytes. Considering the size limitation of SI message, which is 2976bits in 5G, the overhead caused by such digital signature is not trivial.

Further, other aspects of the potential solutions may have further impact on the overall overhead and then impact the design of 6G system information. Such aspects include how frequent the digital signature/HASH needs to be transmitted, whether only one digital signature/HASH for all system information is enough or each SIB/SI message requires a separate digital signature/HASH, and whether we need to design a separate SIB to carry the digital signature.

Thus, we suggest that RAN2 can be involved in the solution development of the Base Station authenticity verification mechanism as early as possible, if SA3 agreed to introduce a solution for 6G.

Observation 3:	The potential solution for Base Station Authenticity verification impacts 6G system information design considering the following aspects:

- The length of digital signature/HASH impacts the SIB size (ranges from 32 bits to 256 bytes).

- The transmission frequency of the digital signature/HASH impacts the SIB design and overhead.

- Whether a separate SIB is needed to carry the digital signature/HASH or is it transmitted per SIB/SI message.

Based on above, we propose RAN 2 to send an LS to SA3 and ask SA3 to provide the details of broadcast system information security framework. Specifically to understand the impact to overall SIB size and frequency of SIB transmissions.

Proposal 1: 	For system information security, RAN2 should send an LS to SA3, offering to work jointly with SA3 on a solution and should ask SA3 to provide answers to the following questions:

- Which parts of the system information needs security protection?

- Is the security protection token provided per SIB block or will there be a security protection token for a set of protected SIBs

Finally, from RAN2 perspective we need to study how to specify the functionality. Traditionally, for UP integrity protection, we specify this in PDCP protocol. However, SIB protection framework may be completely different to the normal radio bearer integrity protection handling. Specifically, the SIBs are fully self-contained and are specified in RRC spec. SIBs are mapped to BCCH logical channel and this has no PDCP header. Thus, specifying the system information security in PDCP layer doesn’t seem logical. So, we propose that the SIB security mechanism is captured fully in the RRC specification.

Proposal 2: 	RAN2 should capture the security protection mechanism for system information directly in the RRC spec (i.e. treat this as a separate procedure from the radio bearer security which is captured in PDCP)

2.2 Security key management for mobility enhancements

In legacy NR specification, the security keys used for CP and UP are separate algorithm keys, both derived from the same AS layer security key, i.e. kgNB. These algorithm keys are updated at the same time whenever their associated AS layer security key is updated: whenever a new AS layer security key is derived, both CP keys and UP keys are re-derived based on this new AS layer key.

In a RAN architecture with split CU-CP and CU-UP, PDCP entities for DRBs reside in the CU-UP. In some network deployments, a single CU-UP entity can be shared among multiple CU-CPs. With the legacy key derivation mechanism, even if the serving CU-UP of a UE remains unchanged during a handover, the UP key will still be updated whenever the AS key is updated. This leads to traffic interruption due to PDCP re-establishment and RLC/MAC reset that is performed during the handover procedure.

If the security key for CP and UP can be updated separately during handover, the following benefit can be achieved in the CU-CP/CU-UP slit architecture:

- Frequent UP key updates can be avoided

- No need to perform PDCP re-establishment

- RLC reset and MAC reset may be avoided; already received RLC segments need not to be discarded.

We believe this kind of mechanism will bring significant improvement to mobility performance. Although the final requirements and mechanism should be determined by SA3, RAN2 can study the necessity of introduction of such mechanism. The study can be focused on analyzing the impact on mobility performance. UP traffic interruption during handover should be the main evaluation metrics.

Observation 4:	Some benefits can be observed to allow the change of CP key without the change of UP key (to minimize the requirement on key refresh and reduce the interruption time during mobility).

On the other hand, whether CU/DU split and CU-CP/CU-UP split are supported in 6G is still pending and decision will be made after June 2026. Thus, RAN2 can decide whether to start the study after RAN plenary make the decision.

Proposal 3: 	Security separation between CP and UP should be studied to minimize the interruption time for UP during mobility. RAN2 should ask SA3 to study the feasibility of the solutions for CP UP key separation for 6G.

2.2 Security protection of UP control PDUs

In 5G, the PDCP and RLC control signalling (status reports and ROHC/EHC control signalling etc) are not security protected. It is unclear whether security protection is needed for this signalling in case of 6G. It is also unclear whether this mechanism comes under the bracket of lower layer control signalling security framework (which is generally aimed at protection of MAC signalling and we already initiated some discussion with SA3 on this). To understand the requirements for this, it is first proposed that RAN2 discusses the security protection needs for the PDCP/RLC control PDUs and if there is consensus, we should ask SA3 to investigate any security requirements for this.

Proposal 4: 	PDCP and RLC control signalling in 5G is currently not security protected. RAN2 should consult SA3 to understand if security is needed for these messages.

Conclusion

On security protection for system information

Observation 1:There has been study on 5G security enhancement against False Base Station (FBS) in Rel-18 but not normally specified. One reason is that it is too late to adopt such enhancement for 5G.

Observation 2:	For 6G, SA1 has captured a potential new requirement in TR 22.870 that 6G system shall support a means for a UE to be able to distinguish a FBS from an authentic base station. This should be adopted as a day 1 feature in 6G to ensure all 6G UEs are protected.

Observation 3:	The potential solution for Base Station Authenticity verification impacts 6G system information design considering the following aspects:

- The length of digital signature/HASH impacts the SIB size (ranges from 32 bits to 256 bytes).

- The transmission frequency of the digital signature/HASH impacts the SIB design and overhead.

- Whether a separate SIB is needed to carry the digital signature/HASH or is it transmitted per SIB/SI message.

Proposal 1: 	For system information security, RAN2 should send an LS to SA3, offering to work jointly with SA3 on a solution and should ask SA3 to provide answers to the following questions:

- Which parts of the system information needs security protection?

- Is the security protection token provided per SIB block or will there be a security protection token for a set of protected SIBs

Proposal 2: 	RAN2 should capture the security protection mechanism for system information directly in the RRC spec (i.e. treat this as a separate procedure from the radio bearer security which is captured in PDCP)

On security key management for mobility enhancements

Observation 4:	Some benefits can be observed to allow the change of CP key without the change of UP key (to minimize the requirement on key refresh and reduce the interruption time during mobility).

Proposal 3: 	Security separation between CP and UP should be studied to minimize the interruption time for UP during mobility. RAN2 should ask SA3 to study the feasibility of the solutions for CP UP key separation for 6G.

on Security protection of UP control PDUs

Proposal 4: 	PDCP and RLC control signalling in 5G is currently not security protected. RAN2 should consult SA3 to understand if security is needed for these messages.

Reference

- RAN2#131bis Chairman notes

- RP-251881, New SID: Study on 6G Radio, June 2025.

- S3-252973, Study on Security for the 6G System, August, 2025.

- 3GPP TR 22.870 V0.4.1, “Study on 6G Use Cases and Service Requirements”

- 3GPP TR 33.809 V18.1.0, “Study on 5G security enhancements against False Base Stations (FBS)”