3GPP TSG RAN WG2#132	R2-2509006

Dallas, USA, 17th - 21st, November 2025

Source:	TCL

Title:	Discussion on TN/NTN Integration

Agenda item:	10.2.2

Document for:	Discussion and decision

# Introduction

The integration of Non-Terrestrial Networks (NTNs) into 5G has evolved significantly through 3GPP 18, 19, and 20 Releases. 6G is expanding NTN’s as native component in 6Gnetworks for achieving the ultimate objective of ubiquitous global connectivity for IUT IMT-2030 vision. This contribution discusses, cell architecture, initial access and RRC modelling for harmonized 6G TN/NTN radio system.

# Discussion

To achieve the objective of IMT-2030 vision of global ubiquitous connectivity, there is an ongoing discussion in both SA and RAN 3GPP working groups on 6G system and radio access architecture encompassing both terrestrial and non-terrestrial network access/infrastructure from day-one. Having a unified TN/NTN design from day-one is crucial for covering the remaining coverage gaps such as oceans, mountains, rural areas, and even airspace for drones and airplanes connectivity. Moreover, a unified TN/NTN design enables seamlessly support of mobility and services that cross between terrestrial and satellite links without the complexity of adding new technology, features, or components to the 6G initial protocols later on.

According to recent progress on 6G, 3GPP SA WGs are currently discussing several use-cases and requirements such as HAPS-based rapid deployable network for public safety, ubiquitous emergency rescue via UAVs, immersive media services for advanced air mobility enabled by 6G NTN, and enhanced user experience with sparse LEO satellite deployment - demanding a unified 6G TN/NTN design [1]. On the other hand, 3GPP RAN WGs are also aiming at supporting NTN deployment scenarios with clear objectives towards harmonizing 6G Radio design for TN and NTN [2-3]. Designing a unified 6G access system that encompasses terrestrial networks and multi-orbit platforms (UAVs, HAPS, LEO, MEO, and GEO satellites) is very complex. These platforms have diverse physical and operational characteristics (Table 1), requiring careful design for the cell addressing, radio interface, and radio protocol signalling configuration, as detailed in Sections 2.1, 2.2 and 2.3.

Figure 1: 6G Terrestrial and Non-terrestrial System

## Unified Cell Architecture

In a unified 6G TN/NTN environment, a User Equipment (UE) would very likely to be located (at any given time) within a coverage of a multi-layered heterogeneous radio access types, such as terrestrial base stations, UAVs, HAPS, and satellite  (as illustrated in Figure 1). The strategic sharing of frequency bands across TN and NTN platforms in 6G would make coverage overlapping issue inevitable, which in turn creates significant challenges for cell addressing. This complicates critical procedures like intelligent initial cell selection, re-selection and efficient handover, as a UE must distinguish between identical-frequency cells from radically different platforms. For instance, a UE must differentiate between C-band (n77/n78) signals from a terrestrial macro-cell, a MEO satellite, and a GEO satellite, all offering broadband. Similarly, it must identify whether a Ka-band (n258) transmission originates from a HAPS platform, a LEO/MEO satellite, or a terrestrial mmWave node. This shared spectrum creates a complex RF environment where traditional cell identification methods are insufficient.

Table 1. Show spectrum sharing across TN and NTN platform

| Platform | Frequency Band | 3GPP Band(s) | Range | Common Use Cases | Real-World Examples |
| --- | --- | --- | --- | --- | --- |
| UAV (Drones) | C-band | n48 (CBRS) | 3.55–3.7 GHz | Command/control, surveillance | DJI Aeroscope (n48) |
| ATG | Sub-6 GHz | n78 | 3.3– 3.8GHz |  |  |
|  | Ku-band | - | 12–18 GHz | Military backhaul | General Atomics MQ-9 (12.7–13.25 GHz) |
| HAPS | Ka-band | n258 | 24.25–27.5 GHz | Stratospheric broadband | Google Loon (n258) |
|  | V-band | n260 | 37–40 GHz | Experimental HAPS (internet access) | Facebook Aquila (n260) |
| LEO Satellites | Ku-band | - | 12–18 GHz | Consumer broadband downlink | Starlink (12–14 GHz) |
|  | Ka-band | n257 | 26.5–29.5 GHz | Gateway links | Starlink Gen2 (n257) |
|  | L-band | n255 | 1.6 GHz | Direct-to-cell IoT/voice | AST SpaceMobile (n255) |
| MEO Satellites | C-band | n77 | 3.7–4.2 GHz | Broadband services | SES O3b (n77) |
|  | Ka-band | n258 | 26.5–40 GHz | High-throughput payloads | Iridium Next (n258) |
| GEO Satellites | C-band | n77 | 3.7–4.2 GHz | Fixed broadband | Viasat (n77) |
|  | Ku-band | - | 12–18 GHz | DTH TV, enterprise VSAT | HughesNet (14 GHz downlink) |
|  | Ka-band | n257 | 26.5–29.5 GHz | High-throughput GEO | Inmarsat Global Xpress (n257) |
|  | L-band | n255 | 1.6 GHz | Maritime/aviation safety | Inmarsat (n255) |
| Terrestrial | Sub-6 GHz | n78 (C-band) | 3.3–3.8 GHz | Urban/macro coverage | T-Mobile 5G (n78) |
|  | mmWave | n258/n260 | 24–40 GHz | Ultra-high-speed hotspots | Verizon 5G Ultra Wideband (n260) |

A. Multi-Layer Cell Stack Problem: At any location, a UE will be within range of a vertical stack of cells from different network layers (e.g., a terrestrial cell, a UAV, a HAPS platform, and a LEO satellite). Current cell 5G addressing schemes, which use a ground-referenced Mapped Cell ID, are insufficient as they ignore altitude and the 3D nature of the network. This would prevent UE from intelligent network node selection. For example, an in-flight UE in air (e.g., a drone UE or in a helicopter,) should select a HAPS or LEO cell, not a ground-based cell.

B. Non-Invertible PCI Confusion:  The shared spectrum and mobility of NTN platforms make coverage overlap a common scenario. Under such a scenario it is very likely that a terrestrial cell, a LEO satellite beam, and a HAPS beam covering the same geographical area to share the same Physical Cell Identity (PCI) range (0-1007). In such a case, a UE detecting a specific PCI cannot natively determine whether it originates from a terrestrial base station or a non-terrestrial platform. This leads to pervasive PCI confusion, where a serving cell has multiple neighboring cells using the same PCI, resulting in:

- Failed Connection Attempts: Applying inappropriate access parameters leads to radio link failure.

- Incorrect Handover/reselection Decisions: The network may hand the UE to the wrong cell type.

- Increased Handover Delays: Resolving the ambiguity requires extra measurements, increasing latency and causing potential call drops.

- Observation 1: The multi-dimensional coverage and shared spectrum in a unified 6G TN/NTN network introduce critical cell addressing challenges, including the multi-layer cell stack problem and non-invertible PCI confusion, which hinder intelligent initial access and mobility management

- Proposal 1: Study and specify new TN/NTN-aware cell addressing and identification architecture for 6G robust initial access and seamless mobility across TN and NTN layers.

## Unified TN/NTN Radio Interface

A unified 6G air interface must support diverse platforms with vastly different characteristics: propagation delays ranging from milliseconds (TN) to hundreds of milliseconds (GEO), extreme Doppler shifts, heterogeneous link budgets, and distinct channel conditions. Key asymmetries exist in coverage areas, beam characteristics, and dynamic topologies - with LEO constellations and UAVs requiring frequent handovers compared to relatively static GEO and terrestrial networks (Table 2). Current 5G initial access and RRC framework face significant challenges in these mixed environments that demand a flexible waveform and air interface configuration. This creates a fundamental requirement for unified and TN/NTN platform-agnostic initial access and connection management of 6G air interface.

- Observation 2: The significant physical layer disparities between TN and NTN platforms necessitate unified access and RRC modelling procedures that can automatically adapt to different waveform and TN/NTN platform signal characteristics.

- Proposal 2: Study unified initial access and connection setup procedures allowing UE to access 6GR irrespective of TN/NTN platform specific configurations.

| Parameter | UAV (Drone) | HAPS | LEO Satellite | MEO Satellite | GEO Satellite | Terrestrial (TN) |
| --- | --- | --- | --- | --- | --- | --- |
| Altitude | 50 m – 10 km | 18–25 km | 500–1,500 km | 8,000–20,000 km | 35,786 km | 10–50 m (Macro) |
| Cell Size (Diameter) | 1–10 km | 50–200 km | 50–500 km | 200–1,000 km | 200–3,500 km | 0.1–3 km (Macro) |
| Beam Type | Narrow (Directional) | Wide (Multi-beam) | Spot Beams (Multi-beam) | Wide Beams | Fixed Continental | Sectorized (120°) |
| Propagation Delay (RTD) | 0.3–3 ms | 0.6–8 ms | 3–15 ms | 50–130 ms | 250–280 ms | 0.1–1 ms |
| Doppler Shift | ±1 kHz | ±5 kHz | ±100 kHz | ±20 kHz | ±1 kHz | ±500 Hz |
| Cell Handover Frequency | Minutes (~0.1–5 km) | Minutes–Hours (~30 km) | 2–10 Seconds (Orbital) | 5–60 min (~3.9 km/s) | None (Geostationary) | User mobility only |
| Atmospheric Loss | Tropospheric | Minimal | Ionospheric (<6 GHz) | Ionospheric | Rain fade (>10 GHz) | Minimal (LOS/NLOS) |
| Bit Error Rate (BER) | 10⁻⁴ – 10⁻⁶ | 10⁻⁵ – 10⁻⁷ | 10⁻⁴ – 10⁻⁶ | 10⁻⁵ – 10⁻⁷ | 10⁻³ – 10⁻⁵ | 10⁻⁶ – 10⁻⁹ |

Table 2: TN/NTN Multi-platform Parameters Variations

## Unified Radio Protocol Signalling

To support satellite access in 5G, the RRC layer introduces extended timers (T300, T304) for connection handling and broadcasts critical parameters via SIB19 (NR) and SIB31 (LTE/NB-IoT), including satellite ephemeris data for real-time positioning, Doppler pre-compensation, and timing adjustments to mitigate propagation delays. The MAC layer optimizes scheduling with K-offset adjustments and uplink synchronization mechanisms, while expanding HARQ processes (up to 32 slots) to accommodate long round-trip times. It also reduces active HARQ processes (from 16 to 4) to prevent buffer overflow, extends Timing Advance (up to 0.86 ms), and modifies RACH procedures with longer response windows (up to 40.96 ms). The RLC layer increases the ARQ window size to prevent stalling and adjusts polling frequency, while the PDCP layer extends reordering timers (t-Reordering) and expands duplicate detection windows to handle out-of-order packets from variable satellite delays.

These 5G radio enhancements (continuously sending adjustments or compensation parameters) are implemented per TN and NTN or per GEO and non-GEO (NGO) satellites. However, a radio protocol based on continuous sending of variable parameters per TN/NTN or per GEO and NGO satellites over the radio interface is not optimal for 6G's unified TN/NTN system. This is due to the asymmetrical requirements of each platform in 6G's multi-orbit and multi-platform scenario, which may require excessive information exchange over the air interface (Figure 2).

- Observation 2: Unlike 5G, which considered TN and NTN separately, 6G radio protocol design must account for and be optimized for the diverse physical parameters and operational characteristics of different TN and NTN platforms from the beginning.

- Proposal 2: Study a unified RRC signalling framework that is inherently adaptable to the parameter variations across terrestrial and multi-orbit non-terrestrial platforms avoiding per-platform specific compensations and excessive signalling overhead.

Figure 2: Non-terrestrial Multi-platform Specific Constraints

# Conclusion

In this contribution, we discussed initial access and RRC modelling in harmonized 6G TN/NTN radio system provided the following observations and proposals:

- Observation 1: The multi-dimensional coverage and shared spectrum in a unified 6G TN/NTN network introduce critical cell addressing challenges, including the multi-layer cell stack problem and non-invertible PCI confusion, which impact initial access and prevent intelligent cell selection and mobility management.

- Proposal 1: Study and specify new TN/NTN-aware cell addressing architecture for 6G robust initial access and seamless mobility across TN and NTN layers.

- Observation 2: The significant physical layer disparities between TN and NTN platforms necessitate unified access and RRC modelling procedures that can automatically adapt to different waveform and TN/NTN platform signal characteristics.

- Proposal 2: Study unified initial access and RRC modelling techniques to allow UE to access 6G unified system irrespective of TN/NTN platform-specific configurations.

- Observation 3: Unlike 5G, which considered TN and NTN separately, 6G radio protocol design must account for and be optimized for the diverse physical parameters and operational characteristics of different TN and NTN platforms from the beginning.

- Proposal 3: Study and specify a unified 6G-radio protocol signalling framework that is inherently adaptable to the parameter variations across terrestrial and multi-orbit non-terrestrial platforms avoiding platform-specific compensations and excessive signalling overhead.

# References

- TR 22.870: "Study on 6G Use Cases and Service Requirements".

- TR 38.914: "Study on 6G Scenarios and Requirements"

- RP-252912: " Revised SID: Study on 6G Radio“ NTT DOCOMO, CMCC, AT&T, Vodafone Beijing, China, September 15-18, 2025,