3GPP TSG-RAN WG2 Meeting #132		R2-2509015

Dallas, USA, Nov. 17th – 21th, 2025

Source:	BYD

Title:	Discussion on functionality management for RRM measurement prediction

Agenda Item:	9.3.2

Document for:	Discussion and Decision

- Introduction

In the RAN#109 meeting, the objectives of the Rel-20 AI mobility work item on RRM measurement prediction for UE sided model have been agreed [1] as follows.

The objectives are as the following: - Specify support for RRM measurement prediction and measurement event prediction for UE sided models [RAN2]: - Temporal domain prediction case A and case B, and Frequency domain prediction for co-located cases are supported. - L3 Cell level predication is supported Note 1: For UE sided model, spatial domain prediction is not supported. Note 2: For UE sided model, L3 beam level prediction is not supported. - For both RRM measurement prediction and measurement event prediction, specify signaling and protocol aspects to enable LCM functionality management for UE sided model including [RAN2]: - UE capability request and response procedure - Applicability report for both full and partial configuration approach - Inference configuration and inference report - Performance monitoring performed in UE or network side, based on which network can manage UE sided functionality - UE sided data collection - For measurement event prediction: - Measurement event A1~A6 are in the scope in this release Note3: Data transfer and model delivery for UE sided model are out of scope of this WI. And model related choices can be up to UE’s implementation.

In this contribution, we will provide our consideration on RRM measurement prediction for UE sided model at the aspects related to inference configuration and reporting.

- Discussion

Temporal domain Case B:

In RAN2#131 meeting, the following agreement was made for temporal domain case B.

RAN2#131 agreement - For the interpretation of “skipping pattern” in temporal domain Case B, RAN2 confirm that it refers to SSB configuration to indicate the timing of NW's SSB transmission—not timing of UE's SSB measurement/skipping.

According to the agreement above, RAN2 has confirmed that the skipping pattern can be defined as an SSB configuration. Therefore, we need to consider how to configure it, and two options can be considered as follows.

- Option 1: Introduce dedicated SMTC configuration.

- Option 2: Muting/masking legacy SSB transmission.

For option 1, allow the UE to skip some legacy SSB transmissions by reconfiguring its SMTC. For example, the periodicity of the dedicated SMTC can be larger than legacy SMTC. To avoid the impact of legacy UE, AI/ML-based prediction is applied without reducing SSB overhead for the network. As a result, the parameter of dedicated SMTC needs to rely on legacy SMTC, for example, having the same offset or duration. For option 2, a single SSB muting/masking provided by RRC signalling is used to identify the subset of the legacy SSB transmission. Specifically, the SSB muting/masking is based on configuration parameters (e.g. bitmap at SMTC level, SSB burst, etc). We think both options are feasible. Thus, we propose:

- For temporal domain case B, RAN2 to discuss the below options for the network to indicate the timing of SSB transmission:

      - Option 1: Introduce dedicated SMTC configuration.

      - Option 2: Muting/masking legacy SSB transmission.

Frequency domain:

The motivation of frequency domain prediction is to predict inter-frequency neighbour cells, which can reduce the number of frequencies that the UE needs to measure, and measurement gaps can be avoided. However, from our understanding, measurement gaps can be avoided relying on whether the UE performs inference. Legacy RRM measurement still needs to be performed before UE starts to inference, so measurement gaps are needed. Furthermore, it is necessary to discuss the conditions for measurement gaps to take effect.

- The UE can be configured with measurement gaps in the case of frequency domain prediction.

- RAN2 to discuss the conditions for measurement gaps to take effect.

RRM measurement framework for AI mobility inference configuration and reporting

In RAN2#129bis meeting, the following agreement was made for inference configuration and reporting.

RAN2#129bis agreement - The inference configuration and reporting for AI/ML mobility can be based on RRM measurement framework.

From our perspective, the existing RRM measurement configuration consisting of MeasObject, MeasId, ReportConfig can be extended to support AI mobility. For the inference configuration, there is a restriction in TS 38.331 that is “for all SSB based measurements there is at most one measurement object with the same ssbFrequency”. To avoid breaking this restriction, legacy MeasObject with existing RRM configuration can be reused to configure the inference/monitoring for AI mobility. Regarding the report configuration, since different triggering conditions for monitoring and inference reporting may apply, for example, inference reporting might be based on RSRP, while monitoring reporting may be triggered by RSRP difference, different reporting configurations for monitoring and inference can be associated with corresponding MeasObject via different MeasId.

Figure 1: Structure of inference configuration and reporting.

- RAN2 to discuss reusing the existing MeasObject to configure the inference/monitoring parameters for AI mobility.

- RAN2 to discuss introducing dedicated ReportConfig for inference and monitoring reporting separately for AI mobility.

- Conclusion

In the contribution, we have the following observations and proposals:

- For temporal domain case B, RAN2 to discuss the below options for the network to indicate the timing of SSB transmission:

      - Option 1: Introduce dedicated SMTC configuration.

      - Option 2: Muting/masking legacy SSB transmission.

- The UE can be configured with measurement gaps in the case of frequency domain prediction.

- RAN2 to discuss the conditions for measurement gaps to take effect.

- RAN2 to discuss reusing the existing MeasObject to configure the inference/monitoring parameters for AI mobility.

- RAN2 to discuss introducing dedicated ReportConfig for inference and monitoring reporting separately for AI mobility.

- Reference

[1] RP-252899 Revision of WID on AI mobility v6.1.