3GPP TSG-RAN WG2 Meeting #132	                                                              R2-2509026

Dallas, USA, 17th – 21st Nov. 2025

Agenda item:	10.3.1.1

Source:	CEWiT

Title:	Considerations for User Plane functionality and design in 6GR

Document for:	Discussion and Decision

# Introduction

In RAN#109 meeting, revised SID on 6G Radio (6GR) was approved [1]. The relevant SID topic related to this paper is

- User Plane architecture and protocol design

In RAN2#131bis following up on the UP design the following agreement was made.

Agreements on UP

- RAN2 first discuss/agree on the required functionalities for the protocol stack, and only afterwards discuss which protocol layer supports each functionality.  Discussions should focus on standalone architecture, until told otherwise by plenary.  When discussing functionalities companies should focus on identifying the problems being address or new requirements.

- UP design should aim to be hardware-processing friendly while keeping memory requirements low and minimize data movements across protocol layer.

- 6G user plane should aim to reduce the radio and end-to-end latency for general services (including eMBB).

- Study potential benefits and standardized mechanisms for applications/service-awareness in the RAN (e.g. immersive communications, AI mobile traffic).  Understand the issues and shortcoming with the current NR QoS.   Aim to look at a general framework.

- Support at least the following scheduling schemes: dynamic grant and configured grant.  Further study configured grant.

- Study need for scheduling enhancements to address the SR/BSR/DSR latency.

- Study how to improve L2 ARQ latency/efficiency in 6GR based on tight coordination with HARQ.  Study should identify enhancements needed for HARQ and L2 ARQ.

In this contribution, we discuss the user plane protocol functionalities and design considerations for optimizing 6G protocol stack and improving end-to-end latency.

# Discussion

In 6G, the user plane protocol should be able to meet required performance of existing services like eMBB and also new services like immersive communication and other latency critical applications. The 6G user plane protocol stack may be designed by incorporating 5G user plane functionalities as a baseline, but it should be allowed to evolve in later releases.

Proposal 1: Consider the following as essential user plane functionalities of 6G as similar to 5G NR:

- IP and Ethernet header compression

- Security: encryption and integrity protection of data

- Re-ordering for in-sequence delivery

- Retransmission

- Segmentation & re-assembly

- Multiplexing and demultiplexing of PDUs

In 5G NR, RLC and PDCP contains separate sequence numbers for different purposes. It can be beneficial for a single sequence number-based protocol to reduce header reduction. In this regard, merging sequence number-based functionalities of RLC and PDCP into a single protocol stack layer may be considered in 6G to reduce protocol overhead and overall processing time.

Proposal 2: Study optimizing 6G user plane protocol with lower header overhead and reduced data movement across protocol layers than 5G NR (e.g. single sequence number-based operations)

In 5G NR, receiver MAC accepts the MAC PDU when MAC TB CRC is passed, i.e., receiver MAC can start decoding the MAC PDU only when all CBs/CBGs in the MAC PDU are successfully received. Even if successfully received CBs/CBGs at receiver PHY may contain packets that can be processed by higher layers, those packets must wait at receiver PHY until the entire CBs/CBGs of the MAC PDU are successfully received. After the final retransmission attempts by HARQ, if the MAC TB CRC fails, the entire received information is rejected by L1. This includes the packets that are part of successfully received CBs/CBGs.

Observation 1: MAC is forced to not decode the successfully received portions of the MAC TB unless the entire MAC TB is received successfully. This results in either unwanted packet failures or delay in packet processing.

Proposal 3. Study to enhance the MAC protocol for enabling immediate processing of the successfully received portions of the MAC TB to improve latency

5G NR approach of one size fit for all may not be an efficient way to deal with advanced 6GR services. A thorough study needed to make sure the new optimized protocol stack for 6GR should meet the requirements of all use cases, otherwise a mix of old and new protocol stack can be considered.

Proposal 4: Study new optimized set of protocol layer stack either as an additional option applicable to certain use cases (e.g. latency sensitive applications) or as a single protocol stack (which should enough meet the requirements of all the use cases including control plane data traffic).

# Conclusion

In this contribution, we discuss the user plane protocol functionalities and design considerations for optimizing 6G protocol stack and improving end-to-end latency. Following are the proposals:

Proposal 1: Consider the following as essential user plane functionalities of 6G as similar to 5G NR:

- IP and Ethernet header compression

- Security: encryption and integrity protection of data

- Re-ordering for in-sequence delivery

- Retransmission

- Segmentation & re-assembly

- Multiplexing and demultiplexing of PDUs

Proposal 2: Study optimizing 6G user plane protocol with lower header overhead and reduced data movement across protocol layers than 5G NR (e.g. single sequence number-based operations)

Proposal 3. Study to enhance the MAC protocol for enabling immediate processing of the successfully received portions of the MAC TB to improve latency

Proposal 4: Study new optimized set of protocol layer stack either as an additional option applicable to certain use cases (e.g. latency sensitive applications) or as a single protocol stack (which should enough meet the requirements of all the use cases including control plane data traffic).

# References

- RP-252912		Revised 	SID: Study on 6G Radio