3GPP TSG-RAN WG2 Meeting #131bis	R2-2509031

Dallas, U.S, Nov. 17th-21st

Source:	CEWiT

Title:	6G Mobility Framework

Agenda item:	10.4

Document for:	Discussion

# Introduction

The following agreements were made in RAN2#131bis related to 6GR mobility

Agreements

1 Study mobility with the following requirements in 6G mobility design:

- Minimize interruption time and ensure service continuity (i.e. minimize throughput degradation during mobility). Consider complexity and gains when discussing solutions.

- Robustness of mobility procedures

- Energy efficiency for both UE and NW

2 Study aspects related to mobility (e.g. early DL/UL synchronization, UE configuration processing, pre-configurations, conditional handover, early CSI acquisition)

In this document we discuss some follow-up points for RAN2 to discuss, along with idle and inactive mode mobility and inter-RAT handover which would be newly discussed in this meeting.

# Discussion

Mobility framework and measurements

NR handover takes several back-and-forth signaling between UE and the network, and this is both time consuming and energy draining. This involves synchronization, higher layer re-establishment, etc. In most real-time deployments, there is a fair chance to make it simpler than a traditional handover, if the target cell is also a current serving cell, or it could be added as a serving cell as a transitional step towards handover. This not only takes away the need to do a synchronization after the handover decision, but also, with proper architectural changes in 6G, there is also an advantage of achieving 0 ms interruption for latency-intolerant bearers.

Observation 1: In NR, the advantages of cases where a target cell is a current serving/aggregated cell are not properly utilized.  For example, the benefit of the target cell being an SCell, PSCell, or a TRP from another cell could be well utlised at least to avoid at least a re-synchronisation.

Proposal 1: Study the procedures for a simplified and faster handover between already connected/aggregated serving cells (e.g. primary and secondary cells) in 6GR to achieve near zero interruption.

Currently, there are various mechanisms to perform synchronization during handovers. Given below is the evolution of RACH over the releases. CBRA was the first and basic wa.y of performing RACH. While being the most time-consuming at the same time, it can be performed with almost zero pre-configurations. However, it would be complex and inefficient for the connected UEs when it has to perform contention-based RACH every time. This brought in CFRA wherein a dedicated preamble is provided to the UE which is reserved for a while at the network. In case of failure, the UE must resort to CBRA. Then came a special RACH procedure introduced for LTM, early synchronization. In this case, the preamble is sent for each candidate cell in the respective PDCCH order, and this is used to send to the indicated candidate cell. RAR is not expected from the candidate cell. TA comes in the cell switch command. This method is, by far, the best so far in terms of the time taken. However, it requires a lot of pre-configurations and resource utilization need not be at its best. Another method is UE-based TA estimation.  However, this demands UE capability.  To conclude, there has been an increase amount of configuration (cost we pay) to minimize the interruption.

Observation 2: In NR, RACH is a trade-off between interruption and heavy pre-configurations and less resource utilization.

Proposal 2: Study the possibilities of improving RACH process for mobility to reduce pre-configurations targeting less interruption time.

6G Mobility is envisioned as a sophisticated mechanism wherein the decisions are taken well in advance, giving the system it's time to prepare itself before the actual handover takes place. For this, handover predictions and accurate timing information for the handover are to be arrived at. This enables better resource utilization, avoiding preambles, and resources to be reserved for a longer duration. This information can be arrived at by using the existing 5GA study outcomes from AI/ML for mobility Rel-19 5GA agenda item where the 2 goals considered are – measurement reduction, and handover performance improvement.

Observation 3: Handover decisions and resource utilization could be efficiently designed in 6G if we use AI/ML from 5GA as the baseline.

Proposal 3: For 6G mobility framework, RAN 2 should consider native AI/ML support for 6G mobility, with 5GA Rel20 as baseline

In all the existing handover procedures, a major bottleneck is updating security keys after every inter-gNB handover. To bypass this, several solutions were previously discussed; PDCP anchoring, providing prior information on the NCC value in MAC CE or a list in RRC configurations, etc. to list a few. They were all slightly inconvenient options in some sense, and many raised concerns in RAN2. An example is sending security related information in MAC CE which is unsafe since there is no protection to the information. PDCP anchoring relies on Xn interface for forwarding user data packets and hence overloads it. In 6G, we need a simplified architecture while not compromising essential security.

Observation 4: In NR, the security architecture was rigid and not scalable. This remained a bottleneck in most mobility enhancements in NR.

Proposal 4: Security key updates may not be mandated in all handovers. e.g. inside a trusted gNB set. RAN2 to consult SA3 on these mobility requirements through LS for consideration while architecting 6G security.

Although LTM is a efficient step towards faster handovers, a major drawback of the initial version of LTM was that it was based on L1 measurements which need not be filtered (or is left to UE implementation).  This led to the ping-pong issue. Although LTM could handle ping pong to a certain extent, there is an underlying unreliability in the procedure. To deal with this, L3 measurements can be considered as a correction. However, there are unnecessary transactions and delays associated which would take away the benefit of having LTM by making it more complicated. This can be avoided by filtered L1 measurements for lower layer triggered mobility in 6GR.

Observation 5: In NR, the L1 measurements need not be filtered and is left to UE implementation

Proposal 5: Study the use of filtered L1 measurements for lower layer triggered mobility in 6GR.

Cell Selection/Reselection

An Idle mode UE or Inactive mode UE could be paged by the CN or RAN (in case of Inactive mode) so that the UE resumes the connection with the network. When there is a broadcast priority in the system information, all UEs would try to camp on to the best available cell. This causes paging and access congestion. This is a reasonable concern in NR, and more so in 6G. When paging/access congestion occurs, the users become un-trackable. This is the last thing we would want to have in any network.  In LTE, there was a redistribution mechanism (MCLD) using which users were allotted to different cells. However, this was discontinued in NR, and the dedicated priorities and the SIB information on absolute priorities were considered sufficient. Although it tackles the problem to a certain extent, the congestion possibility is still open considering the validity of dedicated priorities is decided by the timer T320.

Observation 6: The problem of paging and access congestion is not fully resolved in NR.

Proposal 6: Congestion-optimized cell selection/re-selection to be studied. Paging and broadcast-related agreements to be taken into consideration while designing this.

Inter-RAT mobility

With every new generation in cellular network, inter-RAT mobility is a key point of discussion. 6G has agreed to have stand-alone architecture. This means that there will not be any direct coupling between NR and 6G architectures, unlike LTE-NR. To achieve a lossless handover with minimized interruption, inter-RAT mobility should be discussed.

RAN2 can start discussing the pre-configurations, RRC state management, security key management, system broadcast information, etc. with multiple options for availability of interfaces in the core network for 5G-6G interworking, wherever necessary.

This, however, would be discussed in RAN3. Since the above factor forms the basis of the procedure, once RAN3 agrees on the basic architecture, RAN2 can come to a consensus on the options available.

Observation 7: So far, there is no conclusion on how 5G-6G interworking would be designed in RAN 3 and SA2.

Proposal 7: RAN2 to discuss solutions to 5G-6G inter-RAT mobility considering pre-configurations, RRC state management, security key management, system broadcast information, etc. with different options catering the possible architectures.

# Conclusion

Mobility framework and measurements

Proposal 1: Study the procedures for a simplified and faster handover between already connected/aggregated serving cells (e.g. primary and secondary cells) in 6GR to achieve near zero interruption.

Proposal 2: Study the possibilities of improving RACH process for mobility to reduce pre-configurations targeting less interruption time.

Proposal 3: For 6G mobility framework, RAN 2 should consider native AI/ML support for 6G mobility, with 5GA Rel20 as baseline

Proposal 4: Security key updates may not be mandated in all handovers. e.g. inside a trusted gNB set. RAN2 to consult SA3 on these mobility requirements through LS for consideration while architecting 6G security.

Proposal 5: Study the use of filtered L1 measurements for lower layer triggered mobility in 6GR.

Proposal 6: Congestion-optimized cell selection/re-selection to be studied. Paging and broadcast-related agreements to be taken into consideration while designing this.

Proposal 7: RAN2 to discuss solutions to 5G-6G inter-RAT mobility considering pre-configurations, RRC state management, security key management, system broadcast information, etc. with different options catering the possible architectures.