3GPP TSG RAN WG 2 ##132										R2-2509040

Dallas, US, November 17th - 21st, 2025

Agenda Item:	10.3.2.3

Source:	CSCN

Title:	Discussion on 6G Initial and System Access

Document for:	Discussion

- Introduction

In the RAN2#131-bis meeting, the following agreements were obtained [1].

Agreements on System Information

-	System information design should consider energy efficiency and low complexity for both network and UE.

-	System information design should ensure that the public warning requirements are met.

-	Study RAN2 aspects of system information designs (e.g. on-demand SIB and SSB, periodicity of SSBs)

-	Study how to improve flexibility and extensibility of scheduling SI design

-	Study mechanisms to improve SI update mechanism

-	Study aspects related to SIB1 size. Understand the issues and desired content/size from RAN2 point of view.

-	Study areas specific SIB design. Understand 5G pain points and what improvements can be considered.

NOTE: this is a starting list

Agreements on Paging

- Paging design should consider energy efficiency and simplicity for both UE and Network.

In this contribution, we provide our views to cover the 6G initial and system access, especially the harmonization of TN and NTN, aiming to enable the native support of NTN characteristics.

- Discussion

Based on the agreements of system information in the last meeting [1], the design principles of 6G SI are energy efficiency and low complexity for both network and UE. Among them, the several aspects that this may involve are listed as follows.

  - System information

    - Minimum SI design

One of the fundamental design principles for 6G is to harmonize NTN and TN design. From our perspective, SIB acquisition is one of the essential harmonized designs for TN and NTN. The Minimum SI should contain the essential information required for UE to camp on the 6G cells whatever it is TN or NTN.

For UEs camping on NTN, the UEs need to pre-calculate with the satellite ephemeris and perform the necessary pre-compensation of timing and frequency offsets before initial access. According to the current 5G design, the UE needs to perform SSB detection first and receives SIB1 before receiving SIB19. UE performs initial access after SIB1 and SIB19 successfully received. However, there is a gap time between SSB synchronization and SIB19 acquisition. Due to the high mobility of the satellite, the doppler shift and propagation delay experience a high rate of change. This drift during the SIB19 acquisition window can degrade decoding performance and potentially lead to acquisition failure, preventing the UE from accessing the network.

Based on the above analysis, essential NTN related information for initial access (e.g., ephemerisInfo, cellSpecificKoffset, kmac and polarization) should be included in Minimum SI messages to reduce UE synchronization complexity and shorten access latency.

The 6G Minimum SI design should consider the harmonized integration of TN and NTN, with essential NTN-related access information included in the Minimum SI.

    - SI scheduling

In 5G, it is specified the requirement and restriction of SI messages scheduling. For example, SIBs in schedulingInfoList and schedulingInfoList2 are mapped to separate SI messages, SIBs and posSIBs are mapped to separate SI messages, etc [2].

SIBs and posSIBs are mapped to different SI messages, i.e. an SI message contains either only SIBs or only posSIBs. The mapping of SIBs to SI messages is configured in schedulingInfoList and schedulingInfoList2, while the mapping of posSIBs to SI messages is configured in posSchedulingInfoList and schedulingInfoList2. Each SIB and each posSIB is mapped to a single SI message.

Such restrictions not only increase broadcast overhead, particularly in NTN enhance downlink coverage scenarios, but also increase UE power consumption to repeatedly receive different SIB messages. Certainly, the network should avoid scheduling SIBs of significantly different types within the same SI message to prevent UE from receiving unnecessary SIBs by the network implementation. Therefore, it is recommended that network to flexibly configure multiple SIBs per SI message.

RAN2 to study flexible SI scheduling mechanism for both TN and NTN, including flexible combination of multiple SIBs per SI message.

    - Area specific SIB

To reduce redundant system information broadcasting and to minimize SI update overhead, area specific SIB was introduced for sharing same broadcasting information in multiple cells in current specification. Any SIB or posSIB except SIB1 can be configured to be cell specific or area specific.

Area specific SIB is also critical for NTN with avoiding unnecessary broadcasts and enables the network to broadcast the relevant configuration only within the area where that service is available or where specific configuration is required. Area specific SIB is cell-level granularity for SI broadcasting in TN. However, the coverage of the cell becomes too coarse in NTN. For example, while a cell may have numerous neighbour cells (currently SIB19 can configure up to 8 neighbour cells), only the limited numbers of these can actually serve the UEs. There is no benefit for a UE to receive and decode NTN related info (i.e. ephemeris) for neighbour cells that cannot serve its area. Area specific SIB with an area defined by multiple cells is not suitable for SIB broadcasting in NTN, as it would increase overhead for network and UE. Broadcasting matched system information content for different areas within a cell is beneficial for NTN. The definition of area specific SIB may need to be updated. The area with portion of cell should be considered in NTN for area specific design.

In 6G, in order to introduce area specific SIB both for TN and NTN for harmonization, the fundamental concept of area can be further refined. The area can be defined at the cell level (a single cell or multiple cells) as specified in 5G, or at a finer granularity level smaller than a cell (e.g., beam level, one or multiple beams).

RAN2 to study area specific SIB mechanism both for TN and NTN in 6G, and redefine the concept of the “area” with (1) multiple-cell level and (2) fine-grained cell level (e.g., beam level).

    - SI validity and update

Regarding SI validity, any SIB or posSIB except SIB1 can be configured to be cell specific (one cell) or area specific SIBs (one cell). In area-specific SIB, one of the essential parameters is systemInformationAreaID. When the UE moves to the area of a new systemInformationAreaID, the stored area-specific SIB may no longer be applicable and needs to be re-acquired. Such mechanism of re-acquiring SIB based on area ID changes can be introduced in 6G SI update both for TN and NTN.

In addition to the area-based SIB update mechanism, time-based SIB updates are more straightforward and efficient in NTN. This is due to the service duration of each area is not only limited but also predictable. The network can broadcast SI message with a time indicator for the specific area. When time expires, UE needs to reacquire the SIB message for updating.

Area-based and time-based SIB update mechanism can be introduced in 6G NTN.

    - On-demand SIB

On-demand SIB transmission can reduce the broadcasting of "rarely used" system information, which is beneficial for lowering power consumption of network and reducing UE overhead for SIB monitoring. Considering user distribution may varies differently across areas in NTN, and downlink reception capabilities also differ greatly among UEs within the same area, UEs in different areas may require different types or periodicities of SIBs. This makes it difficult for the gNB to determine which SIBs are "rarely used" across the entire cell. If the gNB configures "frequently used" SIBs as on-demand SIBs, it may lead to wasting of radio resource due to a large number of UEs requesting these SIBs.

From our perspective, a flexible on-demand SIB procedure is needed to accommodate UEs with varying downlink reception capabilities, especially for NTN. Also, more SIB types can be acquired via on-demand procedures (including both RRC_IDLE and RRC_CONNECTED UEs).

RAN2 to study a flexible SIB request procedure and to support the acquisition of additional SIB types through on-demand mechanisms in both RRC_IDLE and RRC_CONNECTED.

    - SI periodicity

The broadcast of system information is required to encompass the entire serving cell area, leading to considerable overhead, particularly in NTN beam-hopping scenarios, where each system information transmission necessitates a full beam sweep across the entire coverage area. To reduce the broadcast overhead for unnecessary SI acquisition, a longer SI-Periodicity for SIBs with low real-time requirements is necessary.

RAN2 to support longer SI-Periodicity for low real-time SIB.

  - Paging

In 5G networks, the minimum paging granularity is defined on a cell-specific basis. However, in NTN, paging a UE across an entire cell becomes highly inefficient, as it consumes excessive time-frequency resources and increases gNB power consumption, thereby reducing paging capacity and consequently leading to increased call setup latency for the UE. Therefore, it is more reasonable to paging UE in an area smaller than the whole cell. It is recommended to define a new granularity rather than cell in paging mechanism.

RAN2 to study enhanced paging tracking area management schemes with finer granularity for NTN.

- Conclusion

Proposal 1:	The 6G Minimum SI design should consider the harmonized integration of TN and NTN, with essential NTN-related access information included in the Minimum SI.

Proposal 2:	RAN2 to study flexible SI scheduling mechanism for both TN and NTN, including flexible combination of multiple SIBs per SI message.

Proposal 3:	RAN2 to study area specific SIB mechanism both for TN and NTN in 6G, and redefine the concept of the “area” with (1) multiple-cell level and (2) fine-grained cell level (e.g., beam level).

Proposal 4:	Area-based and time-based SIB update mechanism can be introduced in 6G NTN.

Proposal 5:	RAN2 to study a flexible SIB request procedure and to support the acquisition of additional SIB types through on-demand mechanisms in both RRC_IDLE and RRC_CONNECTED.

Proposal 6:	RAN2 to support longer SI-Periodicity for low real-time SIB.

Proposal 7:	RAN2 to study enhanced paging tracking area management schemes with finer granularity for NTN.

- Reference

- Chair's Notes RAN2#131-bis, Oct. 13th-17th, 2025.

- TS 38.331, Radio Resource Control (RRC), version 18.7.0 Release 18, 2025.