3GPP TSG RAN WG 2 ##132										R2-2509041

Dallas, US, November 17th - 21st, 2025

Agenda Item:	10.2.2

Source:	CSCN

Title:	Discussion on TN/NTN integration

Document for:	Discussion

- Introduction

In the RAN2#131-bis meeting, the following agreements were obtained [1].

- For next meeting companies should identify essential requirements for NTN and identify which functionality should be considered in the common design with TN and what are NTN specific and whether they are essential.

In this contribution, we provide our views on 6GR TN and NTN integration, aiming to enable the native support of NTN characteristics.

- Discussion

  - Minimal essential set of requirements for NTN

6G NTN deployment scenarios are defined to allow for the provision of services for those areas including vertical space (e.g., up to several kilometers altitude) where terrestrial network service is not available (on a permanent or temporary basis), and also for those services that can be more efficiently supported by Non-Terrestrial Networks.

Non-Terrestrial Networks can overcome coverage limitations of terrestrial networks especially in low-density populated areas and maritime areas.

As such, NTN will mainly contribute to:

- Achieve ubiquitous connectivity and availability of services in remote and sparsely populated areas.

- Improve overall 6G network resiliency, especially in situations where disasters disrupt terrestrial networks or damage undersea communication cables by supporting in priority critical communications and emergency communications (messaging, voice calls).

6G NTN should support both regenerative and transparent payload as in 5G. and also support any orbits range from VLEO (300 to 600 km altitude) to GSO (~35786 km altitude), including LEO (600 to 2000 km altitude) and MEO (2000 to ~35000 km altitude). NTN may be based on a combination of orbit types (e.g. GSO and LEO), multi-orbit collaboration between LEO, MEO and GSO satellites can be considered in 6G. With the goal of seamless global coverage with ubiquitous connectivity, 6G NTN should also consider the scaling evolution from sparse to dense satellite constellations with resilience and service flexibility.

- Cell deployment

As studied and described in R16 TR38.821[2], “Similar to TN, one or several SSB index can be used per PCI to separate SSB transmission on different beams.” We believe that 6G NTN can reuse the designs studied in 5G TN and NTN, whether it is SSB beam or other logical beams newly defined in 6G. As a result, 6G cell deployment can include one-beam cell and multi-beam cell. For multi-beam cell, 6G NTN should study on the impact with the irregular shape of NTN cells.

- Cell type

NTN can be deployed to provide coverage with earth-moving cell, quasi-earth-fixed cell and earth-fixed cell which are supported respectively by the following three types of service link:

- Earth-fixed: provisioned by beam(s) continuously covering the same geographical areas all the time (e.g., the case of GSO satellites); - Quasi-Earth-fixed: provisioned by beam(s) covering one geographic area for a limited period and a different geographic area during another period (e.g., the case of NGSO satellites generating steerable beams); - Earth-moving: provisioned by beam(s) whose coverage area slides over the Earth surface (e.g., the case of NGSO satellites generating fixed or non-steerable beams).

The Quasi-Earth-Fixed deployment can be categorized into two types in the scenario of multi-beam cell, the current serving cell is denoted by a yellow circle, with its beams denoted by blue circles, as shown in Figure 5:

- Case 1: The areas of cell coverage in two consecutive periods are completely different.

- Case 2: The areas of cell coverage in two consecutive periods partly overlap.

Figure 1: Quasi-Earth-Fixed case 1 and case 2

For Quasi-Earth-Fixed case 1, service for UE1 and UE2 starts at time T1 and stops simultaneously at time t-Service, after which the satellite starts to serve the next geographic area (i.e. another cell). The geographical area of both the serving cell and its beams remains unchanged during time T1 to time t-Service. The continuous service duration for each UE is given by t-Service – T₁.

For Quasi-Earth-Fixed case 2, with the satellite moves, the cell coverage area hops over the Earth surface with the earth-fixed beams. Different from all beams within Quasi-Earth-Fixed case 1, one or several beams covering for one geographic area switch to another geographic area after a limited period. The service for UEs in corresponding beams starts and stops simultaneously while the UEs in the other beams of this cell have the different start and stop time. From our perspective, case 2 is beneficial for the link budget with ubiquitous coverage.

- Energy efficiency

In the NTN network, satellites have very limited energy sources, including solar panels and batteries. Once launched, the satellite's energy system is fixed and cannot be recharged in the shadow zone or have its batteries replaced. Given the energy constraints of satellites, improving energy efficiency becomes an important consideration.

Specifically, 6G NTN can consider extending the SSB periodicity (e.g., 320ms) to improve satellite energy efficiency. Such extension may have impact on average latency during initial user access as well as delays in cell handover and reselection, which may decrease user experience. Consequently, it is necessary to evaluate these trade-offs in the system design.

6G NTN may consider implementing on-demand signaling to improve satellite energy efficiency. On-demand SSB has been introduced in 5G and is also applicable to 6G systems. Both base station and UE should enter an inactive state when there is no data transmission, activating signaling only when initiating a service or responding to the requests. A joint energy-saving strategy between xNB and UEs should be prioritized in the design.

- Multi-orbit collaboration

Multi-orbit collaboration means that satellites in different orbits are capable of offering services to UEs. However, a prerequisite for this is NGSO compliance with an ITU regulation as defined in TR38.821[2]. To fulfill the aforementioned interference avoidance principles for GEO networks, the coverage of a single NGSO satellite may become incomplete. That is, partial service blind zones may exist within a serving cell. However, the geographical position of these blind zones on the ground changes as the NGSO satellite moves, and the UEs are unaware of when they will be affected by such blind zones. In order to decrease the impact for UE, it is recommended to study the impact of interference avoidance for GEO.

RAN2 to study the following requirements for NTN:

- One-beam cell and multi-beam cell (irregular/regular shape)

- Earth-moving cell, quasi-earth-fixed cell (case 1 and case 2) and earth-fixed cell type

- Energy efficiency (extended SSB periodicity)

- Multi-orbit collaboration

- The scaling evolution from sparse to dense satellite constellations

  - Critical NTN functions for day-1 and later

In 6G, we aim for NTN to have the necessary functions to support the deployment of 6G NTN systems in day 1. The critical NTN functions need to be considered in priority to ensure the NTN system is functional. We have identified and listed critical NTN functions required for 6G day-1 as follow:

- Initial access: Initial access is the basic function for NTN. Considering the characteristics of long propagation distances and limited link budget, the latency for initial access is significantly longer than TN. It is essential to consider the initial access with lower latency for 6G NTN in day-1.

- Paging: In 5G networks, the minimum paging granularity is defined on a cell-specific basis. However, in NTN, paging a UE across an entire cell becomes highly inefficient, as it consumes excessive time-frequency resources and increases gNB power consumption, thereby reducing paging capacity and consequently leading to increased call setup latency for the UE. Therefore, it is more reasonable to paging UE in an area smaller than the whole cell. It is recommended to define a new granularity rather than cell in paging mechanism.

- Mobility: NTN mobility includes intra-RAT mobility and inter-RAT mobility.

  - For intra-RAT mobility, it is essential to consider mobility within 6G NTN (including multi-orbit mobility) and mobility between 6G TN and NTN for both idle and connected mode. The characteristics of highly dynamic and predictable should be taken into account. The mobility enhancement for DLCE in R19 should be considered as the baseline within 6G NTN.

  - For inter-RAT mobility, it is essential to consider the case of 5G NTN-6G NTN and 5G TN-6G NTN,

  - Measurement: In 5G, location-based and time-based measurement mechanisms were introduced to address the issue of less near-far effects. 6G NTN can reuse the measurement mechanisms of 5G NTN, with consideration given to whether enhancements should be made to address scenarios including adjacent coverage, overlapping coverage, multi-orbit coordination, NTN-TN interworking and inter-RAT mobility.

- Network & UE energy saving: In NTN, satellites have very limited energy sources, including solar panels and non-removable batteries. UE consumes more power for long-distance NTN communication. It is essential to consider the Network & UE energy saving for 6G NTN in day-1. As 5G has made significant progress in energy saving for TN, 6G can leverage the foundation of 5G energy saving research and pursue energy-saving schemes for NTN networks and UE.

- PWS: PWS can send high-priority notifications to millions of mobile devices simultaneously through a robust cell broadcast mechanism. Due to the wide-area coverage characteristics, NTN could ensure comprehensive coverage of PWS by extending their reach to underserved and remote areas, guaranteeing that no community is left behind during critical events. It is suggested to support PWS in 6G NTN in day 1.

- MDT: The global coverage deployment of satellites introduces a challenge in dangerous inaccessible areas and incomplete test coverage of testing. MDT can be used to support efficient drive test in 6G NTN. The application of MDT in satellite communication enables the utilization of NTN UEs to automatically collect and report wireless measurement data, which can replace costly and inefficient traditional drive tests. Due to the limitation in UE power consumption and link conditions, it is suggested to design a UE-friendly test data collection and reporting procedure for 6G NTN.

RAN2 to study the following NTN functions in 6G day-1:

- Initial access with lower latency and finer granularity

- Mobility

        - Mobility for idle mode and connection mode in 6G NTN (including multi-orbit)

        - Mobility between 6G TN and NTN

        - Inter-RAT mobility for 5G NTN - 6G NTN and 5G TN - 6G NTN

        - Time-/Location-based measurement

- Network &UE energy saving

- PWS

- MDT

In subsequent study phases, additional features and optimizations for 6G NTN can be implemented. We have listed some NTN functions that can be studied in day-2 or later:

- AI for NTN: As a new service and capability, AI is under discussion in TN, and it can also be applied in NTN later. We think AI can be beneficial at least for mobility management (e.g. handover for loading balancing) and QoS management (e.g. traffic classification and prediction) in 6G NTN. It is therefore recommended to introduce AI and take into account onboard computing constraints in NTN during the Day-2.

- Sensing in NTN: Remote sensing can be integrated with NTN communication. The satellite serves both as a collection point for remote sensing data and as a component of the NTN communication system, responsible for transmitting vast amounts of remote sensing data back to Earth in real time. It is suggested that the integration of remote sensing and NTN communication can be discussed when the design of 6G NTN becomes relatively mature.

RAN2 to study the following functions in subsequent phases:

- AI in NTN

- Sensing in NTN

  - Common design for NTN functionalities with TN

Based on the critical NTN functions listed above, we identify and analyze the set of functionalities that should be considered in common design with TN. Before the discussion, we would like to clarify the definition of common design for NTN functionalities with TN. In our understanding, there are two types of common design:

- Same design for TN and NTN

- Common framework with NTN specific configuration

Table 1 gives a summary of our analysis on common design for NTN functionalities with TN.

Table 1: Common design for NTN functionalities with TN

| Functions | Design types | Description |
| --- | --- | --- |
| Initial and System Access (system information, paging, random access) | Common framework with NTN specific configuration | Initial access design should consider long propagation distances and limited link budget in NTN, which may require some NTN-specific parameters (e.g. ephemeris). In NTN MSI messages, essential information for initial access (such as ephemerisInfo, cellSpecificKoffset, kmac and polarization) should be included to reduce UE synchronization complexity and shorten access latency. Compared with TN, NTN has extensive coverage and limited transmission sources. In R16 TR38.821[2], tracking area management and paging handling with or without UE location has been studied. However, the minimum paging granularity is on a cell-specific basis in 5G NTN. In order to improve the paging efficiency for 6G NTN, RAN2 should study more granular paging schemes. |
| Mobility | Common framework with NTN specific configuration | RAN2 should study more flexible location-based and time-based mobility management mechanisms to address scenarios including adjacent coverage, overlapping coverage, multi-orbit coordination, NTN-TN interworking and inter-RAT mobility. |
| HARQ | Common framework with NTN specific configuration | HARQ disable for NTN |
| MDT | Common framework with NTN specific configuration | Compared with TN, NTN satellites may cover some inaccessible dangerous areas, making it hard to do drive test. RAN2 should study efficient drive test with MDT in 6G NTN. |
| Network & UE energy saving | Common framework with NTN specific configuration | Beam hopping needs to be considered in 6G NTN |
| Other UP functions, for example data transmission, ROHC, scheduling, security | Same design |  |

RAN2 to study two types of common design:

- Common framework with NTN specific configuration: Initial and System Access, mobility, HARQ, MDT, network & UE energy saving

- Same design: other UP functions

  - Way forward for TN/NTN integration in 6GR study phase

In 5G, NTN was introduced as later enhancements without comprehensive consideration in the initial 5G design, resulting in a patched design approach for 5G NTN. For instance, the requirement for UEs to receive both SIB1 and SIB19 before accessing the network leads to prolonged access latency in 5G NTN. It is therefore recommended that 6G common design should address the characteristics of both NTN and TN from the beginning.

RAN2 to study the 6G common design for both NTN and TN from the beginning.

- Conclusion

Proposal 1:	RAN2 to study the following requirements for NTN:

- One-beam cell and multi-beam cell (irregular/regular shape)

- Earth-moving cell, quasi-earth-fixed cell (case 1 and case 2) and earth-fixed cell type

- Energy efficiency (extended SSB periodicity)

- Multi-orbit collaboration

- The scaling evolution from sparse to dense satellite constellations

Proposal 2:	RAN2 to study the following NTN functions in 6G day-1:

- Initial access with lower latency and finer granularity

- Mobility

        - Mobility for idle mode and connection mode in 6G NTN (including multi-orbit)

        - Mobility between 6G TN and NTN

        - Inter-RAT mobility for 5G NTN - 6G NTN and 5G TN - 6G NTN

        - Time-/Location-based measurement

- Network &UE energy saving

- PWS

- MDT

Proposal 3:	RAN2 to study the following functions in subsequent phases:

- AI in NTN

- Sensing in NTN

Proposal 4:	RAN2 to study two types of common design:

- - Common framework with NTN specific configuration: Initial and System Access, mobility, HARQ, MDT, network & UE energy saving

- Same design: other UP functions

Proposal 5:	RAN2 to study the 6G common design for both NTN and TN from the beginning.

- Reference

- Chair's Notes RAN2#131-bis, Oct. 13th-17th, 2025.

- TR 38.821, Solutions for NR to support non-terrestrial networks (NTN), v16.2.0

- Appendix