3GPP TSG-RAN WG2 #132	R2-2509051

# Dallas, USA, 17th – 21st November 2025

Agenda Item:	 8.1.1

Source:	Ericsson

Title:		TP based on RAN1 Response LS On the implicit indication of TRP location coordinates via Associated ID

Document for:	Discussion

# 1	Introduction

RAN1 in LS “R1-2508026” has replied to RAN2 LS and based on that we propose the text proposal in this paper.

# 2 	Discussion

## 2.1 Associated ID is per cell or per TRP

In the LS RAN1 response is as below

RAN1#121 agreement, an associated ID is configured per cell (e.g., NCGI-r15). It is provided for a group of TRP(s) in the same cell, not for one specific TRP within a cell.

As per RAN1 agreement, the associated ID is configured per cell, that is, all TRPs within a cell should have same associated ID.

RAN1 clarifies that the granularity of the associated ID

- Associated ID(s) are not required to be unique across different cells.

- As associated ID(s) are unique only per cell, sharing associated ID(s) for cells belonging to different PFLs is irrelevant.

- The associated ID is not PFL specific. The associated ID is specific only to geographical coordinates of a group of TRP(s) in a cell

Further, RAN1 clarifies that it is up to NW implementation on when to consider updating the associated ID value.

- It is up to network implementation whether the associated ID is related to the identification of the location of ARP(s).

Based on RAN1 LS, one can conclude that the specification does not need to capture when the associated ID value is changed. Rather the UE assumption, behaviour with respect to the value needs to be specified.

Hence, the field description should be updated as below.

nr-AIML-AssociatedID This field provides an identity associated with the coordinates of the TRP(s) belonging to the indicated cell. This field is configured per cell. All the TRPs belonging to the same cell are asscoiated to the same nr-AIML-AssociatedID value. UE may assume the same associated ID implies that coordinates of the TRP(s) are consistent..

nr-AIML-AssociatedID field description is updated as shown above in TP which should not capture NW implementation aspect but only UEs behaviour with respect to the associated value provided by the NW and in a normative text.

## 2.2 	PRS-only TP

RAN1 response on PRS-only TP can have associated ID is as below:

- From RAN1’s perspective, in Rel-19, the associated ID cannot be used to implicitly indicate the consistency of location coordinates of such PRS-only TPs, as the PRS-only TPs are not associated with a cell.

We see that the above would require a specification update as below.

prs-OnlyTP This field, if present, indicates that the NR-DL-PRS-AssistanceData is provided for a PRS-only TP. Whether the field is present or absent should be the same for all the NR-DL-PRS-AssistanceData of all the DL-PRS transmitted under the same TP. The target device shall not assume that any other signals or physical channels are present for the TRP other than DL-PRS. In this version of the specification, nr-AIML-AssociatedID is not applied to this field.

Prs-OnlyTP field description is updated as provided above in the TP.

# Conclusion

In the previous sections we made the following observations:

Observation 1	Based on RAN1 LS, one can conclude that the specification does not need to capture when the associated ID value is changed. Rather the UE assumption, behaviour with respect to the value needs to be specified.

Based on the discussion in the previous sections we propose the following:

Proposal 1	nr-AIML-AssociatedID field description is updated as shown above in TP which should not capture NW implementation aspect but only UEs behaviour with respect to the associated value provided by the NW and in a normative text.

Proposal 2	Prs-OnlyTP field description is updated as provided above in the TP.

# References

TS 37.355, LPP