3GPP TSG-RAN-WG2 Meeting #132	R2-2509052

Dallas, USA, November 17th – 21st, 2025

Agenda item	:		8.13.2

Source	: 	Sharp

Title	: 	Discussion on RIL [J011]

Document for	:	Discussion and Decision

Introduction

RAN2#131bis made the following agreements [1]:

Agreement: Capture in the RRC spec that when a dedicated configuration is received for SL-RLC1, the intermediate relay UE releases the corresponding default configuration for SL-RLC1 on the same link. Spec impact to be evaluated in CR implementation (refer to TP in R2-2507591, subject to simplification where possible).

In this paper, we prepared the updated TP candidates to address this agreement.

Discussion

- [J011] Intermediate relay UE behaviour for SRB1

In line with the agreement, the specification should clearly state that when an intermediate relay UE receives a dedicated configuration for SL-RLC1, it releases the default configuration for SL-RLC1 on the same link, and this behaviour applies to both uplink and downlink directions.

Based on these principles, this contribution provides two simplified options in the annex derived from the TP in R2-2507591:

Option 1: Separate handling for downlink and uplink processing.

Option 2: Merge downlink and uplink handling into a single procedure for greater compactness.

While Option 2 is more compact, it may lack the level of clarity required by the specification and may introduce ambiguity in UE behaviour. Therefore, despite being less compact, Option 1 is recommended for adoption to ensure clear and unambiguous terminal operation.

- [J011] The Text Proposal option #1 in the annex is adopted.

Conclusion

In this paper, we made following observations and proposals:

- [J011] The Text Proposal option #1 in the annex is adopted.

Reference

- RAN2_131bis meeting notes

- Annex: Text Proposal options (option #1 and option#2)

- Text Proposal option #1

================= Start of Text Proposal =================

- 5.3.5.15.3	L2 U2N or U2U Remote UE Addition/Modification

The L2 U2N Relay UE shall:

1>	if no SRAP entity has been established:

2>	establish a SRAP entity as specified in TS 38.351 [66];

1>	for each sl-L2IdentityRemote value included in the sl-RemoteUE-ToAddModList that is not part of the current UE configuration (L2 U2N Remote UE Addition):

2>	configure the parameters to SRAP entity in accordance with the sl-SRAP-ConfigRelay and sl-SRAP-ConfigRelay-ToAddModList if applicable;

2>	if SRB1 is included in sl-MappingToAddModList, and sl-EgressRLC-ChannelPC5 or sl-EgressRLC-Channel-DL is configured:

3>	release SL-RLC1 for downstream for the sl-L2IdentityRemote, if established;

3>	associate the PC5 Relay RLC channel as indicated by sl-EgressRLC-ChannelPC5 or sl-EgressRLC-Channel-DL for downstream for the sl-L2IdentityRemote with SRB1;

2>	else: (i.e. SRB1 is not included in sl-MappingToAddModList, or SRB1 is included in sl-MappingToAddModList, but sl-EgressRLC-ChannelPC5 or sl-EgressRLC-Channel-DL is not configured)

3>	if SL-RLC1 for downstream for the sl-L2IdentityRemote is not established:

4>	apply the default configuration of SL-RLC1 as specified in clause 9.2.4 for downstream for the sl-L2IdentityRemote and associate it with the SRB1;

2>	if SRB1 is included in sl-MappingToAddModList, and sl-EgressRLC-Channel-UL is configured:

3>	release SL-RLC1 for upstream for the sl-L2IdentityRemote, if established;

3>	associate the PC5 Relay RLC channel as indicated by sl-EgressRLC-Channel-UL for upstream for the sl-L2IdentityRemote with SRB1;

2>	else: (i.e. SRB1 is not included in sl-MappingToAddModList, or SRB1 is included in sl-MappingToAddModList, but sl-EgressRLC-Channel-UL is not configured)

3>	if SL-RLC1 for upstream for the sl-L2IdentityRemote is not established:

4>	apply the default configuration of SL-RLC1 as specified in clause 9.2.4 for upstream for the sl-L2IdentityRemote and associate it with the SRB1;

1>	for each sl-L2IdentityRemote value included in the sl-RemoteUE-ToAddModList that is part of the current UE configuration (L2 U2N Remote UE modification):

2>	modify the configuration in accordance with the sl-SRAP-ConfigRelay and sl-SRAP-ConfigRelay-ToAddModList/ sl-SRAP-ConfigRelay-ToReleaseList if applicable;

================= End of Text Proposal =================

- Text Proposal option #2

================= Start of Text Proposal =================

- 5.3.5.15.3	L2 U2N or U2U Remote UE Addition/Modification

The L2 U2N Relay UE shall:

1>	if no SRAP entity has been established:

2>	establish a SRAP entity as specified in TS 38.351 [66];

1>	for each sl-L2IdentityRemote value included in the sl-RemoteUE-ToAddModList that is not part of the current UE configuration (L2 U2N Remote UE Addition):

2>	configure the parameters to SRAP entity in accordance with the sl-SRAP-ConfigRelay and sl-SRAP-ConfigRelay-ToAddModList if applicable;

2>	if SRB1 is included in sl-MappingToAddModList, and sl-EgressRLC-ChannelPC5/sl-EgressRLC-Channel-DL or sl-EgressRLC-Channel-UL is configured:

3>	release SL-RLC1 for downstream or upstream for the sl-L2IdentityRemote, if established;

3>	associate the PC5 Relay RLC channel as indicated by sl-EgressRLC-ChannelPC5/sl-EgressRLC-Channel-DL for downstream or sl-EgressRLC-Channel-UL for upstream for the sl-L2IdentityRemote with SRB1;

2>	else: (i.e. SRB1 is not included in sl-MappingToAddModList, or SRB1 is included in sl-MappingToAddModList, but sl-EgressRLC-ChannelPC5/sl-EgressRLC-Channel-DL or sl-EgressRLC-Channel-UL is not configured)

3>	if SL-RLC1 for downstream or upstream for the sl-L2IdentityRemote is not established:

4>	apply the default configuration of SL-RLC1 as specified in clause 9.2.4 for downstream or upstream for the sl-L2IdentityRemote and associate it with the SRB1;

1>	for each sl-L2IdentityRemote value included in the sl-RemoteUE-ToAddModList that is part of the current UE configuration (L2 U2N Remote UE modification):

2>	modify the configuration in accordance with the sl-SRAP-ConfigRelay and sl-SRAP-ConfigRelay-ToAddModList/ sl-SRAP-ConfigRelay-ToReleaseList if applicable;

================= End of Text Proposal =================