3GPP TSG-RAN WG2 Meeting #132	R2-2509081

Dallas, USA, Nov. 17th - 21th, 2025

Agenda Item:	8.11.3

Source:	OPPO

Title:	Stage-2 clarification on SBFD RACH configuration in LTM

Document for:	Discussion, Decision

# Introduction

According to the discussion [1] on the coexistence issue between (C)LTM and SBFD RA in RAN2#131bis, RAN2 agree to support SBFD RO type indication in LTM CSC (Cell Switch Command) for intra-DU case.

(C)LTM and SBFD RA O000 R2-2507617 Discussion on cross-WI ASN.1 Issues (O000,O001,O003, O004, O005) OPPO discussion Rel-19 NR_duplex_evo-Core, NR_Mob_Ph4-Core, NR_LPWUS-Core, Netw_Energy_NR_enh-Core, NR_SL_relay_multihop-Core Late - Noted Agreements - [O000] R2 confirm SBFD-RACH is not applicable to early UL sync. Update RRC spec to restrict this. - [O000] R2 confirm SBFD-RACH applicability to CLTM execution. No further specifications changes are required. - [O000] R2 confirm SBFD-RACH applicability to LTM execution upon CSC for intra-DU case as per current specs

According to the RAN2#131 meeting discussion, RAN2 agreed to support SBFD with intra-DU LTM.

- Support co-existence of SBFD with intra-DU LTM. Whether to support the co-existence between SBFD and other LTM cases is not discussed in the Rel-19 SBFD WI.

In this contribution, we provide a text proposal capturing the above RAN2 agreement.

# Discussion

Text proposal: for clarifying SBFD RACH configuration in LTM. ----------------------------------------------------------------------------------------------------- ## 23.2 SBFD Random Access Random access procedure in SBFD symbols is supported for all existing RACH trigger events as described in clause 9.2.6, except for the event of Request for Other SI and Early UL synchronization with an LTM candidate cell. For the event of RACH-based LTM cell switch, the SBFD RACH configuration is only supported for intra-DU case. Both CBRA and CFRA can be supported on SBFD sub-bands. Only the 4-step RA type using SBFD RACH resources can be supported. Two RACH configuration options are specified for SBFD RA operation in TS 38.331 [12]. A cell can configure only one RACH configuration option. This can be either: 1) A single RACH configuration that supports both non-SBFD RA operation and SBFD RA operation, or 2) A dual RACH configuration where a RACH configuration is used for non-SBFD RA operation and an additional RACH configuration is designated for SBFD RA operation, as specified in TS 38.331 [12]. An SBFD aware UE that supports the RACH configuration option configured in the cell applies the corresponding RACH configuration. Otherwise, the SBFD aware UE applies the non-SBFD RA operation. For CFRA triggered by PDCCH order, an SBFD aware UE can be explicitly indicated in the PDCCH order whether to use either the first PRACH occasions or the second PRACH occasions as specified in TS 38.213 [38], for the PRACH transmission. For CBRA, an SBFD aware UE is permitted to switch from the first PRACH occasions to the second PRACH occasions and vice versa as specified in TS 38.213 [38] during a random access procedure. However, no further switch of PRACH occasions type (the first or second PRACH occasions) is allowed in the same random access procedure. If a fall-back from CFRA to CBRA occurs, an SBFD aware UE uses the CBRA resource associated with the same PRACH occasions type.

According to the MAC CR [2] for SBFD indication for LTM CSC, the RO type indication is added for both “LTM Cell Switch Command MAC CE” and “Enhanced LTM Cell Switch Command MAC CE. According to the stage-2 specification, “For the event of RACH-based LTM cell switch, random access procedure in SBFD symbols is only supported for intra-DU case”. However, the RO type indication can indicate either SBFD RO or non-SBFD RO, when the SBFD RACH configuration (i.e. sbfd-RACH-Config) is included in the LTM candidate configuration. Then the UE can be indicated with non-SBFD RO type, when the SBFD RACH configuration is provided in the LTM candidate configuration for inter-DU case.

To avoid the misunderstandings on the scenarios/configuration supported for the SBFD RACH in LTM, we think that we can clarify in TS 38.300 that the SBFD RACH configuration is only supported for LTM in intra-DU case.

Observation: It is unclear whether the following case can be supported:

- The SBFD RACH configuration is provided in the LTM candidate configuration of the inter-DU case, and the SBFD RO type indication in LTM CSC or LTM candidate configuration indicates non-SBFD RO type.

Proposal: To clarify in TS 38.300 that SBFD RACH configuration is only supported for LTM in intra-DU case.

# Conclusion

According to the above analysis, we have the following observation and proposal.

Observation: It is unclear whether the following case can be supported:

- The SBFD RACH configuration is provided in the LTM candidate configuration of the inter-DU case, and the SBFD RO type indication in LTM CSC or LTM candidate configuration indicates non-SBFD RO type.

Proposal: To clarify in TS 38.300 that SBFD RACH configuration is only supported for LTM in intra-DU case.

Reference

[1] RAN2#131bis meeting minutes

[2] R2-2508177: Correction for RO type indication in (enhanced) LTM MAC CE (WI codes: NR_duplex_evo-Core, NR_Mob_Ph4-Core)

[3] RP-233970, “WID on Further NR Mobility Enhancements”

[4] RP-252111, “Work Item: NR mobility enhancements Phase 4”