3GPP TSG-RAN WG2 Meeting #132	           R2-2509088 (Revision of R2-2508482)

Dallas, USA, 17 – 21 November 2025

Agenda item:	8.11.3

Source:				Nokia

Title:	SBFD – Other Aspects

WID/SID:	NR_duplex_evo-Core - Release 19

Document for:	Discussion and Decision

1. Support of L1 CLI measurements

TS 38.300 specifications capture the newly introduced L1-based CLI measurements specified in Rel-19 for SBFD operation. Current version of Section 17.2 Cross-Link Interference Management describes the L1-based UE-to-UE CLI reporting as an SBFD specific feature. However, according to the RAN1 agreement in RAN1#121, the L1-based reporting is also supported for UEs not configured with SBFD symbols, i.e., it is supported for legacy UEs:

Agreement For a CSI report associated with periodic/semi-persistent SRS-RSRP measurement resources or CLI-RSSI measurement resources, the valid symbol type for CSI derivation configured in CSI-ReportConfig is applicable for L1 UE-to-UE CLI measurement and reporting if SBFD symbols are configured for the UE - UE shall perform L1 UE-to-UE CLI measurement based on the valid symbol type configured in CSI-ReportConfig The valid symbol type does not apply for a UE if SBFD symbols are not configured for the UE - UE shall perform L1 UE-to-UE CLI measurement based on the configuration provided in CSI-ResourceConfig indicated by CSI-ReportConfig FG 60-1 is not the prerequisite of FG 60-8 and FG60-9

Therefore, we suggest updating the current description in Section 17.2 as follows:

Two types of UE-to-UE CLI reporting are supported: L1-based reporting and L3-based reporting. A UE is not expected to be configured with both L1 CLI measurement and reporting and L3 CLI measurement and reporting simultaneously.

Proposal 1: Update the Section 17.2 of TS 38.300 to capture the support of L1-based reporting for TDD and SBFD: Two types of UE-to-UE CLI reporting are supported in case of Sub-Band Full Duplex (SBFD) operation: L1-based reporting and L3-based reporting. A UE is not expected to be configured with both L1 CLI measurement and reporting and L3 CLI measurement and reporting simultaneously.

2  SBFD subband frequency location configuration

RAN1 sent an LS to RAN2 in R1-2508108 to inform that the assumptions of firstDL-subbandlocationAndBandwidth, secondDL-subbandlocationAndBandwidth and ul-subbandlocationAndBandwidth highlighted below are described in TS 38.213, clause 11.1 instead of clause 12.

firstDL-subbandlocationAndBandwidth Configures frequency domain location and bandwidth of the first DL subband. The value of the field shall be interpreted as resource indicator value (RIV) as defined in TS 38.214 with assumptions as described in TS 38.213 [13], clause 12. The network does not configure this field for UL carriers.

secondDL-subbandlocationAndBandwidth Configures frequency domain location and bandwidth of the second DL subband. The value of the field shall be interpreted as resource indicator value (RIV) as defined in TS 38.214 with assumptions as described in TS 38.213 [13], clause 12. The network does not configure this field for UL carriers.

ul-subbandlocationAndBandwidth Configures frequency domain location and bandwidth of UL subband. The value of the field shall be interpreted as resource indicator value (RIV) as defined in TS 38.214 with assumptions as described in TS 38.213 [13], clause 12. The network does not configure this field for DL carriers.

Proposal 2:  Replace clause 12 with clause 11.1 in the field description of the firstDL-subbandlocationAndBandwidth, secondDL-subbandlocationAndBandwidth and ul-subbandlocationAndBandwidth.

3. Symbol Type

RAN1 introduced two modes of SBFD operation for the transmission and reception of semi-static signals, e.g., configured-grant PUSCH or semi-persistent PDSCH. These 2 modes are denoted as Configuration 1 and Configuration 2. For Configuration 1, transmissions and receptions are constrained to one symbol type, i.e., either SBFD or non-SBFD symbols. For this to function correctly, the UE must be configured with a symbol type through RRC procedures. This is indicated by the parameter symbolType, which is set as optional in current TS 38.331.

Observation 1: For SBFD Configuration 1 to work correctly, the SBFD UE must be configured with symbolType.

The optional configuration of the parameter has triggered discussions in RAN1 about the UE default behaviour in case symbolType is not configured. In our view, rather than discussing the UE default behaviour, we should ensure that the network always configures the UE with symbolType for Configuration 1. Therefore, from RAN2 perspective, we propose to update the symbolType description to avoid any ambiguity.

Observation 2: Current TS 38.331 specification defines symbolType as optional, which has triggered RAN1 discussions about SBFD UE behaviour if the parameter is not configured.

Taking ConfiguredGrantConfig IE as an example, the symbolType is currently described as:

symbolType Configures the valid symbol type for Type 1 CG PUSCH when the transmissions are restricted to SBFD symbols only or non-SBFD symbols only. The network does not configure this field if the transmissions can be in SBFD symbols and non-SBFD symbols in different slots for the UL BWP. (see TS 38.214 [19], clause 6).

We propose to introduce the following clarification:

#### – ConfiguredGrantConfig The IE ConfiguredGrantConfig is used to configure uplink transmission without dynamic grant according to two possible schemes. The actual uplink grant may either be configured via RRC (type1) or provided via the PDCCH (addressed to CS-RNTI) (type2). Multiple Configured Grant configurations may be configured in one BWP of a serving cell. ***** unchanged IEs and parts are omitted****** ConfiguredGrantConfig field descriptions. symbolType Configures the valid symbol type for Type 1 CG PUSCH if SBFD symbols are configured for the UE. The network configures this field when the transmissions are restricted to SBFD symbols only or non-SBFD symbols only. The network does not configure this field if the transmissions can be in SBFD symbols and non-SBFD symbols in different slots for the UL BWP. (see TS 38.214 [19], clause 6). timeDomainAllocation, timeDomainAllocation-v1710 Indicates a combination of start symbol and length and PUSCH mapping type, see TS 38.214 [19], clause 6.1.2 and TS 38.212 [17], clause 7.3.1. If the field timeDomainAllocation-v1710 is present, the UE shall ignore timeDomainAllocation field (without suffix)..

The same approach can be applied to other semi-static signals which are also relying on the symbolType indication.

Proposal 3: Update the description of symbolType parameter to clarify that the configuration of the parameter is mandatory for Configuration 1 in SBFD.

PS: This issue was identified after the RIL deadline, hence added directly to the contribution.

4. Conclusions

Observation 1: For SBFD Configuration 1 to work correctly, the SBFD UE must be configured with symbolType.

Observation 2: Current TS 38.331 specification defines symbolType as optional, which has triggered RAN1 discussions about SBFD UE behaviour if the parameter is not configured.

Proposal 1: Update the Section 17.2 of TS 38.300 to capture the support of L1-based reporting for TDD and SBFD: Two types of UE-to-UE CLI reporting are supported in case of Sub-Band Full Duplex (SBFD) operation: L1-based reporting and L3-based reporting. A UE is not expected to be configured with both L1 CLI measurement and reporting and L3 CLI measurement and reporting simultaneously.

Proposal 2:  Replace clause 12 with clause 11.1 in the field description of the firstDL-subbandlocationAndBandwidth, secondDL-subbandlocationAndBandwidth and ul-subbandlocationAndBandwidth.

Proposal 3: Update the description of symbolType parameter to clarify that the configuration of the parameter is mandatory for Configuration 1 in SBFD.