3GPP TSG-RAN WG2 Meeting #132	R2-2509375

# Dallas, US, 17–21 November, 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.300 | CR | 1047 | rev | 4 | Current version: | 19.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Miscellaneous Stage 2 corrections for NR NTN phase 3 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | THALES (Rapporteur) |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R2 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_NTN_Ph3-Core |  |  |  |  |  | Date: |  |  | 2025-11-28 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Miscellaneous stage 2 corrections for NR NTN phase 3 after RAN2#131 post-email discussion [301] and during RAN2#131-bis meeting. |  |  |  |  |  |  |  |  |
| Summary of change: |  | The corrections to the issues listed in R2-2507648 adressed by this CR are: - The description of the cell re-selection frequency prioritization for the UE capable of MBS Broadcast with ISA(s) - The description of the related enhancements for measurements in Downlink Coverage enhancements (i.e. 2 SMTC periodicities, 7 SMTC offsets and N closest location report) And editorial correction in clause 16.14.1. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Impact analysis Impacted functionality: NR NTN. Inter-operability: 1. If the network is implemented according to the CR and the UE is not, the UE will not support Rel-19 MBS or Downlink coverage enhancements 2. If the UE is implemented according to the CR and the network is not, there is no impact |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 16.14.1, 16.14.3.3, 16.14.12 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | X | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | R2-2509192 submitted to RAN2#132 R2-2508763 submitted to RAN2#132 R2-2507779 submitted to RAN2#131-bis R2-2507649 submitted to RAN2#131-bis |  |  |  |  |  |  |  |  |

First Modified Subclause

### 16.14.1	Overview

Figures 16.14.1-1 and 16.14.1-2 below illustrate examples of a Non-Terrestrial Network (NTN) providing non-terrestrial NR access to the UE by means of an NTN payload and an NTN Gateway, depicting a service link between the NTN payload and a UE, and a feeder link between the NTN Gateway and the NTN payload. In Figure 16.14.1-1, the payload is transparent; in Figure 16.14.1-2 the payload is regenerative and hosts a gNB.

Figure 16.14.1-1: Overall illustration of an NTN with transparent payload

Figure 16.14.1-2: Overall illustration of an NTN with regenerative payload hosting a gNB

NOTE 1:	Figure 16.14.1-1 and 16.14.1-2 illustrate an NTN; RAN4 aspects are out of scope.

The transparent NTN payload transparently forwards the radio protocol received from the UE (via the service link) to the NTN Gateway (via the feeder link) and vice-versa. The regenerative NTN payload terminates the Uu (via the service link), NG and Xn interfaces. The following connectivity is supported by the NTN payload:

-	An NTN gateway may serve multiple NTN payloads;

-	An NTN payload may be served by multiple NTN gateways.

NOTE 2:	In this release, the transparent NTN-payload may change the carrier frequency, before re-transmitting it on the service link, and vice versa (respectively on the feeder link).

The following connectivity is supported by the NTN regenerative payload:

-	A regenerative NTN payload may terminate one or more inter-satellite links toward other regenerative payloads.

For NTN, the following applies in addition to Network Identities as described in clause 8.2:

-	A Tracking Area corresponds to a fixed geographical area. Any respective mapping is configured in the RAN;

-	A Mapped Cell ID as specified in clause 16.14.5.

NTN can be deployed to provide coverage with earth-moving cell, quasi-earth-fixed cell and earth-fixed cell which are supported respectively by the following three types of service link:

-	Earth-fixed: provisioned by beam(s) continuously covering the same geographical areas all the time (e.g., the case of GSO satellites);

-	Quasi-Earth-fixed: provisioned by beam(s) covering one geographic area for a limited period and a different geographic area during another period (e.g., the case of NGSO satellites generating steerable beams);

-	Earth-moving: provisioned by beam(s) whose coverage area slides over the Earth surface (e.g., the case of NGSO satellites generating fixed or non-steerable beams).

With NGSO satellites, the gNB can provide either quasi-Earth-fixed service link or Earth-moving service link, while gNB operating with GSO satellite can provide Earth fixed service link or quasi-Earth-fixed service link.

In this release, the UE supporting NTN is GNSS-capable.

In NTN, the distance refers to Euclidean distance.

In this release, NTN is only applicable to FDD system.

Next Modified Subclause

#### 16.14.3.3	Measurements

The same principle as described in 9.2.4 applies to measurements in NTN unless hereunder specified.

The network can configure:

-	multiple SMTCs in parallel per carrier and for a given set of cells depending on UE capabilities;

-	measurement gaps based on multiple SMTCs;

-	assistance information (e.g., ephemeris, Common TA parameters, kmac) provided in SIB19 for UE to perform measurement on neighbour cells in RRC_IDLE/RRC_INACTIVE/RRC_CONNECTED;

-	two different SMTC periodicities for SMTCs per frequency layer for UE in RRC_IDLE/RRC_INACTIVE/RRC_CONNECTED;

-	up to 7 SMTCs for UE in RRC_IDLE/RRC_INACTIVE for the (quasi-)Earth fixed cells, including the SMTC of the serving cell, where the UE may down-select SMTCs associated with the closest reference locations.

NW-controlled adjustment of SMTCs can be based on UE assistance information reported in RRC_CONNECTED. The UE may be configured to report the closest reference locations to assist configuration of SMTC and measurement gap. A UE in RRC_IDLE/RRC_INACTIVE can adjust SMTCs based on its location and assistance information in SIB19.

UE assistance information consists of the service link propagation delay difference(s) between serving the cell and neighbour cell(s).

For a UE in idle/inactive mode, it is up to UE implementation whether to perform NTN neighbour cell measurements on a cell indicated in SIB3/SIB4 but not included in SIB19.

For a UE in connected mode, it is up to UE implementation whether to perform NTN neighbour cell measurements on a cell included in the measurement configuration, but without corresponding satellite information in measurement configuration or in SIB19.

UE can perform time-based and location-based measurements on neighbour cells in RRC_IDLE/RRC_INACTIVE:

-	The timing and location information associated to the serving cell is provided in SIB19;

-	Timing information refers to the UTC time when the serving cell stops serving the current geographical area;

-	Location information refers:

-	In the (quasi-)Earth fixed cell scenario, to the reference location of the serving cell and a distance threshold to the reference location.

-	In the Earth moving cell scenario, to the reference location of the serving cell at the epoch time and a distance threshold to the reference location.

The time-based measurement initiation may be applicable for the feeder link switchover case for cell re-selection.

Measurement rules for cell re-selection based on timing information and location information are specified in clause 5.2.4.2 in TS 38.304 [10].

Next Modified Subclause

### 16.14.12	Support of ISA for MBS Broadcast in NTN

An intended service area (ISA) is a geographical area that can be signaled for MBS broadcast services. The ISA(s) is broadcasted in SIB27 and associated to MBS broadcast service(s) by indicating the ISA identifier(s) in MCCH. It may be included in USD within target service area [46].

The NTN-capable UE supporting the MBS broadcast may initiate the broadcast MRB establishment procedure and prioritize the frequency providing these MBS broadcast service(s) when it is inside an ISA. The UE may initiate the broadcast MRB release procedure and no longer prioritize the frequency providing MBS broadcast service(s) when the UE leaves an ISA, as specified in clause 5.9.3 of TS 38.331 [12] and clause 5.2.4.1 of TS 38.304 [10].

NOTE:	When using ISA(s) for MBS broadcast service reception or MBS service continuity, it is up to the UE implementation how to determine if it is in the ISA(s) of MBS broadcast service or not.

End of Changes