3GPP TSG-RAN WG2 Meeting #132    	                                 R2-2509380

Dallas, USA, Nov 17th – 21st, 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.321 | CR | 1599 | rev | 3 | Current version: | 19.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Corrections for CB-MSG3 EDT |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | MediaTek Inc. |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R2 |  |  |  |  |  |  |  |  |  |
| Work item code: | IoT_NTN_Ph3-Core |  |  |  |  |  | Date: |  |  | 2025-11-25 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Some corrections for CB-Msg3-EDT procedure are needed. <1> CB-RNTI is only described as being used for CB-Msg4 monitoring, while the intention is that it should also be used for CB-Msg3 scrambling. <2> R2#132 agreed that - the determination of the MPDCCH narrowband for CB-Msg4 monitoring is based on the value of (Contention Resolution identifier included in CB-Msg3 mod 2). <3> The condition for triggering the Msg3 DCQR needs to be changed to accommodate CB-Msg3-EDT. <4> For efficient control of temporary peak in load on the CB-Msg3-EDT resources, it must be possible to spread failing users access attempts in the future CB-Msg3 transmission windows. The back off values must cover many multiples of CB-Msg3 periodicity. Consider the repetition, we may need longer back off value. It is simple to have back off value configured as a number of CB-Msg3 transmission window periodicities and it could apply to both NB-IoT and eMTC. <5> For CB-Msg3-EDT, the UE can perform uplink transmission without the TAT timer running. This is not clear in current MAC SPEC. <6> The meaning of “CMR reception successful” is unclear. There two terminologies used, one is “CMR” and the other is “MAC CMR”. The terminology should be aligned. <7> For CB-Msg3-EDT, grants are provided via CB-Msg3-ConfigSIB(-NB) in system information. It is a new and scenario-specific UL grant that is exclusively for CB-Msg3-EDT and not covered by existing UL grant definitions in the MAC spec. |  |  |  |  |  |  |  |  |
| Summary of change: |  | <1> Move the CB-RNTI derivation description from “clause 5.1a.3 CB-Msg4” reception to “clause 5.1a.2 CB-Msg3 transmission”; and in “Table 7.1-2: RNTI usage”, define CB-RNTI is used for CB-Msg3 transmission. <2> In 5.1a.3, determinate which MPDCCH narrowband is used for CB-Msg4 monitoring. <3> The condition for triggering the Msg3 DCQR is updated to include CB-Msg3-EDT case. <4> The back off index 0 to 12 indicates a back off parameter value of {0, 2, 4, 8, 10, 12, 16, 20, 24, 28, 32, 36, 40} multiples of the CB-Msg3-transmission window periodicity. The back off indexes 13 to 15 are reserved for future usage. <5> Clarify that for CB-Msg3-EDT, the UE can perform uplink transmission without the timeAlignmentTimer running <6> To align the terminology, use "MAC CMR" instead of just “CMR”. Replace the confusing terminology "CMR reception successful" with “Contention Resolution successful". <7> Clarify in clause 5.4.1 on the usage of CB-Msg3 grants for UL-SCH transmission. <8> Editorial changes and wording refinement. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The CB-Msg3-EDT procedure still has some issue |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.1a.1, 5.1a.2, 5.1a.3, 5.2, 5.3.2.2, 5.4.1, 5.25, 6.1.7, 6.2.6, 7.1, 7.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | x | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | R2-2507555, R2-2507656, R2-2508990 |  |  |  |  |  |  |  |  |

# 5	MAC procedures

<Skip>

## 5.1a	CB-Msg3-EDT Procedure

### 5.1a.1	CB-Msg3-EDT initialization

The CB-Msg3-EDT procedure is initiated by the RRC sublayer and can only be performed in a non-terrestrial network. If the UE is an NB-IoT UE, the CB-Msg3-EDT procedure is performed on the anchor carrier or one of the non-anchor carriers for which CB-Msg3-EDT resource has been configured in system information.

The following information is assumed to be available before the procedure can be initiated for NB-IoT UEs, BL UEs or UEs in CE mode A, as specified in TS 36.331 [8]:

-	if the UE is a BL UE or a UE in CE mode A:

-	the available set of PUSCH resources associated with one or more enhanced coverage levels for the transmission of the CB-Msg3, cb-Msg3-ConfigSIB.

-	if the UE is an NB-IoT UE:

-	the available sets of PUSCH resources on the anchor carrier, cb-Msg3-ConfigSIB-NB in SystemInformationBlockType2-NB and on the non-anchor carriers, in ul-ConfigList.

-	the mapping of the PUSCH resources into enhanced coverage levels is determined according to the following:

-	the number of enhanced coverage levels is equal to one plus the number of RSRP thresholds present in cb-Msg3-RSRP-CE-Levels-NB.

-	each enhanced coverage level has zero or one anchor carrier PUSCH resource present in cb-Msg3-Config-NB and zero or one PUSCH resource for each non-anchor carrier signalled in ul-ConfigList.

-	enhanced coverage levels are numbered from 0 and the mapping of PUSCH resources to enhanced coverage levels are done in increasing npusch-NumRepetitionsIndex order.

-	when multiple carriers provide PUSCH resources for the same enhanced coverage level, the UE shall randomly select one of them using the following selection probabilities:

-	the selection probability of the anchor carrier PUSCH resource for the given enhanced coverage level, cb-Msg3-ProbabilityAnchor-NB is given by the corresponding entry in cb-Msg3-ProbabilityAnchorList-NB.

-	the selection probability is equal for all non-anchor carrier PUSCH resources and the probability of selecting one PUSCH resource on a given non-anchor carrier is (1 - cb-Msg3-ProbabilityAnchor-NB)/(number of non-anchor PUSCH resources).

-	the criteria to select PUSCH resources based on RSRP measurement per enhanced coverage level, cb-Msg3-RSRP-CE-Level (cb-Msg3-RSRP-CE-Levels-NB for NB-IoT).

-	the number of replicas for CB-Msg3 transmission corresponding to the selected enhanced coverage level, cb-Msg3-NumOfReplicas.

-	CB-Msg3 transmission window configuration corresponding to the selected enhanced coverage level, cb-Msg3-TxWindow, including the transmission window periodicity windowPeriodicity.

-	CB-Msg3ResponseTimer corresponding to the selected enhanced coverage level, cb-Msg3-ResponseWindow.

-	the maximum number of transmission attempts per enhanced coverage level, cb-Msg3-MaxAttemptNum.

-	power ramping factor for CB-Msg3 transmission, powerRampingStep-r19.

-	initial CB-Msg3 transmission power, cb-Msg3-InitialReceivedTargetPower.

The CB-Msg3-EDT procedure shall be performed as follows:

-	flush the Msg3 buffer;

-	set the CB_MSG3_TRANSMISSION_COUNTER_CE to 1;

-	if the UE is an NB-IoT UE and if the RSRP threshold of enhanced coverage level 2 configured by upper layers in cb-Msg3-RSRP-CE-Levels-NB and the measured RSRP is less than the RSRP threshold of enhanced coverage level 2:

-	the MAC entity considers to be in enhanced coverage level 2;

-	else if the measured RSRP is less than the RSRP threshold of enhanced coverage level 1 as configured by upper layers in cb-Msg3-RSRP-CE-Level (cb-Msg3-RSRP-CE-Levels-NB for NB-IoT) then:

-	the MAC entity considers to be in enhanced coverage level 1;

-	else:

-	the MAC entity considers to be in enhanced coverage level 0;

-	set the CB-Msg3-EDT backoff parameter value to 0 ms;

-	set current NTA to 0;

-	proceed to the transmission of the CB-Msg3 (see clause 5.1a.2).

### 5.1a.2	CB-Msg3 transmission

Before the CB-Msg3 transmission, the MAC entity shall select cb-Msg3-NumOfReplicas UL grants for CB-Msg3 transmission as follows:

-	select the next upcoming CB-Msg3 transmission window provided by the cb-Msg3-TxWindow associated with the selected enhanced coverage level;

-	randomly select cb-Msg3-NumOfReplicas PUSCH resources in the time domain within the selected CB-Msg3 transmission window from the CB-Msg3 resources provided by cb-Msg3-TimeResource associated with the selected enhanced coverage level;

-	randomly select a frequency domain PUSCH resource for each selected time domain resource provided by prb-AllocationInfoSet (npusch-SubCarrierSetList for NB-IoT) associated with the selected enhanced coverage level.

For BL UEs and UEs in CE mode A, the CB-RNTI associated with the CB-Msg3 transmission window in which the CB-Msg3(s) are transmitted, is computed as:

CB-RNTI = floor (SFN_id/WP) modulo (32) + 32*CE_level + 2401

Where:

-	SFN_id is the first SFN of the selected CB-Msg3 transmission window.

-	WP is the configured CB-Msg3 transmission window periodicity in unit of 10 ms provided by windowPeriodicity.

-	CE_level is the selected enhanced coverage level (0 <= CE_level < 2).

For NB-IoT UEs, the CB-RNTI associated with the CB-Msg3 transmission window in which the CB-Msg3(s) are transmitted, is computed as:

CB-RNTI = floor (SFN_id/WP) modulo (32) + 32*CE_level + 96*carrier_id + 4097

Where:

-	SFN_id is the first SFN of the selected CB-Msg3 transmission window.

-	WP is the configured CB-Msg3 transmission window periodicity in unit of 10 ms provided by windowPeriodicity.

-	CE_level is the selected enhanced coverage level (0 <= CE_level < 3).

-	carrier_id is the index of the UL carrier associated with the selected UL grants (0 <= carrier_id < 16). The carrier_id of the anchor carrier is 0.

The CB-Msg3 transmission shall be performed as follows:

-	the CB-MSG3_RECEIVED_TARGET_POWER is set to cb-Msg3-InitialReceivedTargetPower + (CB_MSG3_TRANSMISSION_COUNTER_CE – 1) * powerRampingStep;

-	if this is the first attempt of CB-Msg3 transmission within the selected CB-Msg3 transmission window:

-	obtain the MAC PDU to transmit from the "Multiplexing and assembly" entity and store it in the Msg3 buffer.

### 5.1a.3	CB-Msg4 reception

Once all the CB-Msg3s of a CB-Msg3 transmission window are transmitted, the MAC entity shall monitor the PDCCH of the SpCell, while CB-Msg3ResponseTimer is running, for CB-Msg4(s) identified by the CB-RNTI as defined in clause 5.1a.2.

For BL UEs and UEs in CE mode A, the CB-RNTI associated with the CB-Msg3 transmission window in which the CB-Msg3(s) are transmitted, is computed as:

CB-RNTI = floor (SFN_id/WP) modulo (32) + 32*CE_level + 2401

Where:

-	SFN_id is the first SFN of the selected CB-Msg3 transmission window.

-	WP is the configured CB-Msg3 transmission window periodicity in unit of 10 ms provided by windowPeriodicity.

-	CE_level is the selected enhanced coverage level (0 <= CE_level < 2).

For NB-IoT UEs, the CB-RNTI associated with the CB-Msg3 transmission window in which the CB-Msg3(s) are transmitted, is computed as:

CB-RNTI = floor (SFN_id/WP) modulo (32) + 32*CE_level + 96*carrier_id + 4097

Where:

-	SFN_id is the first SFN of the selected CB-Msg3 transmission window.

-	WP is the configured CB-Msg3 transmission window periodicity in unit of 10 ms provided by windowPeriodicity.

-	CE_level is the selected enhanced coverage level (0 <= CE_level < 3).

-	carrier_id is the index of the UL carrier associated with the selected UL grants (0 <= carrier_id < 16). The carrier_id of the anchor carrier is 0.

For BL UEs and UEs in CE mode A, the narrowband used for first subframe of MPDCCH for CB-Msg4 monitoring is determined by higher layer parameter mpdcch-Narrowband-r19 if only one narrowband is configured, otherwise, it is determined from Table 5.1a.3-x.

Table 5.1a.3-x: Narrowband for MPDCCH of CB-Msg4

| Mapped UE Contention Resolution Identity | Narrowband for MPDCCH monitoring |
| --- | --- |
| (UE contention resolution Identity) modulo 2 = 0 | First narrowband configured by higher layer parameter mpdcch-Narrowband-r19 |
| (UE contention resolution Identity) modulo 2 = 1 | Second narrowband configured by higher layer parameter mpdcch-Narrowband-r19 |

After the CB-Msg3 transmission, the MAC entity shall:

-	start the CB-Msg3ResponseTimer at the subframe that contains the end of the selected CB-Msg3 transmission window plus UE-eNB RTT;

-	monitor the PDCCH while CB-Msg3ResponseTimer is running;

-	if notification of a reception of a PDCCH transmission is received from lower layers; and

-	if the CB-Msg4 is successfully decoded:

-	if the CB-Msg4 contains a CB Backoff Indicator subheader:

-	set the CB-Msg3-EDT backoff parameter value as indicated by the BI field of the CB Backoff Indicator subheader and Table 7.2-x.

-	else:

-	set the CB-Msg3-EDT backoff parameter value to 0 ms.

-	if the CB-Msg4 contains one or more MAC CMRs; and

-	if there is a UE Contention Resolution Identity in a MAC CMR that matches the first 48 bits of the CCCH SDU transmitted in CB-Msg3(s):

-	consider this Contention Resolution successful;

-	discard the CB-RNTI;

-	stop CB-Msg3ResponseTimer if running;

-	flush the HARQ buffer used for transmission of the MAC PDU in the Msg3 buffer;

-	if the corresponding MAC CMR contains a Timing Advance Command field:

-	process the received Timing Advance Command (see clause 5.2).

-	start the timeAlignmentTimer associated with this TAG;

-	if the corresponding MAC CMR contains a C-RNTI field:

-	set the C-RNTI to the value of the C-RNTI field.

-	finish the disassembly and demultiplexing of the MAC PDU;

-	consider CB-Msg3 transmission successfully completed;

-	if the C-RNTI field is absent in the corresponding MAC CMR and no corresponding MAC SDU is present in CB-Msg4:

-	indicate the CB-Msg3-EDT procedures have been successfully completed without an RRC message to the upper layers.

NOTE:	The UE can receive CB-Msg4 after the CB-Msg3ResponseTimer expired if the corresponding PDCCH is received before the CB-Msg3ResponseTimer expired.

-	if the CB-Msg3ResponseTimer expires:

-	when MAC entity has finished decoding of all received CB-Msg4 and if Contention Resolution is not successful:

-	flush the HARQ buffer used for transmission of the MAC PDU in the Msg3 buffer;

-	increment CB_MSG3_TRANSMISSION_COUNTER_CE by 1;

-	if CB_MSG3_TRANSMISSION_COUNTER_CE = cb-Msg3-MaxAttemptNum for the corresponding enhanced coverage level + 1:

-	discard the CB-RNTI;

-	consider the CB-Msg3 transmission unsuccessfully completed;

-	indicate the transmission using CB-Msg3-EDT has failed to the upper layers.

-	else:

-	based on the CB-Msg3-EDT backoff parameter, select a random backoff time according to a uniform distribution between 0 and the CB-Msg3-EDT backoff parameter value;

-	delay the subsequent CB-Msg3 transmission by the backoff time;

-	proceed to the CB-Msg3 transmission (see clause 5.1a.2).

<Skip>

## 5.2	Maintenance of Uplink Time Alignment

The MAC entity has a configurable timer timeAlignmentTimer per TAG. The timeAlignmentTimer is used to control how long the MAC entity considers the Serving Cells belonging to the associated TAG to be uplink time aligned, as specified in TS 36.331 [8].

The MAC entity shall:

-	when a Timing Advance Command MAC control element is received and if a NTA has been stored or maintained with the indicated TAG:

-	except when the received Timing Advance Command MAC control element is addressed with a PUR-RNTI:

-	apply the Timing Advance Command for the indicated TAG;

-	start or restart the timeAlignmentTimer associated with the indicated TAG.

-	when a Timing Advance Command is received in a Random Access Response message for a serving cell belonging to a TAG:

-	if the UE is configured with pur-Config (see TS 36.331 [8]) and if a NTA has been stored or maintained and no temporary NTA has been stored:

-	store current NTA as temporary NTA (see clause 5.4.7.2).

-	if the Random Access Preamble was not selected by the MAC entity:

-	apply the Timing Advance Command for this TAG;

-	start or restart the timeAlignmentTimer associated with this TAG.

-	else, if the timeAlignmentTimer associated with this TAG is not running:

-	apply the Timing Advance Command for this TAG;

-	start the timeAlignmentTimer associated with this TAG;

-	when the contention resolution is considered not successful as described in clause 5.1.5, stop timeAlignmentTimer associated with this TAG.

-	else:

-	ignore the received Timing Advance Command.

-	when a Timing Advance Command is received in a successful CMR for a serving cell belonging to a TAG:

-	apply the Timing Advance Command for this TAG.

-	when the MAC entity is configured with rach-Skip or rach-SkipSCG:

-	apply timing advance value indicated by targetTA in rach-Skip or rach-SkipSCG for the pTAG;

-	start the timeAlignmentTimer associated with this TAG.

-	when a timeAlignmentTimer expires:

-	if the timeAlignmentTimer is associated with the pTAG:

-	flush all HARQ buffers for all serving cells;

-	notify RRC to release PUCCH/SPUCCH for all serving cells;

-	notify RRC to release SRS for all serving cells;

-	for NB-IoT, notify RRC to release all dedicated resources for SR;

-	clear any configured downlink assignments and uplink grants;

-	consider all running timeAlignmentTimers as expired;

-	else if the timeAlignmentTimer is associated with an sTAG, then for all Serving Cells belonging to this TAG:

-	flush all HARQ buffers;

-	notify RRC to release SRS;

-	notify RRC to release PUCCH/SPUCCH, if configured;

-	clear any configured downlink assignments and uplink grants.

-	upon indication from upper layers to start timeAlignmentTimer, if a NTA has been stored or maintained with the indicated TAG:

-	start or restart the timeAlignmentTimer associated with the indicated TAG.

When the MAC entity stops uplink transmissions for an SCell due to the fact that the maximum uplink transmission timing difference (as described in clause 7.9.2 of TS 36.133 [9]) or the maximum uplink transmission timing difference the UE can handle between TAGs of any MAC entity of the UE is exceeded, the MAC entity considers the timeAlignmentTimer associated with the SCell as expired.

The MAC entity shall not perform any uplink transmission on a Serving Cell, except the Random Access Preamble transmission, transmissions corresponding to a PUR-RNTI, and transmissions for the CB-Msg3-EDT procedure, when the timeAlignmentTimer associated with the TAG to which this Serving Cell belongs is not running. Furthermore, when the timeAlignmentTimer associated with the pTAG is not running, the MAC entity shall not perform any uplink transmission on any Serving Cell except the Random Access Preamble transmission on the SpCell.

The MAC entity shall not perform any sidelink transmission which is performed based on UL timing of the corresponding serving cell and any associated SCI transmissions when the corresponding timeAlignmentTimer is not running.

NOTE:	A MAC entity stores or maintains NTA upon expiry of associated timeAlignmentTimer, where NTA is defined in TS 36.211 [7]. The MAC entity applies a received Timing Advance Command MAC control element and starts associated timeAlignmentTimer also when the timeAlignmentTimer is not running.

<Skip>

#### 5.3.2.2	HARQ process

For each TTI where a transmission takes place for the HARQ process, one or two (in case of downlink spatial multiplexing) TBs and the associated HARQ information are received from the HARQ entity.

For each received TB and associated HARQ information, the HARQ process shall:

-	if the NDI, when provided, has been toggled compared to the value of the previous received transmission corresponding to this TB; or

-	if the HARQ process is equal to the broadcast process and if this is the first received transmission for the TB according to the system information schedule indicated by RRC; or

-	if this is the very first received transmission for this TB (i.e. there is no previous NDI for this TB):

-	consider this transmission to be a new transmission.

-	else:

-	consider this transmission to be a retransmission.

The MAC entity then shall:

-	if this is a new transmission:

-	attempt to decode the received data.

-	else if this is a retransmission:

-	if the data for this TB has not yet been successfully decoded:

-	combine the received data with the data currently in the soft buffer for this TB and attempt to decode the combined data.

-	if the data which the MAC entity attempted to decode was successfully decoded for this TB; or

-	if the data for this TB was successfully decoded before:

-	if the HARQ process is equal to the broadcast process:

-	deliver the decoded MAC PDU to upper layers.

-	else if this is the first successful decoding of the data for this TB:

-	deliver the decoded MAC PDU to the disassembly and demultiplexing entity.

-	generate a positive acknowledgement (ACK) of the data in this TB.

-	else:

-	replace the data in the soft buffer for this TB with the data which the MAC entity attempted to decode.

-	generate a negative acknowledgement (NACK) of the data in this TB.

-	if the HARQ process is associated with a transmission indicated with a Temporary C-RNTI and the Contention Resolution is not yet successful (see clause 5.1.5); or

-	if the HARQ process is equal to the broadcast process; or

-	if the HARQ process is not associated with a transmission indicated with a PUR-RNTI or CB-RNTI, and the timeAlignmentTimer, associated with the TAG containing the serving cell on which the HARQ feedback is to be transmitted, is stopped or expired:

-	do not indicate the generated positive or negative acknowledgement to the physical layer.

-	else if the HARQ process is associated with a transmission indicated with a CB-RNTI:

-	if the Contention Resolution is successful (see clause 5.1a.3); and

-	if the HARQ ACK resource field is present in the associated CMR:

-	indicate to the physical layer the generated positive acknowledgement together with the HARQ ACK resource.

-	else:

-	do not indicate the generated positive or negative acknowledgement to the physical layer.

-	else if the HARQ feedback is disabled for the corresponding HARQ process:

-	if harq-FeedbackEnablingforSPSactive is configured and the transmission is the first SPS PDSCH transmission after SPS activation:

-	indicate the generated positive or negative acknowledgement for this TB to the physical layer.

-	else:

-	do not indicate the generated positive or negative acknowledgement to the physical layer.

-	else:

-	indicate the generated positive or negative acknowledgement for this TB to the physical layer.

The MAC entity shall ignore NDI received in all downlink assignments on PDCCH for its Temporary C-RNTI when determining if NDI on PDCCH for its C-RNTI has been toggled compared to the value in the previous transmission.

NOTE 1:	When the MAC entity is configured with more than one serving cell, UE behaviors for storing data to the soft buffer is specified in TS 36.213 [2].

NOTE 2:	If the MAC entity receives a retransmission with a TB size different from the last valid TB size signalled for this TB, the UE behavior is left up to UE implementation.

<Skip>

## 5.4	UL-SCH data transfer

### 5.4.1	UL Grant reception

In order to transmit on the UL-SCH the MAC entity must have a valid uplink grant (except for non-adaptive HARQ retransmissions) which it may receive dynamically on the PDCCH or in a Random Access Response or which may be configured semi-persistently or preallocated by RRC or provided by RRC for transmission using PUR (see clause 5.4.7) or provided in system information for CB-Msg3 transmission (see clause 5.1a). To perform requested transmissions, the MAC layer receives HARQ information from lower layers. When the physical layer is configured for uplink spatial multiplexing, the MAC layer can receive up to two grants (one per HARQ process) for the same TTI from lower layers.

If the MAC entity has a C-RNTI, a Semi-Persistent Scheduling C-RNTI, a UL Semi-Persistent Scheduling V-RNTI, a AUL C-RNTI, or a Temporary C-RNTI, the MAC entity shall for each TTI and for each Serving Cell belonging to a TAG that has a running timeAlignmentTimer and for each grant received for this TTI and for each SPS configuration that is indicated by the PDCCH addressed to UL Semi-Persistent Scheduling V-RNTI; or if the MAC entity has Preconfigured Uplink Resource RNTI, the MAC entity shall for each TTI and for each grant received for this TTI; or if the MAC entity has CB-RNTI, the MAC entity shall for each TTI and for the uplink grant selected for this TTI:

-	if an uplink grant for this TTI and this Serving Cell has been received on the PDCCH for the MAC entity's C-RNTI, Preconfigured Uplink Resource RNTI or Temporary C-RNTI; or

-	if an uplink grant for this TTI has been received in a Random Access Response; or

-	if the uplink grant was selected by MAC for CB-Msg3-EDT:

-	if the uplink grant is for MAC entity's C-RNTI and if the previous uplink grant delivered to the HARQ entity for the same HARQ process was either an uplink grant received for the MAC entity's Semi-Persistent Scheduling C-RNTI, for the MAC entity's UL Semi-Persistent Scheduling V-RNTI, or a configured uplink grant for which the UL HARQ operation was not autonomous; or

-	if the uplink grant is for MAC entity’s CB-RNTI:

-	consider the NDI to have been toggled for the corresponding HARQ process regardless of the value of the NDI.

-	deliver the uplink grant and the associated HARQ information to the HARQ entity for this TTI.

-	else, if an uplink grant for this TTI has been received for this Serving Cell on the PDCCH for the MAC entity's Semi-Persistent Scheduling C-RNTI or for the MAC entity's UL Semi-Persistent Scheduling V-RNTI; or if an uplink grant for this TTI has been received for this Serving Cell on the PDCCH for the MAC entity's AUL C-RNTI:

-	if the NDI in the received HARQ information is 1:

-	consider the NDI for the corresponding HARQ process not to have been toggled;

-	deliver the uplink grant and the associated HARQ information to the HARQ entity for this TTI.

-	else if the NDI in the received HARQ information is 0:

-	if PDCCH contents indicate AUL release:

-	trigger an AUL confirmation;

-	if an uplink grant for this TTI has been configured:

-	consider the NDI bit for the corresponding HARQ process to have been toggled;

-	deliver the configured uplink grant and the associated HARQ information to the HARQ entity for this TTI;

-	else if PDCCH contents indicate AUL activation:

-	trigger an AUL confirmation;

-	store the uplink grant and the associated HARQ information as configured uplink grant;

-	initialise (if not active) or re-initialise (if already active) the configured uplink grant to start in this TTI and to recur according to rules in clause 5.23;

-	consider the NDI bit for the corresponding HARQ process to have been toggled;

-	deliver the configured uplink grant and the associated HARQ information to the HARQ entity for this TTI.

-	else if PDCCH contents indicate SPS release:

-	if the MAC entity is configured with skipUplinkTxSPS:

-	trigger an SPS confirmation;

-	if an uplink grant for this TTI has been configured:

-	consider the NDI bit for the corresponding HARQ process to have been toggled;

-	deliver the configured uplink grant and the associated HARQ information to the HARQ entity for this TTI;

-	else:

-	clear the corresponding configured uplink grant (if any).

-	else:

-	if the MAC entity is configured with skipUplinkTxSPS:

-	trigger an SPS confirmation;

-	store the uplink grant and the associated HARQ information as configured uplink grant;

-	initialise (if not active) or re-initialise (if already active) the configured uplink grant to start in this TTI, or in TTI according to N=0 in clause 5.10.2 for short TTI, and to recur according to rules in clause 5.10.2;

-	if UL HARQ operation is asynchronous, set the HARQ Process ID to the HARQ Process ID associated with this TTI;

-	consider the NDI bit for the corresponding HARQ process to have been toggled;

-	deliver the configured uplink grant and the associated HARQ information to the HARQ entity for this TTI.

-	else, if an uplink grant for this TTI has been configured for the Serving Cell and if UL HARQ operation is autonomous for the corresponding HARQ process:

-	if the HARQ_FEEDBACK is set to ACK for the corresponding HARQ process or if there is no uplink grant previously delivered to the HARQ entity for the same HARQ process:

-	consider the NDI bit for the corresponding HARQ process to have been toggled.

-	if the aul-RetransmissionTimer is not running:

-	if there is no uplink grant previously delivered to the HARQ entity for the same HARQ process; or

-	if the previous uplink grant delivered to the HARQ entity for the same HARQ process was not an uplink grant received for the MAC entity's C-RNTI; or

-	if the HARQ_FEEDBACK is set to ACK for the corresponding HARQ process:

-	deliver the configured uplink grant, and the associated HARQ information to the HARQ entity for this TTI.

-	else:

-	if this Serving Cell is the SpCell and an uplink grant for this TTI has been preallocated for the SpCell; or

-	except for preconfigured uplink grant for PUR, if an uplink grant for this TTI has been configured for this Serving Cell:

-	if UL HARQ operation is asynchronous, set the HARQ Process ID to the HARQ Process ID associated with this TTI;

-	consider the NDI bit for the corresponding HARQ process to have been toggled;

-	deliver the configured or preallocated uplink grant, and the associated HARQ information to the HARQ entity for this TTI.

NOTE 1:	The period of configured uplink grants is expressed in TTIs.

NOTE 2:	If the MAC entity receives both a grant in a Random Access Response and a grant for its C-RNTI or Semi persistent scheduling C-RNTI requiring transmissions on the SpCell in the same UL subframe, the MAC entity may choose to continue with either the grant for its RA-RNTI or the grant for its C-RNTI or Semi persistent scheduling C-RNTI.

NOTE 3:	When a configured uplink grant is indicated during a measurement gap and indicates an UL-SCH transmission during a measurement gap, the MAC entity processes the grant but does not transmit on UL-SCH. When a configured uplink grant is indicated during a Sidelink Discovery gap for reception and indicates an UL-SCH transmission during a Sidelink Discovery gap for transmission with a SL-DCH transmission, the MAC entity processes the grant but does not transmit on UL-SCH. When a configured uplink grant indicates an UL-SCH transmission during a V2X sidelink communication transmission and transmission of V2X sidelink communication is prioritized as described in clause 5.14.1.2.2, the MAC entity processes the grant but does not transmit on UL-SCH.

NOTE 4:	The NDI transmitted in the PDCCH for the MAC entity's AUL C-RNTI is set to '0' (TS 36.212 [5]).

Except for NB-IoT, for configured uplink grants without harq-ProcID-offset, if UL HARQ operation is not autonomous, the HARQ Process ID associated with this TTI is derived from the following equation for asynchronous UL HARQ operation:

-	if the TTI is a subframe TTI:

-	HARQ Process ID = [floor(CURRENT_TTI/semiPersistSchedIntervalUL)] modulo numberOfConfUlSPS-Processes,

where CURRENT_TTI=[(SFN * 10) + subframe number] and it refers to the subframe where the first transmission of a bundle takes place.

-	else:

-	HARQ Process ID = [floor(CURRENT_TTI/semiPersistSchedIntervalUL-sTTI)] modulo numberOfConfUlSPS-Processes-sTTI,

where CURRENT_TTI = [(SFN * 10 * sTTI_Number_Per_Subframe) + subframe number * sTTI_Number_Per_Subframe + sTTI_number] and it refers to the short TTI occasion where the first transmission of a bundle takes place. Refer to 5.10.2 for sTTI_Number_Per_Subframe and sTTI_number.

For preallocated uplink grants the HARQ Process ID associated with this TTI is derived from the following equation for asynchronous UL HARQ operation:

HARQ Process ID = [floor(CURRENT_TTI/ul-SchedInterval)] modulo numberOfConfUL-Processes,

where CURRENT_TTI=subframe number and it refers to the subframe where the first transmission of a bundle takes place.

For configured uplink grants, if UL HARQ operation is autonomous, the HARQ Process ID associated with this TTI for transmission on this Serving Cell is selected by the UE implementation from the HARQ process IDs that are configured for autonomous UL HARQ operation by upper layers in aul-HARQ-Processes (TS 36.331 [8]).

For configured uplink grants with harq-ProcID-offset, the HARQ Process ID associated with this TTI is derived from the following equation for asynchronous UL HARQ operation:

-	if the TTI is a subframe TTI:

-	HARQ Process ID = [floor(CURRENT_TTI/semiPersistSchedIntervalUL)] modulo numberOfConfUlSPS-Processes + harq-ProcID-offset,

where CURRENT_TTI = [(SFN * 10) + subframe number] and it refers to the subframe where the first transmission of a bundle takes place.

-	else:

-	HARQ Process ID = [floor(CURRENT_TTI/semiPersistSchedIntervalUL-sTTI)] modulo numberOfConfUlSPS-Processes-sTTI + harq-ProcID-offset,

where CURRENT_TTI = [(SFN * 10 * sTTI_Number_Per_Subframe) + subframe number * sTTI_Number_Per_Subframe + sTTI_number] and it refers to the short TTI occasion where the first transmission of a bundle takes place. Refer to 5.10.2 for sTTI_Number_Per_Subframe and sTTI_number. For NB-IoT, for configured uplink grants for BSR, the HARQ Process ID is set to 0.

If the MAC entity is configured with Short Processing Time or short TTI and if current_TTI is a subframe TTI, the HARQ Process ID associated with this TTI is derived from the following equation for synchronous UL HARQ operation:

HARQ Process ID = [SFN * number_of_UL_PUSCH_SFs_per_radio_frame + index_of_UL_PUSCH_SF] modulo number_of_UL_HARQ_processes.

where number_of_UL_PUSCH_SFs_per_radio_frame is the number of subframes that can be used for PUSCH (UL PUSCH subframe) per radio frame:

-	For FDD serving cells and serving cells operating according to Frame structure Type 3, all 10 subframes in a radio frame represent UL PUSCH subframes;

-	For TDD serving cells, all uplink subframes of the TDD UL/DL configuration indicated by tdd-Config, as specified in TS 36.331 [8] of the cell represent UL PUSCH subframes and additionally the subframes including UpPTS if the cell is configured with symPUSCH-UpPts-r14;

and index_of_UL_PUSCH_SF is the index of a subframe that can be used for PUSCH within the radio frame, and number_of_UL_HARQ_processes is the number of parallel HARQ processes per HARQ entity for subframe TTI as specified in TS 36.213 [2], clause 8.

<Skip>

## 5.25	Transmission of Downlink Channel Quality Report

The MAC entity of a BL UE or UE in enhanced coverage may be configured by upper layers to report DL channel quality in Msg3. DL channel quality in Msg3 in RRC_CONNECTED is not reported.

If the UE is a BL UE or UE in enhanced coverage or an NB-IoT UE, a Downlink Channel Quality Report (DCQR) shall be triggered if any of the following events occur:

-	DCQR Command MAC control element is received, in which case the DCQR is referred below to as "Regular DCQR";

-	for BL UE or UE in enhanced coverage, transmission of DCQR in Msg3 is configured by upper layers in mpdcch-CQI-Reporting, in which case DCQR is referred below to as "Msg3 DCQR".

If any type of DCQR has been triggered:

-	start performing DL channel quality measurements according to TS 36.133 [9].

If "Regular DCQR" has been triggered:

-	if an uplink grant has been received on the PDCCH for MAC entity's C-RNTI:

-	instruct the Multiplexing and Assembly procedure to generate a DCQR and AS RAI MAC control element as defined in clause 6.1.3.19;

-	cancel the triggered "Regular DCQR".

If "Msg3 DCQR" has been triggered:

-	if an uplink grant has been received on the PDCCH for MAC entity's RA-RNTI or if the uplink grant is for CB-Msg3-EDT:

-	if the allocated resources can accommodate a DCQR and AS RAI MAC control element plus its subheader as a result of logical channel prioritization:

-	instruct the Multiplexing and Assembly procedure to generate a DCQR and AS RAI MAC control element as defined in clause 6.1.3.19;

-	else if the uplink grant is not for EDT and not for CB-Msg3-EDT:

-	if configured by upper layers in mpdcch-CQI-Reporting, use R and F2 fields in the MAC PDU subheader, to transmit the measurement outcome, as defined in clause 6.2.1;

-	cancel the triggered "Msg3 DCQR".

<Skip>

# 6	Protocol Data Units, formats and parameters

## 6.1	Protocol Data Units

<skip>

### 6.1.7	MAC PDU (CB-Msg4)

A MAC PDU consists of a MAC header, zero or more MAC CB-Msg3-EDT Responses (MAC CMR), zero or more optional MAC SDUs, and optional padding as described in figure 6.1.7-5. Each MAC SDU is associated with the UE identified by the preceding MAC CMR.

The MAC header is of variable size.

A MAC PDU header consists of one or more MAC PDU subheaders; each subheader except for the CB Backoff Indicator subheader corresponding to a MAC CMR, MAC SDU, or padding. If included, the CB Backoff Indicator subheader is only included once and is the first subheader after single-byte or two-byte padding subheader(s) (if present).

The CB Backoff Indicator subheader consists of the four header fields E/T/R/BI (as described in figure 6.1.7-1).

A MAC CMR subheader consists of the seven header fields E/T/R/R/H/TA/C (as described in figure 6.1.7-2).

A MAC PDU subheader corresponding to the MAC SDU consists of the header fields E/T/LCID/(F)/(L). The L field is present in the MAC PDU subheader corresponding to the MAC SDU except for the last subheader in the MAC PDU (as described in figure 6.1.7-3). A MAC PDU subheader corresponding to padding consists of the three header fields E/T/LCID.

For BL UEs and UEs in CE mode A, a MAC CMR consists of the following fields: UE Contention Resolution Identity/(Timing Advance Command)/(HARQ ACK resource)/(C-RNTI) (as described in figure 6.1.7-4a). For NB-IoT UEs, a MAC CMR consists of the following fields: UE Contention Resolution Identity/(R)/(HARQ ACK resource) /(R)/(Timing Advance Command)/(C-RNTI) (as described in figure 6.1.7-4b).

Padding occurs at the end of the MAC PDU, except when single-byte or two-byte padding is required. Padding may have any value and the MAC entity shall ignore it. When padding is performed at the end of the MAC PDU, zero or more padding bytes are allowed.

When single-byte or two-byte padding is required, one or two MAC PDU subheaders corresponding to padding are placed at the beginning of the MAC PDU before any other MAC PDU subheader.

Figure 6.1.7-1: E/T/R/BI MAC subheader

Figure 6.1.7-2: E/T/R/R/H/TA/C MAC subheader

Figure 6.1.7-3:E/T/LCID/(F)/(L) MAC subheader

Figure 6.1.7-4a: MAC CMR for BL UEs and UEs in CE mode A

Figure 6.1.7-4b: MAC CMR for NB-IoT UEs

Figure 6.1.7-5: Example of MAC PDU consisting of a MAC header, MAC CMRs, MAC SDUs and padding

## 6.2	Formats and parameters

<skip>

### 6.2.5	MAC header for CB-Msg4

The MAC header is of variable size and consists of the following fields:

-	E: The Extension field is a flag indicating if more fields are present in the MAC header or not. The E field is set to "1" to indicate at least another subheader follows. The subsequent subheader can be E/T/R/BI MAC subheader, E/T/R/R/H/TA/C MAC subheader or E/T/LCID/(F)/(L) MAC subheader. The E field is set to "0" to indicate that either a MAC CMR, a MAC SDU, or padding starts at the next byte;

-	T: The Type field indicates the type of the MAC subheader. The T field is set to "00" to indicate the presence of a CB Backoff Indicator field in the subheader (BI). The T field is set to "01" to indicate the presence of a CMR in the subheader. The T field is set to "10" to indicate the presence of a Logical Channel ID (LCID) field in the subheader. The value "11" is reserved for future use. The size of the Type field is 2 bits;

-	H: The H field is a flag indicating the presence of the HARQ ACK resource field in the corresponding MAC CMR. For NB-IoT, it also indicates the presence of the 4-bit R fields preceding the HARQ ACK resource field in the same MAC CMR;

-	TA: The TA field is a flag indicating the presence of the Timing Advance Command field in the corresponding MAC CMR. For NB-IoT, it also indicates the presence of the 2-bit R fields preceding the Timing Advance Command field in the same MAC CMR;

-	C: The C field is a flag indicating the presence of the C-RNTI field in the corresponding MAC CMR;

-	R: Reserved bit, set to "0";

-	BI: The CB Backoff Indicator field identifies the overload condition in the cell. The size of the BI field is 4 bits;

-	LCID: The Logical Channel ID field identifies the logical channel instance of the corresponding MAC SDU or padding as described in table 6.2.1-1 for the DL-SCH. There is one LCID field for each MAC SDU, or padding included in the MAC PDU. In addition to that, one or two additional LCID fields are included in the MAC PDU, when single-byte or two-byte padding is required but cannot be achieved by padding at the end of the MAC PDU. The LCID field size is 5 bits;

-	L: The Length field indicates the length of the corresponding MAC SDU in bytes. There is one L field per MAC PDU subheader corresponding to the MAC SDU except for the last subheader. The size of the L field is indicated by the F field;

-	F: The Format field indicates the size of the Length field. There is one F field per MAC PDU subheader except for the last subheader. The size of the F field is 1 bit. If the F field is included; if the size of the MAC SDU is less than 128 bytes, the value of the F field is set to 0, otherwise it is set to 1.

The MAC header and subheaders are octet aligned.

### 6.2.6	MAC payload for CB-Msg4

The MAC CMR is of variable size and consists of the following fields:

-	UE Contention Resolution Identity: This field contains the first 48 bits of the uplink CCCH SDU;

-	R: Reserved bit, set to "0";

-	HARQ ACK resource: This field indicates the resource used to transmit the HARQ ACK for this MAC CMR. For BL UEs and UEs in CE mode A, the size of this field is 2 bits (see HARQ-ACK resource offset in clause 5.3.3.1.13 of TS 36.212 [5]). For NB-IoT UEs, the size of this field is 4 bits (see HARQ-ACK resource in clause 6.4.3.2 of TS 36.212 [5]). For BL UEs and UEs in CE mode A, if this field is not present as indicated in the corresponding MAC CMR subheader and the Timing Advance Command field is present, these bits are set to "00";

-	Timing Advance Command: The Timing Advance Command field indicates the index value TA (0, 1, 2… 63) used to control the amount of timing adjustment that the MAC entity has to apply (see clause 4.2.3 of TS 36.213 [2]). The size of the Timing Advance Command field is 6 bits. For BL UEs and UEs in CE mode A, if this field is not present as indicated in the corresponding MAC CMR subheader and the HARQ ACK resource field is present, these bits are set to "000000";

-	C-RNTI: This field contains the C-RNTI. The length of the field is 16 bits.

The MAC CMR is octet aligned.

The size of the MAC CMR is determined based on its corresponding MAC CMR subheader. It consists of a mandatory 6 octets UE contention resolution identify field followed by one or more optional fields for C-RNTI, Timing Advance Command, and HARQ ACK resource. If the C-RNTI is present, the 2 octets C-RNTI field is included. For BL UEs and UEs in CE mode A, if the HARQ ACK resource, the Timing Advance Command, or both are present, the 1 octet field for Timing Advance Command and HARQ ACK resource is included. For NB-IoT UEs, if the HARQ ACK resource is present, the 1 octet field for HARQ ACK resource is included; if the Timing Advance Command is present, the 1 octet field for Timing Advance Command is included. The MAC CMR size is illustrated in Table 6.2.6-1.

Table 6.2.6-1: MAC CMR size

| H Field | TA Field | C Field | eMTC CMR size (octets) | NB-IoT CMR size (octets) |
| --- | --- | --- | --- | --- |
| 0 | 0 | 0 | 6 | 6 |
| 0 | 0 | 1 | 8 | 8 |
| 0 | 1 | 0 | 7 | 7 |
| 0 | 1 | 1 | 9 | 9 |
| 1 | 0 | 0 | 7 | 7 |
| 1 | 0 | 1 | 9 | 9 |
| 1 | 1 | 0 | 7 | 8 |
| 1 | 1 | 1 | 9 | 10 |

# 7	Variables and constants

## 7.1	RNTI values

RNTI values are presented in Table 7.1-1 and their usage and associated Transport Channels and Logical Channels are presented in Table 7.1-2.

Table 7.1-1: RNTI values.

| Value (hexa-decimal) | RNTI |
| --- | --- |
| 0000 | N/A |
| 0001-0960 0001-1000 (Note 3) | RA-RNTI, C-RNTI, Semi-Persistent Scheduling C-RNTI, Temporary C-RNTI, eIMTA-RNTI, TPC-PUCCH-RNTI, TPC-PUSCH-RNTI, SL-RNTI (see note), G-RNTI, SL-V-RNTI, UL Semi-Persistent Scheduling V-RNTI, SL Semi-Persistent Scheduling V-RNTI, SRS-TPC-RNTI, AUL C-RNTI, and PUR-RNTI |
| 0961-FFF3 1001-FFF3 (Note 3) | C-RNTI, Semi-Persistent Scheduling C-RNTI, eIMTA-RNTI, Temporary C-RNTI, TPC-PUCCH-RNTI, TPC-PUSCH-RNTI, SL-RNTI, G-RNTI, SL-V-RNTI, UL Semi-Persistent Scheduling V-RNTI, SL Semi-Persistent Scheduling V-RNTI, SRS-TPC-RNTI, AUL C-RNTI, CB-RNTI, and PUR-RNTI |
| FFF4-FFF8 | Reserved for future use |
| FFF9 | SI-RNTI |
| FFFA | SC-N-RNTI |
| FFFB | SC-RNTI |
| FFFC | CC-RNTI |
| FFFD | M-RNTI |
| FFFE | P-RNTI |
| FFFF | SI-RNTI |

NOTE 1:	A MAC entity uses the same C-RNTI on all Serving Cells.

NOTE 2:	SI-RNTI value FFFF may be used for MBMS-dedicated carrier. SI-RNTI value FFF9 is only used for MBMS-dedicated carrier.

NOTE 3:	Range applicable for NB-IoT.

Table 7.1-2: RNTI usage.

| RNTI | Usage | Transport Channel | Logical Channel |
| --- | --- | --- | --- |
| P-RNTI | Paging and System Information change notification | PCH | PCCH |
| SI-RNTI | Broadcast of System Information | DL-SCH | BCCH, BR-BCCH |
| M-RNTI | MCCH Information change notification | N/A | N/A |
| RA-RNTI | Random Access Response | DL-SCH | N/A |
| eIMTA-RNTI | eIMTA TDD UL/DL configuration notification | N/A | N/A |
| Temporary C-RNTI | Contention Resolution (when no valid C-RNTI is available) | DL-SCH | CCCH, DCCH |
| Temporary C-RNTI | Msg3 transmission | UL-SCH | CCCH, DCCH, DTCH |
| C-RNTI | Dynamically scheduled unicast transmission | UL-SCH | DCCH, DTCH |
| C-RNTI | Dynamically scheduled unicast transmission | DL-SCH | CCCH, DCCH, DTCH |
| C-RNTI | Triggering of PDCCH ordered random access | N/A | N/A |
| Semi-Persistent Scheduling C-RNTI | Semi-Persistently scheduled unicast transmission (activation, reactivation and retransmission) | DL-SCH, UL-SCH | DCCH, DTCH |
| Semi-Persistent Scheduling C-RNTI | Semi-Persistently scheduled unicast transmission (deactivation) | N/A | N/A |
| TPC-PUCCH-RNTI | Physical layer Uplink power control | N/A | N/A |
| TPC-PUSCH-RNTI | Physical layer Uplink power control | N/A | N/A |
| SL-RNTI | Dynamically scheduled sidelink transmission for sidelink communication | SL-SCH | STCH |
| SC-RNTI | Dynamically scheduled SC-PTM control information | DL-SCH | SC-MCCH |
| G-RNTI | Dynamically scheduled SC-PTM transmission | DL-SCH | SC-MTCH |
| SC-N-RNTI | SC-MCCH Information change notification | N/A | N/A |
| CC-RNTI | Providing common control PDCCH information | N/A | N/A |
| SL-V-RNTI | Dynamically scheduled sidelink transmission for V2X sidelink communication | SL-SCH | STCH |
| UL Semi-Persistent Scheduling V-RNTI | Semi-Persistently scheduled uplink transmission for V2X communication (activation, reactivation and retransmission) | UL-SCH | DCCH, DTCH |
| UL Semi-Persistent Scheduling V-RNTI | Semi-Persistently scheduled uplink transmission for V2X communication (deactivation) | N/A | N/A |
| SL Semi-Persistent Scheduling V-RNTI | Semi-Persistently scheduled sidelink transmission for V2X sidelink communication (activation, reactivation and retransmission) | SL-SCH | STCH |
| SL Semi-Persistent Scheduling V-RNTI | Semi-Persistently scheduled sidelink transmission for V2X sidelink communication (deactivation) | N/A | N/A |
| SRS-TPC-RNTI | SRS and TPC for the PUSCH-less SCells | N/A | N/A |
| AUL C-RNTI | Autonomous Uplink C-RNTI unicast transmission (activation and reactivation) | UL-SCH | DCCH, DTCH |
| AUL C-RNTI | Autonomous Uplink C-RNTI unicast transmission (deactivation) | N/A | N/A |
| PUR-RNTI | Transmission using Preconfigured Uplink Resource | DL-SCH, UL-SCH | CCCH, DCCH, DTCH |
| PUR-RNTI | Transmission using Preconfigured Uplink Resource (L1 ACK or fallback indication) | N/A | N/A |
| CB-RNTI | CB-Msg3 transmission | UL-SCH | CCCH, DTCH |
| CB-RNTI | CB-Msg4 | DL-SCH | CCCH, DCCH,DTCH |

## 7.2	Backoff Parameter values

Backoff Parameter values are presented in Table 7.2-1 except for NB-IoT where Table 7.2-2 shall be used and except for CB-Msg3-EDT procedure.

Table 7.2-1: Backoff Parameter values.

| Index | Backoff Parameter value (ms) |
| --- | --- |
| 0 | 0 |
| 1 | 10 |
| 2 | 20 |
| 3 | 30 |
| 4 | 40 |
| 5 | 60 |
| 6 | 80 |
| 7 | 120 |
| 8 | 160 |
| 9 | 240 |
| 10 | 320 |
| 11 | 480 |
| 12 | 960 |
| 13 | Reserved |
| 14 | Reserved |
| 15 | Reserved |

The reserved values of the backoff parameter if received by the current release version UEs shall be taken as 960 ms.

Table 7.2-2: Backoff Parameter values for NB-IoT.

| Index | Backoff Parameter value (ms) |
| --- | --- |
| 0 | 0 |
| 1 | 256 |
| 2 | 512 |
| 3 | 1024 |
| 4 | 2048 |
| 5 | 4096 |
| 6 | 8192 |
| 7 | 16384 |
| 8 | 32768 |
| 9 | 65536 |
| 10 | 131072 |
| 11 | 262144 |
| 12 | 524288 |
| 13 | Reserved |
| 14 | Reserved |
| 15 | Reserved |

The reserved values of the backoff parameter if received by the current release version NB-IoT UEs shall be taken as 524288 ms.

Backoff parameter value for CB-Msg3-EDT procedure is computed according to the backoff factor in Table 7.2-x.

Table 7.2-x: Backoff Factor for CB-Msg3-EDT.

| Index | Backoff factor |
| --- | --- |
| 0 | 0 |
| 1 | 2 |
| 2 | 4 |
| 3 | 8 |
| 4 | 10 |
| 5 | 12 |
| 6 | 16 |
| 7 | 20 |
| 8 | 24 |
| 9 | 28 |
| 10 | 32 |
| 11 | 36 |
| 12 | 40 |
| 13 | Reserved |
| 14 | Reserved |
| 15 | Reserved |

The backoff parameter values for CB-Msg3-EDT is the backoff factor multiplied by the configured CB-Msg3 transmission window periodicity provided by windowPeriodicity in ms.

The reserved values of the backoff factor if received by the current release version UEs shall be taken as 40.