# 3GPP TSG RAN WG2 Meeting #81                                                         	R2-130069

# St. Julian’s, Malta, Jan. 28 – Feb. 1, 2013

# Title: 	HetNet Mobility Improvement with eICIC

# Source: 	Research In Motion UK Limited

# Agenda Item:	7.1

# Document for: 	Discussion

# Introduction

HetNet mobility enhancement for LTE has been approved as a Work Item for Rel-12 [1]. The WI will focus on the aspects or problems already studied in the Rel-11 HetNet Mobility SI and documented in TR36.839. One objective of the WI is to improve overall HO performance with regard to HO failure and ping-pong in HetNet environments.

During the SI, as captured in TR36.839, based on the simulation study the following conclusion was reached on HetNet mobility performance with eICIC:

- Use of a large CRE bias (e.g. 6dB) with non-ideal ABS pattern coordination among macro cells can lead to mobility performance degradation.

In this contribution, we will discuss possible solutions to improve mobility performance with eICIC for the case of large CRE bias with non-ideal ABS pattern.

# Mobility problems with CRE and possible solutions

The network usually configures an A3 event to trigger the UE to perform periodic measurement reporting in order for the network to make handover decisions. When CRE is enabled at the pico cell, in case of the UE moving from macro to pico cell, the network will hand the UE over to the pico cell early, i.e., when RSRP(neighbour pico) > RSRP(serving macro) + A3_offset – CRE bias. When the UE moves from pico to macro cell, the network will hand the UE over to the macro cell late, i.e., when RSRP(neighbour macro) > RSRP(serving pico) + A3_offset + CRE bias.

Depending on the ABS patterns of the neighbouring macro cells and the pico-cell location, the UE may not always experience sufficient SINR while in the pico range expansion area. This could degrade the mobility performance as shown in [2]. For example, when the ABS patterns of the macro cells are not synchronized in time (i.e. non-ideal ABS pattern coordination), the UE in the pico cell is protected from the overlay macro-cell interference but still affected by interference from other neighbour macro cells. The insufficient SINR in pico range expansion area could increase the pico-to-macro HOF rate due to the missing HO command from the serving pico cell. It also increases the macro-to-pico HOF rate due to the failed RACH to the target pico cell. Therefore solutions are needed to ensure that in low SINR cases the UE that is still in the range expansion area will handover to macro; similarly in macro-to-pico case the UE will not hand over to the pico range expansion area until the pico SINR is high enough.

To improve the mobility performance with CRE, in case of the UE moving from pico to macro:

- We could apply a RSRQ threshold. In addition to an A3 event, the network could configure an A2 event (serving RSRQ becomes worse than threshold) for a pico-cell UE in the range expansion area. In this case the network will be notified via an A2 event if the radio quality from the serving pico cell deteriorates. Then the network could further configure the UE to perform periodic measurement reporting to report the strongest neighbouring cells so that the network could pick the best cell for the UE. To avoid delay due to the network configuring periodic measurement reporting, the UE could include the measurements of the neighbouring cells in the measurement report that has been triggered by an A2 event. Note that in the current specification, the measurement report triggered by an A2 event includes the measurement of the serving cell only.

- Alternatively, the pico-cell eNB may estimate the UE’s signal quality via CQI reports or packet error rate (PER) statistics. For example, if the pico-cell eNB receives a number of consecutive CQI reports with index 0 (CQI index 0 means that the UE cannot support the lowest MCS) or the pico eNB observes a high PER with lowest MCS transmission, it is a good indication that the UE’s signal quality is not good. In this case, the pico-cell eNB may configure the UE to perform periodic measurement reporting to report the strongest neighbouring cells so that the network will pick the best cell for the UE. However this approach may not work well for UEs with bursty traffic as the eNB may not get sufficient CQI reports and PER statistics.

In the case of the UE moving from macro to pico cell, we could also apply a RSRQ threshold to improve the mobility performance. In addition to an A3 event, the network could configure an A4 event (neighbour RSRQ becomes better than threshold) for a macro UE. When the UE moves from a macro to pico cell, an A3 event might be triggered due to CRE. From the A3 measurement report, if the target cell is a pico cell with CRE, the network will not hand the UE into the pico until the A4 event is triggered for the same pico cell.

To maximize traffic offloading to pico cells, in the above approaches instead of RSRQ the UE could trigger A2 and A4 events based on post-processing signal quality such as post-processing SINR at the receiver. The post-processing signal quality reflects the actual QoS at the UE. Triggering A2 and A4 events based on the post-processing signal quality could improve the triggering point setting for the measurement reports. Although the eNB might translate a post-processing SINR threshold to a RSRQ threshold, for some advanced UE receivers, e.g. MMSE-IRC (minimum mean square error – interference rejection combining) or IC (interference cancellation) receiver, the mapping between the post-processing SINR and RSRQ could be complicated.

# Simulation

The mobility performance of CRE with RSRQ thresholding has been evaluated using the large area HetNet simulation. As CRE causes more degradation to pico-to-macro HO than to macro-to-pico HO, in the simulation we applied a RSRQ threshold for pico-to-macro HO with A2 event. That is, a pico UE will be handed out to the macro if the pico RSRQ is low than the threshold. We expect such a thresholding mechanism to improve the pico-to-macro HO performance.

We follow the simulation assumptions in TR36.839 and TR 36.814. Other simulation assumptions are described in Table 1.

Table 1 Simulation assumptions

| Items | Descriptions |
| --- | --- |
| Configuration parameter set | Set 3 in TR36.839 with UE speed 30km/h |
| Pico placement | 1 pico per macro cell, the pico is placed at boresight 1/3 ISD (i.e., 2/3 of the cell radius) |
| CRE bias | 0dB, 6dB |
| eICIC | ABS is configued at the overlay macro cell only. When a UE is connected to a pico, it is assumed that the UE does not experience any interference from the overlay macro cell. However the UE still sees the interference from other macro cells. In other words, we assume that the ABS patterns of the macro cells are not synchronized. |
| A2 event | TTT 160ms, RSRQ threshold -19.5dB and -17dB |
| RSRQ | L3 filter K=1; L1 to L3 period 200ms; The same measurement error modelling as RSRP, i.e., a normal distribution with deviation 1.216dB |

The triggering criteria of A3 event for differnt types of handovers with CRE are described as follows:

- Macro-to-macro: RSRP(neighbour macro) > RSRP(serving macro) + A3_offset

- Macro-to-pico: RSRP(neighbour pico) > RSRP(serving macro) + A3_offset – CRE bias

- Pico-to-macro: RSRP(neighbour macro) > RSRP(serving pico) + A3_offset + CRE bias

In the simulation, we considered the performance of the following four cases:

Case 1: without CRE (i.e., CRE 0dB)

Case 2: CRE with bias value 6dB, ABS is configued at the overlay macro cell only

Case 3: CRE with bias value 6dB, ABS is configued at the overlay macro cell only, RSRQ threshold -19.5dB for pico-to-macro HO

Case 4: CRE with bias value 6dB, ABS is configued at the overlay macro cell only, RSRQ threhold -17dB for pico-to-macro HO

While Table 2 to Table 4 in Appendix capture the complete set of simulation results, Figure 2 shows the improvement of the P2M State 2 HOF rates using the RSRQ threshold. The P2M State 2 HO failure is caused by the missing HO command due to the low SINR in pico cell. As expected, with CRE bias 6dB, the P2M State 2 HOF rate is reduced from 21% without RSRQ threshold to 12.5% with RSRQ threshold -19.5dB. When we further increase the RSRQ threshold to -17dB, the P2M State 2 HOF rate is further down to 7% but at the expense of more frequent ping-pongs as shown in Figure 3.

Figure 2 HOF rate of pico-to-macro with RSRQ threshold

Figure 3 Short ToS rate

In Figure 4 we show the CDF curves of the time-of-stay in pico cells. Although a high RSRQ threshold is preferred to reduce the P2M HOF rate, it may limit the traffic offloading to pico cells as the UE may be handed out to macro early. From Figure 4 we observe that for a high RSRQ threshold such as -17dB the traffic offloading is impacted as the ToS in pico is much reduced compared to the case of no RSRQ thresholding. However for the RSRQ threshold -19.5dB the pico traffic offloading is not impacted much while the P2M HOF rate is significantly improved. Therefore for the simulated scenarios, the RSRQ threshold of -19.5dB is appropriate when we consider the overall performance of HOF rate, ping-pong rate and pico traffic offloading.

Figure 4 Time-of-stay in pico cells

# Conclusion

In this paper, we discussed mechanisms to improve HetNet mobility performance with CRE. One possible solution is that, in addition to an A3 event, the network configures an A2 or A4 event triggered by either the existing RSRQ or a post-processing signal quality.

# Reference

RP-122007, New WI proposal: Hetnet mobility enhancement for LTE

R2-121279, HetNet mobility performance with cell range expansion, #77b, RIM

# Appendix

Table 2 Handover performance

|  | Handover state | Handover metrics | macro-pico | pico-macro | macro-macro | pico-pico | Overall |
| --- | --- | --- | --- | --- | --- | --- | --- |
| CRE 0dB | 2 | HOFs/UE/s | 0.000055 | 0.000571 | 0.003556 | 0.000000 | 0.004183 |
|  |  | HO failure rate [%] | 0.508475 | 5.073650 | 4.193829 | 0.000000 | 3.909749 |
|  | 3 | HOFs/UE/s | 0.000018 | 0.000018 | 0.000129 | 0.000000 | 0.000166 |
|  |  | HO failure rate [%] | 0.169492 | 0.163666 | 0.152108 | 0.000000 | 0.155012 |
|  | Total | Successful HOs/UE/s | 0.010798 | 0.010669 | 0.081114 | 0.000055 | 0.102636 |
|  |  | HOFs/UE/s | 0.000074 | 0.000590 | 0.003685 | 0.000000 | 0.004349 |
|  |  | HO failure rate [%] | 0.677966 | 5.237316 | 4.345937 | 0.000000 | 4.064761 |
| CRE 6dB | 2 | HOFs/UE/s | 0.000152 | 0.005690 | 0.002743 | 0.000229 | 0.008814 |
|  |  | HO failure rate [%] | 0.570885 | 20.645161 | 3.680982 | 52.941176 | 6.821309 |
|  | 3 | HOFs/UE/s | 0.000584 | 0.000025 | 0.000127 | 0.000025 | 0.000762 |
|  |  | HO failure rate [%] | 2.188392 | 0.092166 | 0.170416 | 5.882353 | 0.589739 |
|  | Total | Successful HOs/UE/s | 0.025960 | 0.021845 | 0.071658 | 0.000178 | 0.119641 |
|  |  | HOFs/UE/s | 0.000737 | 0.005715 | 0.002870 | 0.000254 | 0.009576 |
|  |  | HO failure rate [%] | 2.759277 | 20.737327 | 3.851397 | 58.823529 | 7.411048 |
| CRE 6dB, RSRQ threshold -19.5dB | 2 | HOFs/UE/s | 0.000215 | 0.003673 | 0.002677 | 0.000039 | 0.006604 |
|  |  | HO failure rate [%] | 0.759144 | 12.625923 | 3.556594 | 20.000000 | 4.970588 |
|  | 3 | HOFs/UE/s | 0.000430 | 0.000000 | 0.000039 | 0.000000 | 0.000469 |
|  |  | HO failure rate [%] | 1.518288 | 0.000000 | 0.051921 | 0.000000 | 0.352941 |
|  | Overall | Successful HOs/UE/s | 0.027665 | 0.025418 | 0.072542 | 0.000156 | 0.125782 |
|  |  | HOFs/UE/s | 0.000645 | 0.003673 | 0.002716 | 0.000039 | 0.007073 |
|  |  | HO failure rate [%] | 2.277433 | 12.625923 | 3.608515 | 20.000000 | 5.323529 |
| CRE 6dB, RSRQ threshold -17dB | 2 | HOFs/UE/s | 0.000238 | 0.002141 | 0.002654 | 0.000018 | 0.005051 |
|  |  | HO failure rate [%] | 0.769231 | 6.956005 | 3.481393 | 33.333333 | 3.660477 |
|  | 3 | HOFs/UE/s | 0.000659 | 0.000037 | 0.000073 | 0.000000 | 0.000769 |
|  |  | HO failure rate [%] | 2.130178 | 0.118906 | 0.096038 | 0.000000 | 0.557029 |
|  | Total | Successful HOs/UE/s | 0.030034 | 0.028607 | 0.073502 | 0.000037 | 0.132180 |
|  |  | HOFs/UE/s | 0.000897 | 0.002178 | 0.002727 | 0.000018 | 0.005820 |
|  |  | HO failure rate [%] | 2.899408 | 7.074911 | 3.577431 | 33.333333 | 4.217507 |

Table 3 RLF performance

|  | Average number of RLFs/UE/second |  |  |  |
| --- | --- | --- | --- | --- |
|  | State 1 | State 2_Normal | State 2_HOF | Overall |
| CRE 0dB | 0.000000 | 0.000000 | 0.002967 | 0.002967 |
| CRE 6dB | 0.000000 | 0.000025 | 0.006579 | 0.006604 |
| CRE 6dB, RSRQ threshold -19.5dB | 0.000000 | 0.000000 | 0.004357 | 0.004357 |
| CRE 6dB, RSRQ threshold -17dB | 0.000000 | 0.000000 | 0.003386 | 0.003386 |

Table 4 Short time-of-stay (ToS) performance

|  | ToS metrics | Overall |
| --- | --- | --- |
| CRE 0dB | Short ToS/UE/s | 0.014631 |
|  | Short ToS rate [%] | 14.254937 |
| CRE 6dB | Short ToS/UE/s | 0.019280 |
|  | Short ToS rate [%] | 16.114650 |
| CRE 6dB, RSRQ threshold -19.5dB | Short ToS/UE/s | 0.022178 |
|  | Short ToS rate [%] | 17.952084 |
| CRE 6dB, RSRQ threshold -17dB | Short ToS/UE/s | 0.029723 |
|  | Short ToS rate [%] | 22.486846 |

For example, as shown in [2], comparing the case of no CRE to the case of CRE bias 6dB, the pico-to-macro HOF rate increases from 5% to 21% while the macro-to-pico HOF rate increases from 0.7% to 2.7%.

The handover failure rates in this column should be ignored as they are not significant statistical results due to the extremely few samples.