3GPP TSG-RAN WG2 #89bis	Tdoc R2-151550

Bratislava, Slovakia, 20–24 Apr 2015

Agenda Item:	7.8

Source:	Ericsson

Title:	Packet delay and drop rate for MDT QoS measurements

Document for:	Discussion, Decision

# Introduction

RAN plenary has agreed a new Study Item evaluating MDT procedures and capabilities to support QoS and its limiting factors for MMTEL voice and video traffic [1]. Within this framework, this contribution discusses packet delay and drop rate aspects for the MDT study.

# Background

Voice is a fundamental service to consider in any mobile Network. With the introduction of Voice over LTE (VoLTE) services, ITU-R and 3GPP technical requirements related to QoS for conversional services (MMTEL form Voice and Video) need to be met with the flat all-IP Evolved Packet Core network (EPC). As LTE only support Packet- Switched services, it is especially important for a successful service delivery that the QoS performance in terms of end-to-end delay, jitter and packet loss rate is fulfilled.

This eventually means that an operator that can provide one way VoLTE end-to-end packet delay of less than e.g. 150 ms with a minimum of 98% packet success rate as recommended in [2], would be able ensure that criteria for conversational services are fulfilled.

## Existing MDT and OAM functionality

In the current MDT functionality of Rel-12, we today have several means of collecting measurement reports and logs form UE; that in parallel with traces and logs at the NW side (eNB) may assist in operators service performance evaluation.

As a summary we have today for MDT defined [3]:

- Reporting triggers and criteria utilized for RRM (UE RRC connected), Immediate MDT:

  - M1: RSRP and RSRQ measurement by UE (3GPP TS 36.214).

  - M2: Power Headroom measurement by UE (3GPP TS 36.213).

  - M3: Received Interference Power measurement by eNB, (3GPP TS 36.214).

  - M4: Data Volume measurements separately for DL and UL, per QCI per UE, by eNB [4].

  - M5: Scheduled IP Throughput for MDT measurement separately for DL and UL, per RAB per UE and per UE for the DL, per UE for the UL, by eNB [4]

    - QCI values of the RABs that have contributed to a measurement value are logged with the measurement values.

- Logged MDT, idle mode measurements in the UE

  - Pilot strength measurements logging (RSRP, RSRQ)

    - Neighboring cells, if available, up to: 6 for intra-frequency, 3 for inter-frequency neighboring cells

  - MBSFN measurements (MCH/MCCH BLER)

- Location information is based on available location information in the UE

In addition to the eNB specific Data Volume measurement and the Scheduled IP throughput measurements above for MDT, 4.1.4 in [4] also defines L2 measurements for Packet Delay in the DL per QCI and in 4.1.5 Data Loss in the DL, UL and Uu per QCI. These additional measurements are defined for OAM purposes. [Annex, sect. 7]

Current MDT and OAM performance observability together include L2 measurements for DL packet delay and packet loss rate (UL,DL and Uu) per QCI.

# Discussion

As can be seen, there are currently a number of measurements already defined for QoS type of assessment in the NW. Considering MMTel Voice and video specifically, it can be assume that at least for DL, packet delay and packet loss rate seems to be observable. In any instance, shortcomings of the already defined eNB measurements should in many cases be possible to address by implementation.

The difference in relative arrival time between packets, or jitter, is an important aspect of conversational services like Voice. UL Scheduling Request (SR) based transmissions; including the aspects on varying delay between packets caused by HARQ, may introduce intermittent queuing delays due to coverage and load. As low jitter variation in general is beneficial for a stable system with steady packet stream, small delay variation of received packets is preferred. This reduces reordering delays and eases detection of lost packets. High jitter and packet loss on the other hand may result in poor service quality. It is hence of interest to be able to track this aspect in the NW and be able to determine what e.g. parameters need to be tuned.

It can be discussed what upper bound delay variation (jitter) is acceptable for various service types and at which point packets shall be discarded. If for example a specific upper bound jitter value is targeted in the NW, one could foresee that it comes with complexity in handling variations caused by different back haul delays, UE implementation and scheduling strategy. In addition, UL packet delay (variation) is usually not proportional to the DL delay and may be more influenced by coverage problems and scheduling delay. It is thus of interest in studying additional UE measurements and reporting that may result in better observability, and in the end improved end user performance in the NW.

In view of the above, we believe the SI for MDT should study the benefit and feasibility of defining UE measurements and reporting for jitter and packet discard due to exceeding queuing delay.

In the study one could foresee different measurements provided by the UE to the NW, for example UE reporting per QCI:

- A count of packets exceeding delay budget (c.f packet discard rate)

- providing a histogram of packet delay

- stamp each PDCP PDU with delay info

- …

or any optimized variations or combination of those.

Naturally the defined measurements and definitions used at the eNB side should be considered when evaluating these.

Proposal: Study the benefit and feasibility of defining UE (UL) measurements and reporting for jitter and packet discard per QCI due to exceeding queuing delay.

Complexity

The possible inclusion of the above measurements will naturally impact RRC connected mode UE measurements. The Study should thus also include a comprehensive evaluation of the complexity as well as the impact of existing MDT/OAM functionality.

For tracking single UEs across specific areas, there may also be architectural and signalling aspects that need input from RAN3 and/or SA5.

# Conclusion

This contribution discusses packet delay and drop rate aspects for the MDT study and in conclusion propose to:

Proposal: Study the benefit and feasibility of defining UE (UL) measurements and reporting for jitter and packet discard per QCI due to exceeding queuing delay.

# References

RP-150472, New Study Item Proposal: Further Enhancements of Minimization of Drive Tests for E-UTRAN, RAN#67 March 2015

ITU-T Recommendation G.114, “One way transmission time”, February 2003.

3GPP TS 37.320, Radio measurement collection for Minimization of Drive Tests (MDT); Overall description;

3GPP TS 36.314, Layer 2 – Measurements V12.0.0 (2014-09)

# Annex

# Excerpt from [4]

### 4.1.4	Packet Delay

#### 4.1.4.1	Packet Delay in the DL per QCI

The objective of this measurement is to measure L2 Packet Delay for OAM performance observability.

If there is one or more RNs served in a cell, for that cell the eNB performs each measurement separately for packets transmitted between the eNB and UEs and for packets transmitted between the eNB and RNs.

Protocol Layer: MAC, RLC, PDCP

| Definition | Packet Delay in the DL per QCI. This measurement refers to packet delay for DRBs. For arrival of packets the reference point is PDCP upper SAP. For successful reception the reference point is MAC lower SAP. The measurement is done separately per QCI. Detailed Definition: ,where explanations can be found in the table 4.1.4.1-1 below. |
| --- | --- |

Table 4.1.4.1-1

|  | Packet Delay in the DL per QCI, averaged during time period . Unit: Integer ms. |
| --- | --- |
|  | The point in time when PDCP SDU arrives. |
|  | The point in time when the last piece of PDCP SDU i was received by the UE according to received HARQ feedback information. |
|  | A PDCP SDU that arrives at the PDCP upper SAP during time period . PDCP SDU for which HARQ acknowledgement is not received for all parts shall not be included in the calculation. |
|  | Total number of PDCP SDUs . |
|  | Time Period during which the measurement is performed |

### 4.1.5	Data Loss

#### 4.1.5.1	Packet Discard Rate in the DL per QCI

The objective of this measurement is to measure packets that are dropped due to congestion, traffic management etc, for OAM performance observability.

For an eNB serving one or more RNs, packets transmitted between the eNB and RNs are excluded, i.e., only packets transmitted between the eNB and UEs are counted.

Protocol Layer: MAC, RLC, PDCP

| Definition | Packet Discard Rate in the DL per QCI. This measurement refers to discard for DRBs. One packet corresponds to one PDCP SDU. The reference point is PDCP upper SAP. The measurement is done separately per QCI. Detailed Definition: ,where explanations can be found in the table 4.1.5.1-1 below. |
| --- | --- |

NOTE:	Packet loss is expected to be small or very small The statistical accuracy of an individual discard rate measurement result is dependent on how many packets has been received, and thus the time for the measurement.

Table 4.1.5.1-1

|  | Packet Discard Rate in the DL per QCI, averaged during time period . Unit: number of discarded packets per received packets * 106, Integer. |
| --- | --- |
|  | Number of DL packets, for which no part has been transmitted over the air, of a data radio bearer with QCI = , that are discarded during time period in the PDCP, RLC or MAC layers due to reasons other than hand-over. |
|  | Number of DL packets of bearer with QCI = that has entered PDCP upper SAP during time period (NOTE). |
|  | Time Period during which the measurement is performed, Unit: minutes (NOTE). |

#### 4.1.5.2	Packet Uu Loss Rate in the DL per QCI

The objective of this measurement is to measure packets that are lost at Uu transmission, for OAM performance observability.

.If there is one or more RNs served in a cell, for that cell the eNB performs each measurement separately for packets transmitted between the eNB and UE and for packets transmitted between the eNB and RNs.

Protocol Layer: MAC, RLC, PDCP

| Definition | Packet Uu Loss Rate in the DL per QCI. This measurement refers to packet loss for DRBs. One packet corresponds to one PDCP SDU. The measurement is done separately per QCI. Detailed Definition: , where explanations can be found in the table 4.1.5.2-1 below. |
| --- | --- |

NOTE:	Packet loss is expected to be upper bounded by the PELR of the QCI which takes values between 10-6 and 10-2. The statistical accuracy of an individual packet loss rate measurement result is dependent on how many packets have been received, and thus the time for the measurement.

Table 4.1.5.2-1

|  | Packet Uu Loss Rate in the DL per QCI. Unit: number of lost packets per transmitted packets * 106, Integer. |
| --- | --- |
|  | Number of DL packets, of a data radio bearer with QCI = , for which at least a part has been transmitted over the air but not positively acknowledged, and it was decided during time period that no more transmission attempts will be done. If transmission of a packet might continue in another cell, it shall not be included in this count. |
|  | Number of DL packets, of a data radio bearer with QCI = , which has been transmitted over the air and positively acknowledged during time period . |
|  | Time Period during which the measurement is performed, Unit: minutes (NOTE). |

#### 4.1.5.3	Packet Loss Rate in the UL per QCI

The objective of this measurement is to measure packets that are lost in the UL, for OAM performance observability.

If there is one or more RNs served in a cell, for that cell the eNB performs each measurement separately for packets transmitted between the eNB and UE and for packets transmitted between the eNB and RNs.

Protocol Layer: PDCP

| Definition | Packet Loss Rate in the UL per QCI. This measurement refers to packet loss for DRBs. One packet corresponds to one PDCP SDU. Reference point is the PDCP upper SAP. The measurement is done separately per QCI. Detailed Definition: , where explanations can be found in the table 4.1.5.3-1 below. |
| --- | --- |

NOTE:	Packet loss is expected to be upper bounded by the PELR of the QCI which takes values between 10-6 and 10-2. The statistical accuracy of an individual packet loss rate measurement result is dependent on how many packets have been received, and thus the time for the measurement.

Table 4.1.5.3-1

|  | Packet Loss Rate in the UL per QCI. Unit: number of lost packets per transmitted packets * 106, Integer. |
| --- | --- |
|  | Number of missing UL PDCP sequence numbers, representing packets that are not delivered to higher layers, of a data radio bearer with QCI = during time period . If transmission of a packet might continue in another cell, it shall not be included in this count. |
|  | Total number of UL PDCP sequence numbers (also including missing sequence numbers) of a bearer with QCI = , starting from the sequence number of the first packet delivered by PDCP upper SAP to higher layers until the sequence number of the last packet during time period . |
|  | Time Period during which the measurement is performed, Unit: minutes (NOTE). |