3GPP TSG-RAN WG2 Meeting #91	             R2-153069

Beijing, P. R. China, August 24-28, 2015

Source:   	Apple

Title:     	UTRA/E-UTRA Protocol aspects of Indoor positioning enhancements using hybrid Wi-Fi/BT and barometric sensors

Agenda item:   	5.2

Document for:		Discussions and Approval

- Introduction

A Study Item on Indoor Positioning Enhancements for UTRA and E-UTRA is ongoing, with work that began in October 2014.  At RAN1 #80bis meeting, it was agreed to consider Wi-Fi/Bluetooth and Barometric Sensor as enhancement technologies for indoor positioning.

At RAN2 #89, the higher layer implications have been discussed and an agreement has been reached on defining the operating modes of WiFi/BT and barometric sensors positioning. The protocol aspects of E-UTRA, in the other hand, is still pending. Three options are identified and are expected to be discussed during RAN2 #91 meeting: LPP, LPPe and LPPa+RRC.

In this contribution, we start by providing additional details on UTRA RRC[2] signaling requirements to support WiFi-Bluetooth and Barometric sensor indoor positioning. We follow this by an analysis of E-UTRA protocol options already identified.

Based on analysis results, a text proposal, covering both UTRA RRC/PCAP and E-UTRA LPP signaling, is provided for TR37.857 [1]

- UTRA Protocols Extension

In order to enable Wi-Fi/Bluetooth and barometric pressure sensors positioning in UMTS control plane location, following protocols extension should be considered:

- RRC protocol needs to be extended to support any of the three Wi-Fi/BT/Barometric modes: “Standalone/Autonomous”, “UE-assisted/SMLC based” and “UE-based/SMLC assisted”.

- In addition to RRC; PCAP [3] protocol should be updated to support “UE-assisted/SMLC based” and “UE-based/SMLC assisted” modes. PCAP assistance and reporting IEs between RNC and SAS could be extended.

- RANAP interface between Core Network and RNC, may be extended to support dispatchable civic address, but this is a generic requirements, not related directly to WiFi/BT/Barometric.

The following table provides more details on UTRA specifications impact

Table 2.1: WiFi/BT/Barometric UTRA specification impacts

| UTRA specifications | Section | Description |
| --- | --- | --- |
| TS 25.305 (RRC) | 2. References | Addition of references for WiFi/BT/barometric |
|  | 3.2 Definitions & Abbreviations | Addition of abbreviations for WiFi/BT/barometric |
|  | 4.3 standards UE Positioning Methods and “new section “14” | General modes and procedures description |
| TS 25.331(RRC) | 2. References | Addition of references for WiFi/BT/barometric |
|  | 3.2 Abbreviations | Addition of abbreviations for WiFi/BT/barometric |
|  | 9-11 Positioning Method IEs | Capabilities, assistance and reporting extensions |
| TS 25.453 (PCAP) | 2. References, 3.2 Abbreviation | Addition of references/Abbreviation for WiFi/BT/barometric |
|  | 9 Positioning Methods IE | Information Elements for WiFi/BT/Barometric assistance/report IEs |

Observation 1: WiFi/BT/Barometric “Autonomous” positioning mode requires RRC protocol extension.

Observation 2: “UE-assisted/SMLC based” and “UE-based/SMLC assisted” modes require RRC and PCAP protocols extensions.

- E-UTRA Protocols Analysis

Three protocols options has been identified in LTE in order to support WiFi/BT and barometric sensors: LPP [4], LPPe/LPP [5] and LPPa+RRC.

## 3.1   LPPa + RRC

This approach proposed by some companies [6] considers extending RRC protocol to support indoor positioning control and measurements reports; LPPa extension could be used as interface between the eNodeB and e-SMLC server.

Motivation:

- The main motivation behind this approach is to allow eNodeB to exploit the UE WLAN/BT/Barometric measurements for other features such as WLAN/LTE resources aggregations or offloading[6].

Disadvantages:

- This solution introduces a new framework for indoor positioning in LTE, which is inconvenient to UEs and NW architecture. In the field, there is no real separation between indoor and outdoor positioning features; the UE should be able to use the same protocol for both indoor and outdoor positioning configuration and reporting.

- RRC protocol has been used in UMTS for positioning. In Release 9, we decided to use LPP/LPPe for LTE to avoid the complexity of supporting and maintaining positioning native procedures in RRC/PCAP protocols.

- Privacy problem:  the UE could report non-Operator’s access points measurements information for E911indoor positioning. This information should be sent only and directly to the national database and used only, as requested by FCC, for the purpose of E911 emergency positioning.

- No benefits for the UE autonomous mode and UE based mode: The synergy between indoor positioning feature and WLA/LTE resources aggregation features does not apply to “UE-based/SMLC assisted” and “autonomous” mode, where the UE report only position coordinates.

## 3.2  LPPe/LPP

In this approach, it is proposed to use OMA LPPe specification via EPDU.

Motivations:

- WiFi/BT and barometric sensors are already supported for indoor positioning.

Disadvantages

- Today, there is a limited support of LPPe protocol in the field.

- OMA LPPe specification is a very large specification with a lack of modularity. 3GPP could benefit from specifying a more precise and narrowed LPP extension that meets the new of FCC requirements.

### 3.3  LPP

In this approach, it is proposed to extend LPP protocol to support WiFi/BT and Barometric positioning.

Motivations:

- Under the control of 3GPP and could be specified and maintained to meet FCC requirements.

- Widely supported by operators today and by legacy devices.

- Legacy UEs could be updated easily over the air, as they already support LPP.

- Single framework for positioning in LTE that could be used for both indoor and outdoor.

- Meet the privacy requirements, by ensuring to deliver WiFi/BT information to the “national database only” for “UE assisted, SMLC based” mode.

Disadvantages

- LPP specifications extension.

Observation 3: Extending LPP protocol or using LPPe/LPP are both suitable options to support Wi-Fi/BT/Barometric sensor indoor positioning and meet FCC requirements in timely manner.

### 3.4  LPP Protocol Extension

The following table summarizes UTRA LPP protocol extensions to support WiFi/BT/Barometric

Table 3.4.1: WiFI/BT/Barometric LPP specifications impact

| E-UTRA Specifications | Section | Description |
| --- | --- | --- |
| TS 36.305 | 2. References | Addition of references for WiFi/BT/barometric |
|  | 3.2 Abbreviations | Addition of abbreviations for WiFi/BT/barometric |
|  | 8. Positioning methods | General capabilities, modes and procedures description |
| TS 36.355 (LPP) | 2. References | Addition of references for WiFi/BT/barometric |
|  | 3.2 Abbreviations | Addition of abbreviations for WiFi/BT/barometric |
|  | 6.3 Message Body IEs | Capabilities, assistance and Location IEs |
|  | 6.5 Positioning Method IEs | Additional IEs specific to WiFi/BT/barometric methods |

- Conclusion

In this contribution we discussed the protocols requirements to support enhanced indoor positioning using WiFi/BT and Barometric sensors in UTRA and E-UTRA. Benefits and issues related to E-UTRA protocols extension are discussed. Based on the discussion we have the following proposals:

Proposals:

- WiFi/BT/Barometric “Autonomous” positioning mode requires RRC protocol extension; “UE-assisted/SMLC based” and “UE-based/SMLC assisted” modes require both RRC and PCAP protocols extensions.

- Both “LPPe” and “Extending LPP” are suitable options to support Wi-Fi/BT/Barometric sensor indoor positioning and meet FCC requirements in timely manner.

- Include the enclosed text proposal in TR 37.857.

- References

- 3GPP TR 37.857, "Study on Indoor Positioning Enhancements for UTRA and LTE", V0.4.0.

- 3GPP TS 25.331, "Radio Resource Control (RRC); Protocol Specification".

- 3GPP TS 25.453, "UTRAN Iupc interface Positioning Calculation Application Part (PCAP) signalling".

- 3GPP TS 36.355, "LTE Positioning Protocol (LPP)".

- OMA-TS-LPPe-V1_1-20150414-C, "OMA LPP Extension (LPPe) v1.1", 14 Apr 2015.

- R1-150267, “Utilizing WiFi/Bluetooth measurement for indoor positioning”, Ericsson

# Annex:  Text Proposal for TR 37.857 V0.4.0

### 7.2.2	Wi-Fi/Bluetooth based positioning

Existing Wi-Fi and/or Bluetooth technologies can be used as potential indoor positioning enhancement techniques.

WiFi-based positioning systems are widely used for commercial Location Based Services (LBS). Various smartphones and operating system vendors provide the ability to provide position estimates using WiFi transceivers. This is usually accomplished by collecting RSSI and other information from the UE’s WiFi receiver, and applying a location determination algorithm using databases of the estimated positions or coverage areas of WiFi access points.

In addition, Bluetooth (BT) Low Energy (LE) beacons are realized as a potential technology to provide location information and relevant contextual interactions to users. Bluetooth beacons are transmitters that use Bluetooth Low Energy to broadcast signals that can be heard by compatible smartphones and other devices.  When the device is in a beacon’s proximity, the device may obtain the beacon ID and the device location may be obtained from a database query.

RAN2 identified the following WiFi/Bluetooth based positioning options which may operate in the following modes:

- Standalone/autonomous:
The UE performs WiFi/Bluetooth position measurements and location computation without network assistance.

- UE-assisted, E-SMLC based:
The UE provides WiFi/Bluetooth position measurements to the network for computation of a location estimate by the network. The network may provide assistance data to the UE (e.g. AP IDs, supported channels, coverage areas) to help or enable position measurements.

- UE-based, E-SMLC assisted:
The UE performs WiFi/Bluetooth position measurements and computation of a location estimate. The assistance data useful or essential to one or both of these functions (e.g. AP locations) may be provided to the UE by the network.

To enable WiFi/Bluetooth based positioning in UMTS control plane location, RRC [28] and PCAP [29] protocols need to be updated (e.g., UE capabilities, assistance data delivery, position measurement reporting).

For specification impacts, see tables below for UTRA impacted specifications

Table 7.2.2-1 Wi-Fi/BT UTRA specifications impacts

| UTRA specifications | Section | Description |
| --- | --- | --- |
| TS 25.305 (RRC) | 2. References | Addition of references for WiFi/BT |
|  | 3.2 Definitions & Abbreviations | Addition of abbreviations for WiFi/BT |
|  | 4.3 standards UE Positioning Methods and “new section “14” | General modes and procedures description |
| TS 25.331(RRC) | 2. References | Addition of references for WiFi/BT |
|  | 3.2 Abbreviations | Addition of abbreviations for WiFi/BT. |
|  | 9-11 Positioning Method IEs | Capabilities, assistance and reporting extensions |
| TS 25.453 (PCAP) | 2. References, 3.2 Abbreviation | Addition of references/Abbreviation for WiFi/BT |
|  | 9 Positioning Methods IE | Information Elements for WiFi/BT assistance/report IEs |

To enable WiFi/Bluetooth based positioning in LTE control plane location, either LPPe could be used or LPP protocol could be updated.

WiFi and Bluetooth measurements and assistance data may be included in LPP[19], together with corresponding UE capability updates. The method used to compute the location estimate may also need to be included in the UE measurement report (e.g., for standalone/autonomous mode).

For specification impacts, see tables below for E-UTRA LPP impacted specifications

Table 7.2.2-2 Wi-Fi/BT E-UTRA specification impacts

| E-UTRA Specifications | Section | Description |
| --- | --- | --- |
| TS 36.305 | 2. References | Addition of references for WiFi/BT/barometric |
|  | 3.2 Abbreviations | Addition of abbreviations for WiFi/BT/barometric |
|  | 8. Positioning methods | General capabilities, modes and procedures description |
| TS 36.355 (LPP) | 2. References | Addition of references for WiFi/BT/barometric |
|  | 3.2 Abbreviations | Addition of abbreviations for WiFi/BT/barometric |
|  | 6.3 Message Body IEs | Capabilities, assistance and Location IEs |
|  | 6.5 Positioning Method IEs | Additional IEs specific to WiFi/BT/barometric methods |

### -

### 7.2.3	Barometric pressure sensor positioning

Barometric sensor information is relevant for this study item, but will not impact RAN1 specifications.

The use of barometric sensors for identifying height information is motivated by the fundamental property that atmospheric pressure drops with an increase in altitude. Barometric sensors have started appearing on smartphones and other portable devices. As deployment rates improve, barometers represent a potential solution for accurate vertical location for e.g., E911.

RAN2 identified the following Barometric pressure sensor positioning options which may operate in the following modes:

- Standalone/autonomous:
The UE performs barometric pressure sensor measurements and altitude computation without network assistance.

- UE-assisted, E-SMLC based:
The UE provides barometric pressure sensor measurements, e.g air pressure, to the network for computation of altitude information by the network.

- UE-based, E-SMLC assisted:
The UE performs barometric pressure sensor measurements and uses the measurements (possibly together with other location measurements) for the computation of altitude information. The assistance data required for this (e.g., reference atmospheric pressure) may be provided to the UE by the network.

To enable barometric pressure sensor positioning in UMTS control plane location, RRC[28] and PCAP[29] protocols need to be updated (e.g., UE capabilities, assistance data delivery, sensor measurement reporting).

For specification impacts, see tables below for UTRA impacted specifications:

Table 7.2.3-1 Wi-Fi/BT UTRA specifications impacts

| UTRA specifications | Section | Description |
| --- | --- | --- |
| TS 25.305 (RRC) | 2. References | Addition of references for barometric sensor |
|  | 3.2 Definitions & Abbreviations | Addition of abbreviations for barometric sensor |
|  | 4.3 standards UE Positioning Methods and “new section “14” | General modes and procedures description |
| TS 25.331(RRC) | 2. References | Addition of references for barometric sensor |
|  | 3.2 Abbreviations | Addition of abbreviations for barometric sensor. |
|  | 9-11 Positioning Method IEs | Capabilities, assistance and reporting extensions |
| TS 25.453 (PCAP) | 2. References, 3.2 Abbreviation | Addition of references/Abbreviation for barometric sensor |
|  | 9 Positioning Methods IE | Information Elements for barometric sensor assistance/report IEs |

To enable barometric pressure sensor positioning in LTE control plane location  either LPPe could be used or LPP protocol could be updated.

Barometric pressure sensor positioning (e.g., sensor measurements and assistance data) may be included in LPP[19], together with corresponding UE capability updates. The method used to compute vertical location may also need to be included in the UE measurement report (e.g., for standalone/autonomous mode).

For specification impacts, see tables below for E-UTRA LPP impacted specifications

Table 7.2.3-2 Wi-Fi/BT UTRA specifications impacts

| E-UTRA Specifications | Section | Description |
| --- | --- | --- |
| TS 36.305 | 2. References | Addition of references for barometric sensor |
|  | 3.2 Abbreviations | Addition of abbreviations barometric sensor |
|  | 8. Positioning methods | General capabilities, modes and procedures description |
| TS 36.355 (LPP) | 2. References | Addition of references for barometric sensor |
|  | 3.2 Abbreviations | Addition of abbreviations for barometric sensor |
|  | 6.3 Message Body IEs | Capabilities, assistance and Location IEs |
|  | 6.5 Positioning Method IEs | Additional IEs specific to barometric sensor methods |

Assistance data can be used to further improve the positioning accuracy of the barometric pressure sensor, including information regarding the relationship between air pressure and altitude (e.g. humidity, temperature, reference atmospheric pressure), or the information regarding to the relationship between altitude and floor number (e.g. altitude and floor number for the cells located in the building).