3GPP TSG-RAN WG2#91bis	R2-154753								R2-15xxxx

5—9 October, Malmo, Sweden.

Agenda Item:	7.1.2

Source: 	Motorola Mobility

Title:  	RSRP measurement for LAA

Document for:	Discussion and Decision

# Introduction

LAA operations depend heavily on the LBT procedure. If the channel is occupied, the eNB cannot transmit. If the carrier has a significant amount of activity from other nodes (Wi-Fi, LAA…) transmissions will be delayed. The delay of transmissions applies to all transmissions including specifically Discovery Signal transmissions, which are used for RRM measurements. The unpredictability of Discovery Signal transmissions can lead to issues with RSRP measurements.

# Discussion

RSRP Measurements in LTE have relied on the presence of CRS signals. The UE physical layer provides one physical layer measurement to RRC once every 200 ms when the UE is not in DRX. These physical layer measurements are smoothed using a layer 3 filter, and the filtered measurements are used for decisions regarding measurement reporting, handovers, etc.

RSRP measurements for LAA are performed based on measurements of Discovery Signal. . The UE is provided a DMTC configuration, from which the UE derives time windows during which it expects to detect a Discovery Signal transmission. However, transmissions of Discovery Signal are subject to the LBT procedure. Thus if the channel is occupied, during the DMTC window, the Discovery Signal transmission is delayed or skipped. Note also that the DMTC window needs to be kept small to ensure that the UE power consumption for detecting and measuring Discovery Signal signals remains manageable.

It has been agreed that if a Discovery Signal transmission is skipped (or significantly delayed) the UE physical layer does not provide a corresponding physical layer measurement to RRC [1]. Thus there can be long periods during which the UE does not detect a Discovery Signal transmission. If the signal conditions have changed from the previous measurement, when the UE does detect and measure Discovery Signal after a long period, the measured value could have changed dramatically. This change combined with the L3 filtering can cause undesirable effects, which we discuss below.

Figure 1 shows a simulated sequence of RSRP measurements and the corresponding mean RSRPs used for the simulated measurements. The RSRP measurements are generated as normally distributed values with a standard deviation of 3 db and a mean value that changes over time as follows. If the duration between measurements is greater than 1 second, the mean decreases by 8 db for a randomly selected duration. Such changes in the mean RSRP represent the changes that can occur, for example, due to increased activity from other nodes on the carrier. The measurement sequence is intentionally designed to have some short and some long periods with the low mean.

Figure 1: Simulated RSRP measurement sequence and Corresponding Mean RSRPs

Figure 2 shows the effect of applying a L3 filter with a filter coefficient value k of 4. Below are details of the L3 filter as described in TS 36.331. The updated filtered value Fn is computed as follows:

Fn=1-a∙Fn-1+a∙Mn

where Mn is the latest received measurement result from the physical layer, and Fn-1 is the old filtered measurement result. a=12k4, where k is the filter coefficient.

Figure 2: Effect of L3 Filtering

As can be seen from Figure 2, the L3 filter is not sensitive to the length of time for which the measurements are low. And since the low measurements in this scenario occur after a long period of no Discovery Signal transmissions, in effect the L3 filter is not sensitive to the time durations between Discovery Signal transmissions.

A similar situation occurs when the UE performs measurements when configured with DRX. When configured with DRX, the rate at which the physical layer measurements are available changes based on the DRX configuration. TS 36.331 allows for this and places the following requirement on the UE:

2>	adapt the filter such that the time characteristics of the filter are preserved at different input rates, observing that the filterCoefficient k assumes a sample rate equal to 200 ms;

In the case of LAA however, there isn’t a clear notion of filter “input rates” due to the unpredictability of Discovery Signal transmissions. So, while it is clear that the UE needs to perform some adaptation of the filter characteristics to support Discovery Signal measurements, it is not clear how this would be done.

Observation: It is clear that the UE needs to perform some adaptation of the filter characteristics to support Discovery Signal measurement. However, it is not clear what this adaptation would be.

In particular, with LAA, the duration between any two successive Discovery Signal transmissions is unpredictable. So, it is necessary to adjust the filter characteristics based on the duration between successive Discovery Signal transmissions.

Below we consider one approach where if the duration between successive Discovery Signal transmissions is more than a predefined duration, the UE “forgets history” (i.e., sets a=1). Specifically,

Fn=1-a'∙Fn-1+a'∙Mn

where Mn is the latest received measurement result from the physical layer, and Fn-1 is the old filtered measurement result.

a'=1  if the duration between Mn andMn-1 is more than T12k4  otherwise,

where k is the filter coefficient and T is the predefined duration.

Figure 3 shows the effect of using modified filter described above. To summarize, the forgetting history approach does not help much with the sensitivity of the filter to time durations between Discovery Signal transmissions.

Figure 3: Forgetting History Based on Duration Between Measurements

An alternative filter modification is described below.

Fn=1-a'∙Fn-1+a'∙Mn

where Mn is the latest received measurement result from the physical layer, and Fn-1 is the old filtered measurement result.

a'=12k4(1+e1-tT), where k is the filter coefficient, t is the duration between Mn and Mn-1, and T is the predefined duration.

Figure 4 shows the effect of using the modification described above. The filtering with this approach is more sensitive to the length of durations without Discovery Signal measurements.

Proposal: UE should adjust filter characteristics based on the duration between successive measurements.

Figure 4: Filter Modification Based on Duration Between Successive Measurements

# Summary

We have shown that there is a significant impact, on L3 filtered measurements, from periods when the Discovery Signal transmission is delayed significantly. This suggests a need for adaptation of the filter. A filter adaptation for DRX configurations is required per [2]. However, such adaptation relies on the periodic nature of DRX operations and corresponding performance requirements are specified in [3].

Observation: It is clear that the UE needs to perform some adaptation of the filter characteristics to support Discovery Signal measurement. However, it is not clear what this adaptation would be.

Proposal: UE should adjust filter characteristics based on the duration between successive measurements

# References

- RAN2#90 Meeting Report, May 25—29, Fukuoka, Japan.

- 3GPP TS 36.331

- 3GPP TS 36.133