3GPP TSG-RAN WG2 #95	Tdoc R2-165548

# Gothenburg, Sweden, 22nd – 26th August 2016

Agenda Item:	9.5.2

Source:	Ericsson

Title:	Mobility between LTE and NR for inactive UEs

Document for:	Discussion, Decision

# Introduction

At the previous RAN2/3 meeting it has been agreed that LTE eNBs should be connected to Next Gen CN to support RAN level inter-working (DC, fast mobility) between LTE and NR for UEs capable of supporting NR. This mobility can be made transparent to the CN assuming the same CN/RAN interface is used for LTE and NR.

In addition to active mode mobility also inactive mode mobility should be considered when the cell re-selection is performed by the UE. Currently in LTE there is support for UE triggered mobility in IDLE mode, in addition there is ongoing work in the “light connection” WI which may lead to a solution where UE triggered mobility is also supported when there is an active S1 connection (MME thinks the UE is in MM-Connected).

For NR there has also been work considering similar solution where the UE context is kept in the RAN and the CN/RAN interface connection is maintained, but where the UE is allowed to move around within local areas without notifying the network.

This contribution discusses possible requirements and solutions for supporting inactive UEs moving between LTE and NR, and proposes some working assumptions.

# Desired behaviour for inactive inter-RAT mobility

In order to support UE reachability for inactive UEs it is required that there is some sort of inactive mode mobility between NR and LTE. This means that the UE should, depending on network controlled criteria (e.g. thresholds, priorities), also perform inter-RAT measurement and trigger inter-RAT cell-reselection when conditions so dictate.

There needs to be support for inactive mode mobility between NR and LTE

Furthermore it is expected in many cases that the radio coverage will be different for different frequency bands, node types and RATs which in the worst case could lead to quite frequent toggling between RATs for some UEs in inactive mode. At the same time for UE performance it is desired to always camp on the best RAT. For this reason it is desirable that the signalling is minimized for UEs performing this transition.

The signalling for inactive mode mobility between NR and LTE should be minimized.

# Solutions with no RAN context or active CN / RAN connection

SA2 has agreed that Next Gen CN will support a CN IDLE state with no RAN context or active CN / RAN connection. Solutions for this state is left for further discussion in SA2, however it is expected that the solution would be harmonised for LTE and NR, allowing things like overlapping Tracking Area (or equivalent) list which spans across both NR and LTE. This would make it possible to minimize signalling when the UE move between NR and LTE coverage. When the UE performs a Service Request or Tracking Area update the normal procedure of that RAT will be followed.

Observation 1: CN IDLE state will be supported in Next Gen CN and can be used both in LTE and NR

# Solutions with RAN context and active CN / RAN connection

In this case it is assumed that the RAN / CN connection is maintained when the UE enters inactive mode. One possible solution to support mobility in this case between LTE and NR would be for the UE to fall back to solutions when there is no RAN context, i.e., the UE would enter CN IDLE.

In this case two sub-solutions could be considered:

- One solution where the UE performs signaling e.g. a Tracking Area Update. In this case it is possible to put back the UE in RAN connected inactive state if desired.

- Another solution is that the UE is only required to perform signaling if it enters a new TA which it is not registered to. In case the UE enters a TA it is registered the UE will do nothing.

The drawback with this solution could be the following:

- With the first sub-solution the UE will always generate signaling (at least for the initial transition), if the UE re-enters a state where the RAN context is stored, this will increase signaling further, especially if UE later moves back to the first RAT

- With the second sub solution the drawback is that the UE would no longer be reachable for RAN paging, but the network would not know this and would first try RAN paging before falling back to CN paging. There is also a risk that any DL packet that was sent to the old RAT will be lost during the transition since the RAN context will be re-build from the CN. Another drawback is that this would slow down the subsequent transition to active state e.g. when the UE has data.

Observation 2: For UEs with a RAN context, falling back to CN IDLE solutions with no RAN context has some drawbacks

An alternative solution to falling back to CN IDLE based procedure is to use a RAN based procedure for supporting mobility between LTE and NR. The procedure can be similar to the procedure used intra-RAT for UEs in this state. The basic principle for the procedure is as follows:

- UE is allowed to perform cell-reselection between the RATs

- if the UE enters an area which it is not registered in it will perform a registration update (the area and the registration update can depending on the solution be TA list and TAU)

- when performing the registration update the UE will include a RAN level UE context identifier and security information (e.g. a MAC)  which is common for both LTE and NR

- target RAN node will then “fetch” the RAN context from the source RAN node using the UE context identifier (e.g. over an X2 like interface) and update the Next Gen CN about the mobility (e.g. a path update procedure)

Even though (large) parts of an LTE RRC Configuration (L1, HARQ, …) may be irrelevant for setting up an NR RRC Connection, the re-use of flows/bearers as well as security credentials can significantly reduce the CN signaling and corresponding delays. With this solution in cases where the registration areas are allowed to overlap LTE and NR there is no need for any signaling when the UE changes RAT. The registration area (or area list) can be managed by the Next Gen CN.

Support solutions where RAN context and S1 connection can be kept and transferred between LTE and NR for UEs with RRC context and active CN / RAN connection.

# Conclusion

Based on the discussion we make the following proposals:

Proposal 1	There needs to be support for inactive mode mobility between NR and LTE

Proposal 2	The signalling for inactive mode mobility between NR and LTE should be minimized.

Proposal 3	Support solutions where RAN context and S1 connection can be kept and transferred between LTE and NR for UEs with RRC context and active CN / RAN connection.