3GPP TSG-RAN2 Meeting #95	R2-165731

# Gothenburg, Sweden, 22nd Aug 2016 - 26th Aug 2016

| CR-Form-v11.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 25.331 | CR | 5874 | rev | 1 | Current version: | 12.10.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | On DCH-enhancements concurrency with HS/EUL |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Qualcomm Europe Inc.(France) |  |  |  |  |  |  |  |  |  |
| Source to TSG: |  |  |  |  |  |  |  |  |  |  |
| Work item code: | UTRA_DCHenh-Core |  |  |  |  |  | Date: |  |  | 2016-08-22 |
| Category: | C |  |  |  |  |  | Release: |  |  | Rel-12 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) |  |
| Reason for change: |  | With DCH enhancements, there are no UE battery power gains when HS/EUL are configured concurrently with 20 ms only mode. This requirement causes unnecessary high complexity of implementation but provides no gains at the UE. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Allow DCH enhancements and HS/EUL concurrency when CPC DTX/DRX is not active only for UEs that (1) support Full Mode Capability (2) when 20 ms UL mode is signalled. Impact analysis Impacted functionality: DCH Enhancements Inter-operability: - If the network is implemented according to the CR but the UE is not, there is no inter-operability issue - If the UE is implemented according to the CR but the network is not, UE will reject the configuration when network configures HS/EUL with CPC DTX/DRX in DCH enhancement in basic mode or in full mode with UL mode not set to 20ms. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | High complexity of implementation and no UE power gains. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.5.84 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | x | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

****************************************** TEXT START *****************************************

### 8.5.84	Actions related to DCH_ENHANCEMENTS_STATUS variable (FDD only)

If the UE receives an ACTIVE SET UPDATE, CELL UPDATE CONFIRM, or any reconfiguration message, the UE shall:

1>	take actions related to the IE "DCH Enhancements info FDD" as specified in subclause 8.6.6.64;

1>	determine the value of the DCH_ENHANCEMENTS_STATUS variable.

The DCH_ENHANCEMENTS_STATUS variable shall be set to TRUE only when all the following conditions are met:

1>	the UE is in CELL_DCH state;

1>	the UE supports DCH Enhancements;

1>	the UE has stored IE "DCH Enhancements info FDD".

If any of the above conditions is not met and the DCH_ENHANCEMENTS_STATUS variable is set to TRUE, the UE shall:

1>	set the DCH_ENHANCEMENTS_STATUS variable to FALSE;

1>	trigger the lower layers to stop operation with DCH Enhancements.

Whenever the variable DCH_ENHANCEMENTS_STATUS is set to TRUE, the UE shall:

1>	if the UE indicates basic support for DCH Enhancements and choice "DL FET Mode" is set to Full; or

1>	if the IE "Power offset PTPC-DPDCH" is not included for any one or more of the radio links in the active set; or

1>	if the IE "Power offset POSRB" is not included for any one or more of the radio links in the active set; or

1>	if the IE "Number of bits for Pilot bits" is not set to 0; or

1>	if the variables HS_DSCH RECEPTION or E_DCH_TRANSMISSION are set to TRUE, the variable DTX_DRX_STATUS is set to FALSE, and DL FET Basic mode is indicated in the IE "DL FET Mode"; or

1>	if the variables HS_DSCH RECEPTION or E_DCH_TRANSMISSION are set to TRUE, the variable DTX_DRX_STATUS is set to FALSE, and DL FET Full mode is indicated in the IE "DL FET Mode" with choice "UL Transmission Mode" is not set to 20ms:

2>	set the variable INVALID_CONFIGURATION to TRUE.

1>	else:

2>	trigger the lower layers to operate with DCH Enhancements in DL FET Basic mode or DL FET Full mode as indicated in the IE "DL FET Mode".

Whenever the variable DCH_ENHANCEMENTS_STATUS is set to FALSE, the UE shall:

1>	if the IE "Number of bits for Pilot bits" is set to 0:

2>	if the UE is in CELL_DCH state:

3>	set the variable INVALID_CONFIGURATION to TRUE.

******************************************* TEXT END ******************************************