3GPP TSG-RAN WG2 Meeting #95	R2-165797

Gothenburg, Sweden, 22 - 26 August 2016

Title:	[DRAFT] Reply LS on RAN impacts of Shared MBMS Network and Receive Only Mode in eMBMS

Response to:	LS (S2-163212) on “LS out on RAN impacts from enTV study of SA2” from SA2

LS (S2-164141) on “LS on RAN impacts from Receive Only Mode UE” from SA2

Release:	Rel-14

Work Item:	SA2: Study on System Architecture Enhancements to eMBMS for TV Video Service

RAN2: eMBMS enhancements for LTE

Source:	Nokia [to be RAN2]

To:	SA2, RAN3

Cc:	-

Contact Person:

#### Name:	Dawid Koziol

Tel. Number:

E-mail Address:	dawid.koziol@nokia.com

Attachments:	-

1. Overall Description:

RAN2 would like to thank SA2 for their LSs concerning RAN impacts of their enTV study provided in S2-163212 and S2-164141. RAN2 has analysed the mentioned issues and would like to provide the following feedback:

For “SA2 Requested feedback 1 (RAN2/RAN3):

SA2 would like to request feedback from RAN2/RAN3 regarding the feasibility of MBSFN synchronization area spanning within and across PLMNs (Option A) from RAN perspective.” from LS S2-163212

From RAN2 perspective MBSFN synchronization area spanning within and across PLMNs is feasible. It has to be noted that two conditions need to be met:

- eNBs need to belong to only one MBSFN Synchronization Area at a time meaning that eNBs belonging to different PLMNs need to be configured with the same MBSFN Synchronization Area ID

- eNBs belonging to different PLMNs need to be synchronized with each other, which RAN2 finds technically feasible, but the details of how to achieve that are out of the responsibility of RAN2.

For “SA2 request (RAN2):

SA2 would like to request feedback on the feasibility of  Receive Only Mode and Receive Only Mode with independent unicast from RAN perspective.” from LS S2-163212 and for feedback requested in LS S2-164141

From RAN2 point of view there is no difference in the way the MBMS data is delivered to Receive Only Mode and legacy eMBMS UEs. RAN2 finds Receive Only Mode and Receive Only Mode with independent unicast feasible from RAN perspective.

For feedback requested in LS S2-164141

RAN2 discussed the concerns of SA2 on UE radio performance and other issues and identified the following possibilities:

- One bit indication that a cell might provide services for Receive Only Mode UEs:

  - When this indication is used to indicate the cell does not provide MBMS services for Receive Only Mode UEs, then the Receive Only Mode UE does not need to read other SIBs and MCCH.

  - When this indication is used to indicate that there is a possibility of a cell providing services destined for Receive Only Mode UEs, it may still happen that the service is not currently provided in the cell and UE will still have to read SIB13 and MCCH to find that out. It is worth mentioning that if such indication is allowed to be changed dynamically such indication would also impact normal UEs as these would also have to re-read the system information due to such change.

- No indication – UEs follow existing procedures and apply currently specified behaviour:

  - In case Receive Only Mode UE is preconfigured only with TMGIs and frequencies then UE needs to read SIB13 and MCCH to verify whether the service is provided in a cell

  - Receive Only Mode UE can be additionally pre-configured with MBMS SAI(s), which can be specific to the services destined for Receive Only Mode UEs. In this situation UE can read SIB15 to check whether the pre-configured services are available in its serving or neighbouring cells. UE would read SIB13 and MCCH only in case its pre-configured information matches the one signalled in SIB15.

From RAN2 perspective both options allow to achieve the same goal, but using existing procedures has no impact on RAN and therefore is a preferred option from RAN2 point of view.

RAN2 shortly discussed testability and security of the aforementioned options as well, but has reached a conclusion that SA2 should contact RAN5 and SA3 respectively if SA2 has concerns about these issues.

2. Actions:

To SA2, RAN3 groups.

ACTION: 	RAN2 respectfully asks SA2 and RAN3 to take the above information into account in their further work within related Work Items.

3. Date of Next TSG-RAN2 Meetings:

TSG-RAN WG2 Meeting #95bis	10th – 14th October 2016	Kaohsiung, Taiwan

TSG-RAN WG2 Meeting #96	14th – 18th November 2016	Reno, Nevada, US