3GPP TSG-RAN WG2 Meeting #95bis 	R2-166873

# Kaohsiung, Taiwan, 10th -14th  October 2016

| CR-Form-v11.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 25.331 | CR | - | rev | - | Current version: | 14.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Introduction of eDECOR in RAN |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R2 |  |  |  |  |  |  |  |  |  |
| Work item code: | DECOR-UTRA_LTE-Core |  |  |  |  |  |  | Date: |  | 2016-09-30 |
| Category: | B |  |  |  |  |  |  | Release: |  | Rel-14 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) |  |
| Reason for change: |  | eDECOR has been finalized in SA2#116. It needs to be introduced in RAN to support the funcationality. |  |  |  |  |  |  |  |  |
| Summary of change: |  | DCN ID is added in RRC Connection Setup Complete. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | eDECOR is not supported in UTRAN system. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 2, 3.2, 8.1.3.6, 10.2.41, 11.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  | X |  | Other core specifications |  |  | TS 25.413 CR R3-162450 TS 36.413 CR R3-162451 TS 36.331 CR R2-166862 |  |  |  |
| affected: |  |  | X | Test specifications |  |  | TS/TR ... CR ... |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  | TS/TR ... CR ... |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

- References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

- For a specific reference, subsequent revisions do not apply.

- For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 25.301: "Radio Interface Protocol Architecture".

[3]	3GPP TS 25.303: "Interlayer Procedures in Connected Mode".

[4]	3GPP TS 25.304: "UE Procedures in Idle Mode and Procedures for Cell Reselection in Connected Mode".

[5]	3GPP TS 24.008: "Mobile radio interface layer 3 specification; Core Network Protocols; Stage 3".

[6]	3GPP TS 25.103: "RF parameters in support of RRM".

[7]	3GPP TS 25.215: "Physical layer – Measurements (FDD)".

[8]	3GPP TS 25.225: "Physical layer – Measurements (TDD)".

[9]	3GPP TS 25.401: "UTRAN overall description".

[10]	3GPP TS 25.402: "Synchronization in UTRAN; Stage 2".

[11]	3GPP TS 23.003: "Numbering, addressing and identification".

[12]	ICD-GPS-200: "Navstar GPS Space Segment/Navigation User Interface".

[13]	RTCM-SC104: "RTCM Recommended Standards for Differential GNSS Service (v.2.2)".

[14]	3GPP TR 25.921: "Guidelines and principles for protocol description and error handling".

[15]	3GPP TS 25.321: "Medium Access Control (MAC) protocol specification".

[16]	3GPP TS 25.322: "Radio Link Control (RLC) protocol specification".

[17]	3GPP TS 24.007: "Mobile radio interface signalling layer 3; General aspects".

[18]	3GPP TS 25.305: "Stage 2 Functional Specification of UE Positioning in UTRAN".

[19]	3GPP TS 25.133: "Requirements for Support of Radio Resource Management (FDD)".

[20]	3GPP TS 25.123: "Requirements for Support of Radio Resource Management (TDD)".

[21]	3GPP TS 25.101: "UE Radio Transmission and Reception (FDD)".

[22]	3GPP TS 25.102: "UE Radio Transmission and Reception (TDD)".

[23]	3GPP TS 23.060: "General Packet Radio Service (GPRS); Service description; Stage 2".

[24]	3GPP TS 23.032: "Universal Geographical Area Description (GAD)".

[25]	3GPP TS 23.122: "Non-Access-Stratum functions related to Mobile Station (MS) in idle mode".

[26]	3GPP TS 25.211: "Physical channels and mapping of transport channels onto physical channels (FDD)".

[27]	3GPP TS 25.212: "Multiplexing and channel coding (FDD)".

[28]	3GPP TS 25.213: "Spreading and modulation (FDD)".

[29]	3GPP TS 25.214: "Physical layer procedures (FDD)".

[30]	3GPP TS 25.221: "Physical channels and mapping of transport channels onto physical channels (TDD)".

[31]	3GPP TS 25.222: "Multiplexing and channel coding (TDD)".

[32]	3GPP TS 25.223: "Spreading and modulation (TDD)".

[33]	3GPP TS 25.224: "Physical Layer Procedures (TDD)".

[34]	3GPP TS 25.302: "Services provided by the physical layer ".

[35]	3GPP TS 25.306 "UE Radio Access Capabilities".

[36]	3GPP TS 25.323: "Packet Data Convergence Protocol (PDCP) Specification".

[37]	3GPP TS 25.324: "Broadcast/Multicast Control BMC".

[38]	3GPP TR 25.922: "Radio resource management strategies".

[39]	3GPP TR 25.925: "Radio interface for broadcast/multicast services".

[40]	3GPP TS 33.102: "3G Security; Security Architecture".

[41]	3GPP TS 34.108: "Common Test Environments for User Equipment (UE) Conformance Testing".

[42]	3GPP TS 34.123-2: "User Equipment (UE) conformance specification; Part 2: Implementation Conformance Statement (ICS) proforma specification".

[43]	3GPP TS 44.018: "Mobile radio interface layer 3 specification; Radio Resource Control Protocol".

[44]	3GPP TS 44.060: "General Packet Radio Service (GPRS); Mobile Station (MS) - Base Station System (BSS) interface; Radio Link Control/Medium Access Control (RLC/MAC) protocol".

[45]	3GPP TS 45.005: "Radio transmission and reception".

[46]	3GPP TS 45.008: "Radio subsystem link control".

[47]	ITU-T Recommendation X.680 (07/2002) "Information Technology - Abstract Syntax Notation One (ASN.1): Specification of basic notation".

[48]	ITU-T Recommendation X.681 (07/2002) "Information Technology - Abstract Syntax Notation One (ASN.1): Information object specification".

[49]	ITU-T Recommendation X.691 (07/2002) "Information technology - ASN.1 encoding rules: Specification of Packed Encoding Rules (PER)".

[50]	3GPP TS 31.102: "Characteristics of the USIM Application".

[51]	3GPP TS 25.308: "High Speed Downlink Packet Access (HSDPA): Overall Description; Stage 2".

[52]	IANA ROHC profile identifier definition (http://www.iana.org/assignments/rohc-pro-ids).

[53]	3GPP TS 44.118: "Mobile radio interface layer 3 specification; Radio Resource Control Protocol, Iu Mode".

[54]	3GPP TS 23.246: "Multimedia Broadcast Multicast Service; Architecture and Functional Description".

[55]	3GPP TS 25.346: " Introduction of the Multimedia Broadcast Multicast Service (MBMS) in the Radio Access Network (Stage-2)".

[56]	3GPP TR 25.992: "Multimedia Broadcast Multicast Service (MBMS); UTRAN/GERAN Requirements".

[57]	3GPP TS 25.413: "UTRAN Iu Interface RANAP Signalling".

[58]	3GPP TS 25.309: "FDD Enhanced Uplink; Overall Description; Stage 2".

[59]	3GPP TS 25.319: "Enhanced Uplink; Overall Description; Stage 2".

[60]	3GPP TR 25.827: "1.28Mcps TDD Enhanced Uplink; Physical Layer Aspects".

[61]	Galileo OS Signal in Space ICD (OS SIS ICD), Issue 1.2,  February 2014, European Union.

[62]	3GPP TS 26.101: "Adaptive Multi-Rate (AMR) speech codec frame structure".

[63]	3GPP TS 26.201: "Adaptive Multi Rate – Wideband (AMR-WB) speech codec frame structure".

[64]	3GPP TS 36.101: "Evolved Universal Terrestrial Radio Access (E-UTRA); User Equipment (UE) radio transmission and reception".

[65]	3GPP TS 36.104: "Evolved Universal Terrestrial Radio Access (E-UTRA); Base Station (BS) radio transmission and reception".

[66]	3GPP TS 36.304: "Evolved Universal Terrestrial Radio Access (E-UTRA); User Equipment (UE) procedures in idle mode".

[67]	3GPP TS 36.331: "Evolved Universal Terrestrial Radio Access (E-UTRA); Radio Resource Control (RRC); Protocol Specification".

[68]	IS-GPS-200, Revision D, Navstar GPS Space Segment/Navigation User Interfaces, March 7th, 2006.

[69]	IS-GPS-705, Navstar GPS Space Segment/User Segment L5 Interfaces, September 22, 2005.

[70]	IS-GPS-800, Navstar GPS Space Segment/User Segment L1C Interfaces, March 31, 2008.

[71]	Specification for the Wide Area Augmentation System (WAAS), US Department of Transportation, Federal Aviation Administration, DTFA01-96-C-00025, 2001.

[72]	IS-QZSS, Quasi Zenith Satellite System Navigation Service Interface Specifications for QZSS, Ver.1.0, June 17, 2008.

[73]	Global Navigation Satellite System GLONASS Interface Control Document, Version 5.1, 2008.

[74]	3GPP TS 36.133: "Evolved Universal Terrestrial Radio Access (E-UTRA); Requirements for support of radio resource management".

[75]	3GPP TS 22.011: "Service accessibility".

[76]	3GPP TS 23.003: "Numbering, addressing and identification".

[77]	3GPP TS 23.041: "Technical realization of Cell Broadcast Service (CBS)".

[78]	3GPP TS 33.401: "3GPP System Architecture Evolution (SAE): Security architecture".

[79]	3GPP TS 24.301: " Non-Access-Stratum (NAS) protocol for Evolved Packet System (EPS)".

[80]	3GPP TS 37.320: "Universal Terrestrial Radio Access (UTRA) and Evolved Universal Terrestrial Radio Access (E-UTRA); Radio measurement collection for Minimization of Drive Tests (MDT); Overall description; Stage 2".

[81]	3GPP TS 32.422: "Telecommunication management; Subsriber and equipment trace; Trace control and confiuration management".

[82]	3GPP TS 22.368: "Service Requirements for Machine Type Communications; Stage 1".

[83]	RFC 3095: "RObust Header Compression (ROHC): Framework and four profiles".

[84]	RFC 4815: "RObust Header Compression (ROHC): Corrections and Clarifications to RFC 3095".

[85]	3GPP TS 25.307: "Requirement on User Equipments (UEs) supporting a release-independent frequency band".

[86]	BDS-SIS-ICD-2.0: "BeiDou Navigation Satellite System Signal In Space Interface Control Document Open Service Signal (Version 2.0), December 2013".

[87]	IEEE 802.11-2012, Part 11: "Wireless LAN Medium Access Control (MAC) and Physical Layer (PHY) specifications, IEEE Std.".

[88]	3GPP TS 24.312: "Access Network Discovery and Selection Function (ANDSF) Management Object (MO)".

[89]	ATIS-0500027: "Recommendations for Establishing Wide Scale Indoor Location Performance," May 2015.

[90]				Bluetooth Special Interest Group: "Bluetooth Core Specification v4.2", December 2014

[91]	3GPP TS 23.401: "General Packet Radio Service (GPRS) enhancements for Evolved Universal Terrestrial Radio Access Network (E-UTRAN) access".

------------------------------------------

Skip to next change

------------------------------------------

## 3.2	Abbreviations

For the purposes of the present document, the following abbreviations apply:

ACDC	Application specific Congestion control for Data Communication

ACK	Acknowledgement

AG	Absolute Grant

AICH	Acquisition Indicator CHannel

AM	Acknowledged Mode

ANDSF	Access Network Discovery and Selection Function

ANR	Automatic Neighbour Relation

AP	Access Point

AS	Access Stratum

ASC	Access Service Class

ASN.1	Abstract Syntax Notation.1

BCCH	Broadcast Control Channel

BCD	Binary Coded Decimal

BCFE	Broadcast Control Functional Entity

BDS	BeiDou Navigation Satellite System

BER	Bit Error Rate

BLER	BLock Error Rate

BSS	Base Station Sub-system

CCCH	Common Control Channel

CCPCH	Common Control Physical CHannel

CH	Conditional on history

CLTD	Closed Loop Transmit Diversity

CM	Connection Management

CN	Core Network

C-RNTI	Cell RNTI

CSFB	CS Fallback

CSG	Closed Subscriber Group

CTCH	Common Traffic CHannel

CTFC	Calculated Transport Format Combination

CV	Conditional on value

DBDS	Differential BDS

DCA	Dynamic Channel Allocation

DCCH	Dedicated Control Channel

DCFE	Dedicated Control Functional Entity

DCH	Dedicated Channel

DCN	Dedicated Core Networks

DC-SAP	Dedicated Control SAP

DDI	Data Description Indicator

DGANSS	Differential GANSS

DGPS	Differential Global Positioning System

DL	Downlink

DPCCH2	Dedicated Physical Control Channel 2

DSCH	Downlink Shared Channel

DTCH	Dedicated Traffic Channel

DTM	Dual Transfer Mode

EAB	Extended Access Barring

E-AGCH	E-DCH Absolute Grant Channel

ECEF	Earth-Centered, Earth-Fixed

ECI		Earth-Centered-Inertial

E-DCH	Enhanced uplink DCH

E-DPCCH	E-DCH Dedicated Physical Control Channel (FDD Only)

E-DPDCH	E-DCH Dedicated Physical Data Channel (FDD Only)

EGNOS	European Geostationary Navigation Overlay Service

E-HICH	E-DCH HARQ Acknowledgement Indicator Channel

E-PUCH	Enhanced Uplink Physical Channel (TDD only)

E-RGCH	E-DCH Relative Grant Channel (FDD only)

E-RNTI	E-DCH RNTI

E-ROCH	E-DCH Rank and Offset Channel (FDD only)

E-RUCCH	E-DCH Random Access Uplink Control Channel (TDD only)

E-TFCI	E-DCH Transport Format Combination Indicator

ETWS	Earthquake and Tsunami Warning System

E-UCCH	E-DCH Uplink Control Channel (TDD only)

E-UTRA	Evolved Universal Terrestrial Radio Access

E-UTRAN	Evolved Universal Terrestrial Radio Access Network

FACH	Forward Access Channel

FDD	Frequency Division Duplex

F-DPCH	Fractional DPCH

F-TPICH	Fractional Transmitted Precoding Indicator Channel

GAGAN	GPS Aided Geo Augmented Navigation

GANSS	Galileo and Additional Navigation Satellite Systems

GC-SAP	General Control SAP

GERAN	GSM/EDGE Radio Access Network

GLONASS	GLObal'naya NAvigatsionnaya Sputnikovaya Sistema (Engl.: Global Navigation Satellite System)

GNSS	Global Navigation Satellite System

GRA	GERAN Registration Area

G-RNTI	Geran Radio Network Temporary Identity

HARQ	Hybrid Automatic Repeat Request

HCS	Hierarchical Cell Structure

HFN	Hyper Frame Number

H-RNTI	HS-DSCH RNTI

HS-DSCH	High Speed Downlink Shared Channel

ICD	Interface Control Document

ID	Identifier

IDNNS	Intra Domain NAS Node Selector

IE	Information element

IETF	Internet Engineering Task Force

IMB	Integrated Mobile Broadcast

IMEI	International Mobile Equipment Identity

IMSI	International Mobile Subscriber Identity

IP	Internet Protocol

ISCP	Interference on Signal Code Power

L1	Layer 1

L2	Layer 2

L3	Layer 3

LAI	Location Area Identity

MAC	Media Access Control

MBS	Metropolitan Beacon System

MBMS	Multimedia Broadcast Multicast Service

MBSFN	MBMS over a Single Frequency Network

MCC	Mobile Country Code

MCCH	MBMS point-to-multipoint Control Channel

MD	Mandatory default

MDT	Minimization of Drive Tests

MICH	MBMS notification Indicator Channel

MM	Mobility Management

MNC	Mobile Network Code

MP	Mandatory present

MTCH		MBMS point-to-multipoint Traffic Channel

MSAS	Multi-functional Satellite Augmentation System

MSCH	MBMS point-to-multipoint Scheduling Channel

NACC	Network Assisted Cell Change

NAS	Non Access Stratum

Nt-SAP	Notification SAP

NW	Network

OLTD	Open Loop Transmit Diversity

OP	Optional

PCCH	Paging Control Channel

PCH	Paging Channel

PDCP	Packet Data Convergence Protocol

PDSCH	Physical Downlink Shared Channel

PDU	Protocol Data Unit

PLMN	Public Land Mobile Network

PNFE	Paging and Notification Control Functional Entity

PRACH	Physical Random Access CHannel

PRN	Pseudo-Random Noise

PSI	Packet System Information

p-t-m	Point-to-Multipoint

P-TMSI	Packet Temporary Mobile Subscriber Identity

p-t-p	Point-to-Point

PUSCH	Physical Uplink Shared Channel

QoS	Quality of Service

QZSS	Quasi-Zenith Satellite System

RAB	Radio access bearer

RACH	Random Access CHannel

RAI	Routing Area Identity

RAT	Radio Access Technology

RB	Radio Bearer

RFE	Routing Functional Entity

RG	Relative Grant

RL	Radio Link

RLC	Radio Link Control

RLS	Radio Link Set

RNC	Radio Network Controller

ROHC	RObust Header Compression

RNTI	Radio Network Temporary Identifier

RRC	Radio Resource Control

RSCP	Received Signal Code Power

RSN	Retransmission Sequence Number

RSRP	Reference Signal Received Power

RSRQ	Reference Signal Received Quality

RSSI	Received Signal Strength Indicator

SAP	Service Access Point

SBAS	Satellite Based Augmentation System

SCFE	Shared Control Function Entity

SCTD	Space Code Transmit Diversity

SCTO	Soft Combining Timing Offset (MBMS)

S-DPCCH	Secondary Dedicated Physical Control Channel

S-E-DPCCH	Secondary E-DPCCH (FDD only)

S-E-DPDCH	Secondary E-DPDCH (FDD only)

SF	Spreading Factor

SG	Serving grant

SHCCH	Shared Control Channel

SI	System Information

SIR	Signal to Interference Ratio

SPS	Semi-Persistent Scheduling

S-RNTI	SRNC - RNTI

SV		Space Vehicle

TDD	Time Division Duplex

TF	Transport Format

TFCS	Transport Format Combination Set

TFS	Transport Format Set

TM	Transparent Mode

TME	Transfer Mode Entity

TMSI	Temporary Mobile Subscriber Identity

Tr	Transparent

TSN	Transmission Sequence Number

Tx	Transmission

UE	User Equipment

UL	Uplink

UM	Unacknowledged Mode

URA	UTRAN Registration Area

U-RNTI	UTRAN-RNTI

USCH	Uplink Shared Channel

UTC	Universal Coordinated Time

UTRAN	Universal Terrestrial Radio Access Network

WAAS	Wide Area Augmentation System

WGS-84	World Geodetic System 1984

WLAN	Wireless Local Area Network

------------------------------------------

Skip to next change

------------------------------------------

#### 8.1.3.6	Reception of an RRC CONNECTION SETUP message by the UE

The UE shall compare the value of the IE "Initial UE identity" in the received RRC CONNECTION SETUP message with the value of the variable INITIAL_UE_IDENTITY.

If the values are different, the UE shall:

1>	ignore the rest of the message.

If the values are identical, the UE shall:

1>	if IE "Specification mode" is set to "Preconfiguration" and IE "Preconfiguration mode" is set to "Predefined configuration":

2>	initiate the radio bearer and transport channel configuration in accordance with the predefined parameters identified by the IE "Predefined configuration identity" with the following exception:

3> ignore the IE "RB to setup list" and the IE "Re- establishment timer".

NOTE:	IE above IEs are mandatory to include in IE “Predefined RB configuration” that is included in System Information Block 16 but should be ignored since it is not possible to establish a RAB during RRC connection establishment.

2>	initiate the physical channels in accordance with the received physical channel information elements.

1>	if IE "Specification mode" is set to "Preconfiguration" and IE "Preconfiguration mode" is set to "Default configuration":

2>	initiate the radio bearer and transport channel configuration in accordance with the default parameters identified by the IE "Default configuration mode" and IE "Default configuration identity" with the following exception:

3>	ignore the radio bearers other than signalling radio bearers.

2>	initiate the physical channels in accordance with the received physical channel information elements.

NOTE:	IE "Default configuration mode" specifies whether the FDD or TDD version of the default configuration shall be used.

1>	if IE "Specification mode" is set to "Complete specification":

2>	initiate the radio bearer, transport channel and physical channel configuration in accordance with the received radio bearer, transport channel and physical channel information elements.

1>	if IE "Default configuration for CELL_FACH" is set:

2>	act in accordance with the default parameters according to section 13.8.

1>	clear the variable ESTABLISHMENT_CAUSE;

1>	for FDD and 1.28 Mcps TDD, if the HS_DSCH_RECEPTION_OF_CCCH_ENABLED is set to TRUE:

2>	set the variable HS_DSCH_RECEPTION_OF_CCCH_ENABLED to FALSE.

1>	stop timer T300 or T318, whichever one is running, and act upon all received information elements as specified in subclause 8.6, unless specified otherwise in the following:

2>	if the UE, according to subclause 8.6.3.3, will be in the CELL_FACH state at the conclusion of this procedure:

3>	if the IE "Frequency info" is included:

4>	select a suitable UTRA cell according to [4] on that frequency;

4>	for 1.28Mcps TDD, decide the working frequency according to subclause 8.6.6.1.

3>	else:

4>	for 1.28Mcps TDD, continue to use the currently used frequency as working frequency.

3>	enter UTRA RRC connected mode;

3>	determine the value for the READY_FOR_COMMON_EDCH variable and take the corresponding actions as described in subclause 8.5.47;

3>	determine the value for the COMMON_E_DCH_TRANSMISSION variable and take the corresponding actions as described in subclause 8.5.46;

3>	for FDD, determine the value for the READY_FOR_COMMON_ERGCH variable and take the corresponding actions as described in subclause 8.5.75;

3>	if variable READY_FOR_COMMON_EDCH is set to FALSE:

4>	select PRACH according to subclause 8.5.17.

3>	else:

4>	configure the Enhanced Uplink in CELL_FACH state and Idle mode as specified in subclause 8.5.45 for FDD and 8.5.45a for 1.28 Mcps TDD.

3>	for 3.84 Mcps and 7.68 Mcps TDD; or

3>	for FDD and 1.28 Mcps TDD, if the UE does not support HS-DSCH reception in CELL_FACH state; or

3>	if the IE "HS-DSCH common system information" is not included in System Information Block type 5 or System Information Block type 5bis; or

3> for 1.28 Mcps TDD, if the IE "Common E-DCH system info" is not included in System Information Block type 5:

4>	select Secondary CCPCH according to subclause 8.5.19.

3>	else:

4>	set variable HS_DSCH_RECEPTION_OF_CCCH_ENABLED to FALSE;

4>	For FDD if variable READY_FOR_COMMON_EDCH is set to FALSE:

5>	if the RBs have the multiplexing option with transport channel type "HS-DSCH" for the DL and transport channel type “RACH” in the UL; and

5>	if "new H-RNTI" and "new C-RNTI" are included:

6>	store the "new H-RNTI" according to subclause 8.6.3.1b;

6>	store the "new C-RNTI" according to subclause 8.6.3.9;

6>	and start to receive HS-DSCH according to the procedure in subclause 8.5.36.

5>	else:

6>	clear variable C_RNTI and delete any stored C-RNTI value;

6>	clear variable H_RNTI and delete any stored H-RNTI value;

6>	clear any stored IE "HARQ Info";

6>	set the variable INVALID_CONFIGURATION to TRUE.

4>	else:

5>	if the RBs have the multiplexing option with transport channel type "HS-DSCH" for the DL and transport channel type "E-DCH" in the UL; and

5>	if the IEs "new Primary E-RNTI", "new H-RNTI" and "new C-RNTI" are included:

6>	store the "new Primary E-RNTI" according to subclause 8.6.3.14;

6>	store the "new H-RNTI" according to subclause 8.6.3.1b;

6>	store the "new C-RNTI" according to subclause 8.6.3.9;

6>	configure Enhanced Uplink in CELL_FACH state and Idle mode according to subclause 8.5.45 for FDD and 8.5.45a for 1.28 Mcps TDD;

6>	and start to receive HS-DSCH according to the procedure in subclause 8.5.36.

5>	else:

6>	clear variable C_RNTI and delete any stored C-RNTI value;

6>	clear variable H_RNTI and delete any stored H-RNTI value;

6>	clear variable E_RNTI and delete any stored E-RNTI value;

6>	clear any stored IE "HARQ Info";

6>	set the variable INVALID_CONFIGURATION to TRUE.

3>	ignore the IE "UTRAN DRX cycle length coefficient" and stop using DRX.

1>	if the UE, according to subclause 8.6.3.3, will be in the CELL_DCH state at the conclusion of this procedure:

2>	perform the physical layer synchronisation procedure A as specified in [29] (FDD only);

2>	enter UTRA RRC connected mode;

2>	ignore the IE "UTRAN DRX cycle length coefficient" and stop using DRX.

1>	submit an RRC CONNECTION SETUP COMPLETE message to the lower layers on the uplink DCCH after successful state transition per subclause 8.6.3.3, with the contents set as specified below:

2>	set the IE "RRC transaction identifier" to:

3>	the value of "RRC transaction identifier" in the entry for the RRC CONNECTION SETUP message in the table "Accepted transactions" in the variable TRANSACTIONS; and

3>	clear that entry.

2>	if the USIM or SIM is present:

3>	set the "START" for each CN domain in the IE "START list" in the RRC CONNECTION SETUP COMPLETE message with the corresponding START value that is stored in the volatile memory of the ME if the USIM [50], or the SIM is present.

2>	if neither the USIM nor SIM is present:

3>	set the "START" for each CN domain in the IE "START list" in the RRC CONNECTION SETUP COMPLETE message to zero;

3>	set the value of "THRESHOLD" in the variable "START_THRESHOLD" to the default value [40].

2>	retrieve its UTRA UE radio access capability information elements from variable UE_CAPABILITY_REQUESTED; and then

2>	include this in IE "UE radio access capability" and IE "UE radio access capability extension", provided this IE is included in variable UE_CAPABILITY_REQUESTED;

2>	retrieve its inter-RAT-specific UE radio access capability information elements from variable UE_CAPABILITY_REQUESTED; and then

2>	include this in IE "UE system specific capability";

2>	if the UE supports contiguous multi-cell operation on four cells in the same band, include the IE "Additional Secondary Cells" in the IE "UE radio access capability extension" and set it to 'a2' for the supported bands; otherwise, if the UE supports contiguous multi-cell operation on three cells in the same band, include the IE "Additional Secondary Cells" in the IE "UE radio access capability extension" and set it to 'a1' for the supported bands;

2>	if the UE supports contiguous multi-cell operation on eight cells in the same band, include the IE "Additional Secondary Cells 2" in the IE "UE radio access capability extension" and set it to 'a6' for the supported bands;

2>	if the UE supports contiguous multi-cell operation on seven cells in the same band, include the IE "Additional Secondary Cells 2" in the IE "UE radio access capability extension" and set it to 'a5' for the supported bands;

2>	if the UE supports contiguous multi-cell operation on six cells in the same band, include the IE "Additional Secondary Cells 2" in the IE "UE radio access capability extension" and set it to 'a4' for the supported bands;

2>	if the UE supports contiguous multi-cell operation on five cells in the same band, include the IE "Additional Secondary Cells 2" in the IE "UE radio access capability extension" and set it to 'a3' for the supported bands;

2>	if the UE supports non-contiguous multi-cell operation on two cells in the same band:

3>	set the IE "Aggregated cells" in IE "Non-contiguous multi-cell" in the IE "UE radio access capability extension" to 'nc-2c' for the supported band;

3>	set the IE "Gap size" in IE "Non-contiguous multi-cell" in the IE "UE radio access capability extension" for the supported band to 'fiveMHz' or 'tenMHz' or 'anyGapSize', or

2>	if the UE supports non-contiguous multi-cell operation on three cells in the same band:

3>		set the IE "Aggregated cells" in IE "Non-contiguous multi-cell" in the IE "UE radio access capability extension" to 'nc-3c' for the supported band;

3>	set the IE "Gap size" in IE "Non-contiguous multi-cell" in the IE "UE radio access capability extension" for the supported band to 'fiveMHz' or 'tenMHz' or 'anyGapSize', or

2>	if the UE supports non-contiguous multi-cell operation on four cells in the same band:

3>		set the IE "Aggregated cells" in IE "Non-contiguous multi-cell" in the IE "UE radio access capability extension" to 'nc-4c' for the supported band;

3>	set the IE "Gap size" in IE "Non-contiguous multi-cell" in the IE "UE radio access capability extension" for the supported band to 'fiveMHz' or 'tenMHz' or 'anyGapSize';

3>	if the UE supports an equal number of contiguous cells on each side of the gap, include the IE "Non-contiguous multi-cell Combination (2,2)";

3>	if the UE supports a different number of contiguous cells on each side of the gap, include the IE "Non-contiguous multi-cell Combination (3,1) (1,3)".

2>	if the UE supports Multiflow operation on two cells on one frequency:

3>	set the IE "Number of cells" in IE "Multiflow per band capability" to two cells;

3>	set the IE "Number of frequencies" in IE "Multiflow per band capability" to one frequency.

2>	if the UE supports Multiflow operation on three cells on two frequencies in the same band:

3>	set the IE "Number of cells" in IE "Multiflow per band capability" to three cells;

3>	set the IE "Number of frequencies" in IE "Multiflow per band capability" to two frequencies.

2>	if the UE supports Multiflow operation on four cells on two frequencies in the same band:

3>	set the IE "Number of cells" in IE "Multiflow per band capability" to four cells;

3>	set the IE "Number of frequencies" in IE "Multiflow per band capability" to two frequencies.

2>	if the UE supports Multiflow operation on four cells on three frequencies in the same band:

3>	set the IE "Number of cells" in IE "Multiflow per band capability" to four cells;

3>	set the IE "Number of frequencies" in IE "Multiflow per band capability" to three frequencies.

2>	if the UE support Multiflow operation on three or four non-contiguous cells in the same band:

3>	set the IE "Gap size" in IE "Multiflow per band capability".

2>	if the UE supports Multiflow operation with MIMO:

3>	set IE "MIMO support" in IE "Multiflow per band capability" to single or dual stream MIMO transmission in all the frequency bands where the Multiflow operation is supported;

3>	if the UE needs more processing to generate HARQ feedback for Multiflow with MIMO operation:

4>	set IE "Longer HARQ processing time" in IE "Multiflow capability".

2>	if the UE supports Multiflow operation on three or four cells on two frequencies in different bands:

3>	set the IE "Supported Multiflow Combination" in IE "Radio Access Capability Band Combination List";

3>	if the UE supports three cells on two frequencies in different bands:

4>	set the IE "Number of cells in band A" to two cells;

4>	set the IE "Number of cells in band B" to one cell;

4>	set the IE "Number of frequencies in band A" to one frequency;

4>	set the IE "Number of frequencies in band B" to one frequency.

3>	if the UE supports four cells on two frequencies in different bands:

4>	set the IE "Number of cells in band A" to two cells;

4>	set the IE "Number of cells in band B" to two cells;

4>	set the IE "Number of frequencies in band A" to one frequency;

4>	set the IE "Number of frequencies in band B" to one frequency.

3>	if the UE supports four cells on three frequencies in different bands:

4>	set the IE "Number of cells in band A" to three cells;

4>	set the IE "Number of cells in band B" to one cell;

4>	set the IE "Number of frequencies in band A" to two frequencies;

4>	set the IE "Number of frequencies in band B" to one frequency.

3>	if the UE supports MIMO with Multiflow in different bands:

4>	set IE "Support for Multiflow with MIMO operation in different bands" in IE "Multiflow capability".

2>	if the variable DEFERRED_MEASUREMENT_STATUS is TRUE:

3>	if System Information Block type 11 is scheduled on the BCCH and the UE has not read nor stored the IEs present in this System Information Block; or

3>	if System Information Block type 11bis is scheduled on the BCCH and the UE has not read nor stored the IEs present in this System Information Block; or

3>	if System Information Block type 11ter is scheduled on the BCCH and the UE has not read nor stored the IEs present in this System Information Block; or

3>	if System Information Block type 12 is scheduled on the BCCH and the UE has not read nor stored the IEs present in this System Information Block:

4>	include IE "Deferred measurement control reading".

2>	if an IE "Logged Measurement Info-FDD" or "Logged Measurement Info-TDD" in variable LOGGED_MEAS_REPORT_VARIABLE is present and registered PLMN is present in the IE "PLMN Identity List" stored in variable LOGGED_MEAS_REPORT_VARIABLE:

3>	include IE "Logged Meas Available".

2>	if an IE "Logged ANR Report Info" in variable LOG_ ANR_REPORT_VARIABLE is present and the registered PLMN is the same as one of the PLMNs in the IE "PLMN Identity" or IE "Equivalent PLMN Identity List" stored in variable LOG_ANR_REPORT_VARIABLE:

3>	include IE "ANR Logging Results Available".

2>	if an IE "Logged Connection Establishment Failure Info-FDD" or "Logged Connection Establishment Failure Info-TDD" in variable LOGGED_CONNECTION_ESTABLISHMENT_FAILURE is present and the current Registered PLMN is the same as the PLMN in IE "PLMN Identity" stored in variable LOGGED_CONNECTION_ESTABLISHMENT_FAILURE:

3>	include IE "Connection Establishment Failure Info Available".

NOTE:	If the "RRC State indicator" is set to the value "CELL_FACH", the UE continues to read and store the IEs in System Information Block type 11, System Information Block type 11bis, System Information Block type 11ter, System Information Block type 12, System Information Block type 18 and System Information Block type 19, if transmitted, after submitting the RRC Connection Setup Complete message to lower layers (see 8.5.31).

2>	if available, set the IE "DCN Identity" to the DCN value for the selected PLMN if the UE has a PLMN specific DCN-ID; else set the IE "DCN Identity" to the pre-provisioned default standardized DCN-ID [91].

When the RRC CONNECTION SETUP COMPLETE message has been submitted to lower layers for transmission the UE shall:

1>	if the UE has entered CELL_DCH state:

2>	clear the variable SYSTEM_INFORMATION_CONTAINER;

2>	if the IE "Deferred measurement control reading" was included in the RRC CONNECTION SETUP COMPLETE message:

3>	clear variable MEASUREMENT_IDENTITY;

3>	clear the variable CELL_INFO_LIST.

2>	if the UE supports RAN-assisted WLAN interworking:

3>	stop the timer T330, if it is running;

3>	stop using the information stored in the variable SYSTEM_INFO_WLAN_OFFLOAD_INFO, and forward this indication to the upper layers;

3>	if the variable WLAN_OFFLOAD_INFO is not empty:

4>	use the information stored in the variable WLAN_OFFLOAD_INFO as specified in [4], and forward it to the upper layers.

1>	if the UE has entered CELL_FACH state:

2>	if the variable SYSTEM_INFORMATION_CONTAINER is not empty:

3>	the UE behaviour is unspecified.

2>	start timer T305 using its initial value if periodical update has been configured by T305 in the IE "UE Timers and constants in connected mode" set to any other value than "infinity" in the variable TIMERS_AND_CONSTANTS;

2>	if the UE supports RAN-assisted WLAN interworking:

3>	stop the timer T330, if it is running;

3>	if the variable WLAN_OFFLOAD_INFO is not empty:

4>	use the information stored in the variable WLAN_OFFLOAD_INFO as specified in [4], and forward it to the upper layers.

3>	else:

4>	use the information stored in the variable SYSTEM_INFO_WLAN_OFFLOAD_INFO as specified in [4] and forward it to the upper layers.

1>	store the contents of the variable UE_CAPABILITY_REQUESTED in the variable UE_CAPABILITY_TRANSFERRED;

1>	initialise variables upon entering UTRA RRC connected mode as specified in subclause 13.4;

1>	consider the procedure to be successful.

And the procedure ends.

------------------------------------------

Skip to next change

------------------------------------------

### 10.2.41	RRC CONNECTION SETUP COMPLETE

This message confirms the establishment of the RRC Connection by the UE.

RLC-SAP: AM

Logical channel: DCCH

Direction: UE  UTRAN

| Information Element/Group name | Need | Multi | Type and reference | Semantics description | Version |
| --- | --- | --- | --- | --- | --- |
| Message Type | MP |  | Message Type |  |  |
| UE Information Elements |  |  |  |  |  |
| RRC transaction identifier | MP |  | RRC transaction identifier 10.3.3.36 |  |  |
| START list | MP | 1 to <maxCNdomains> |  | START [40] values for all CN domains. |  |
| >CN domain identity | MP |  | CN domain identity 10.3.1.1 |  |  |
| >START | MP |  | START 10.3.3.38 | START value to be used in this CN domain. |  |
| UE radio access capability | OP |  | UE radio access capability 10.3.3.42 |  |  |
| UE radio access capability extension | OP |  | UE radio access capability extension 10.3.3.42a |  |  |
| Other information elements |  |  |  |  |  |
| UE system specific capability | OP | 1 to <maxInterSysMessages> |  |  |  |
| >Inter-RAT UE radio access capability | MP |  | Inter-RAT UE radio access capability 10.3.8.7 |  |  |
| Deferred measurement control reading | OP |  | Enumerated (TRUE) | Indicates the UE has not read SIB11, SIB11bis, SIB11ter (if supported) and SIB12, if available. | REL-7 |
| Logged Meas Available | OP |  | Enumerated (TRUE) | Indicates the UE has logged measurements to report to the network | REL-10 |
| ANR Logging Results Available | OP |  | Enumerated (TRUE) | True indicates the UE has ANR logging results to report to the Network. | REL-10 |
| Connection Establishment Failure Info Available | OP |  | Enumerated (TRUE) | True indicates the UE has logged measurements from a RRC connection establishment failure to report to the network. | REL-11 |
| DCN Identity | OP |  | Octet string(2) |  | REL-14 |

------------------------------------------

Skip to next change

------------------------------------------

-- ***************************************************

--

-- RRC CONNECTION SETUP COMPLETE

--

-- ***************************************************

RRCConnectionSetupComplete ::= SEQUENCE {

-- TABULAR: Integrity protection shall not be performed on this message.

-- User equipment IEs

rrc-TransactionIdentifier		RRC-TransactionIdentifier,

startList						STARTList,

ue-RadioAccessCapability		UE-RadioAccessCapability			OPTIONAL,

--	Other IEs

ue-RATSpecificCapability		InterRAT-UE-RadioAccessCapabilityList	OPTIONAL,

-- Non critical extensions

v370NonCriticalExtensions			SEQUENCE {

rrcConnectionSetupComplete-v370ext	RRCConnectionSetupComplete-v370ext,

v380NonCriticalExtensions			SEQUENCE {

rrcConnectionSetupComplete-v380ext	RRCConnectionSetupComplete-v380ext-IEs,

-- Reserved for future non critical extension

v3a0NonCriticalExtensions			SEQUENCE {

rrcConnectionSetupComplete-v3a0ext	RRCConnectionSetupComplete-v3a0ext-IEs,

laterNonCriticalExtensions			SEQUENCE {

-- Container for additional R99 extensions

rrcConnectionSetupComplete-r3-add-ext		BIT STRING

(CONTAINING RRCConnectionSetupComplete-r3-add-ext-IEs)	OPTIONAL,

v3g0NonCriticalExtensions			SEQUENCE {

rrcConnectionSetupComplete-v3g0ext	RRCConnectionSetupComplete-v3g0ext-IEs,

v4b0NonCriticalExtensions			SEQUENCE {

rrcConnectionSetupComplete-v4b0ext

RRCConnectionSetupComplete-v4b0ext-IEs,

v590NonCriticalExtensions			SEQUENCE {

rrcConnectionSetupComplete-v590ext

RRCConnectionSetupComplete-v590ext-IEs,

v5c0NonCriticalExtensions			SEQUENCE {

rrcConnectionSetupComplete-v5c0ext

RRCConnectionSetupComplete-v5c0ext-IEs,

v690NonCriticalExtensions			SEQUENCE {

rrcConnectionSetupComplete-v690ext

RRCConnectionSetupComplete-v690ext-IEs,

v770NonCriticalExtensions		SEQUENCE {

rrcConectionSetupComplete-v770ext

RRCConnectionSetupComplete-v770ext-IEs,

va40NonCriticalExtensions		SEQUENCE {

rrcConectionSetupComplete-va40ext

RRCConnectionSetupComplete-va40ext-IEs,

vb50NonCriticalExtensions		SEQUENCE {

rrcConnectionSetupComplete-vb50ext

RRCConnectionSetupComplete-vb50ext-IEs,

vexyNonCriticalExtensions		SEQUENCE {

rrcConnectionSetupComplete-vexyext

RRCConnectionSetupComplete-vexyext-IEs,

nonCriticalExtensions	SEQUENCE {}	OPTIONAL

}		OPTIONAL

}		OPTIONAL

}		OPTIONAL

}		OPTIONAL

}		OPTIONAL

}		OPTIONAL

}		OPTIONAL

}		OPTIONAL

}		OPTIONAL

}		OPTIONAL

}		OPTIONAL

}		OPTIONAL

}		OPTIONAL

}

RRCConnectionSetupComplete-v370ext ::= SEQUENCE {

-- User equipment IEs

ue-RadioAccessCapability-v370ext	UE-RadioAccessCapability-v370ext	OPTIONAL

}

RRCConnectionSetupComplete-v380ext-IEs ::= SEQUENCE {

-- User equipment IEs

ue-RadioAccessCapability-v380ext	UE-RadioAccessCapability-v380ext		OPTIONAL,

dl-PhysChCapabilityFDD-v380ext		DL-PhysChCapabilityFDD-v380ext

}

RRCConnectionSetupComplete-v3a0ext-IEs ::= SEQUENCE {

-- User equipment IEs

ue-RadioAccessCapability-v3a0ext	UE-RadioAccessCapability-v3a0ext	OPTIONAL

}

RRCConnectionSetupComplete-v3g0ext-IEs ::= SEQUENCE {

-- User equipment IEs

ue-RadioAccessCapability-v3g0ext	UE-RadioAccessCapability-v3g0ext	OPTIONAL

}

RRCConnectionSetupComplete-r3-add-ext-IEs ::= SEQUENCE {

rrcConnectionSetupComplete-v650ext		RRCConnectionSetupComplete-v650ext-IEs	OPTIONAL,

v680NonCriticalExtensions				SEQUENCE {

rrcConnectionSetupComplete-v680ext		RRCConnectionSetupComplete-v680ext-IEs,

v7e0NonCriticalExtensions				SEQUENCE {

rrcConnectionSetupComplete-v7e0ext		RRCConnectionSetupComplete-v7e0ext-IEs,

v7f0NonCriticalExtensions				SEQUENCE {

rrcConnectionSetupComplete-v7f0ext

RRCConnectionSetupComplete-v7f0ext-IEs,

va40NonCriticalExtensions					SEQUENCE {

rrcConnectionSetupCompleteBand-va40ext

RRCConnectionSetupCompleteBand-va40ext-IEs,

nonCriticalExtensions			SEQUENCE {}	OPTIONAL

}		OPTIONAL

}		OPTIONAL

}		OPTIONAL

}		OPTIONAL

}

RRCConnectionSetupComplete-v4b0ext-IEs ::= SEQUENCE {

-- User equipment IEs

ue-RadioAccessCapability-v4b0ext		UE-RadioAccessCapability-v4b0ext		OPTIONAL

}

RRCConnectionSetupComplete-v590ext-IEs ::= SEQUENCE {

-- User equipment IEs

ue-RadioAccessCapability-v590ext		UE-RadioAccessCapability-v590ext		OPTIONAL,

-- Other IEs

ue-RATSpecificCapability-v590ext		InterRAT-UE-RadioAccessCapability-v590ext	OPTIONAL

}

RRCConnectionSetupComplete-v5c0ext-IEs ::= SEQUENCE {

-- User equipment IEs

ue-RadioAccessCapability-v5c0ext		UE-RadioAccessCapability-v5c0ext		OPTIONAL

}

RRCConnectionSetupComplete-v650ext-IEs ::= SEQUENCE {

-- User equipment IEs

ue-RadioAccessCapability-v650ext		UE-RadioAccessCapability-v650ext

}

RRCConnectionSetupComplete-v680ext-IEs ::= SEQUENCE {

-- User equipment IEs

ue-RadioAccessCapability-v680ext		UE-RadioAccessCapability-v680ext

}

RRCConnectionSetupComplete-v690ext-IEs ::= SEQUENCE {

-- User equipment IEs

ueCapabilityContainer				BIT STRING

(CONTAINING UE-CapabilityContainer-IEs)	OPTIONAL

}

RRCConnectionSetupComplete-v770ext-IEs ::= SEQUENCE {

-- Other IEs

deferredMeasurementControlReading	ENUMERATED { true }						OPTIONAL

}

RRCConnectionSetupComplete-v7e0ext-IEs ::= SEQUENCE {

-- User equipment IEs

ue-RadioAccessCapability				UE-RadioAccessCapability-v7e0ext

}

RRCConnectionSetupComplete-v7f0ext-IEs ::= SEQUENCE {

-- User equipment IEs

ue-RadioAccessCapability				UE-RadioAccessCapability-v7f0ext	OPTIONAL

}

RRCConnectionSetupComplete-va40ext-IEs ::= SEQUENCE {

-- Other IEs

loggedMeasAvailable							ENUMERATED { true }					OPTIONAL,

loggedANRResultsAvailable					ENUMERATED { true }					OPTIONAL}

RRCConnectionSetupCompleteBand-va40ext-IEs ::= SEQUENCE {

-- User equipment IEs

ue-RadioAccessCapabBand					UE-RadioAccessCapabBand-va40ext		OPTIONAL

}

RRCConnectionSetupComplete-vb50ext-IEs ::= SEQUENCE {

-- Other IEs

connEstFailInfoAvailable			ENUMERATED { true }	OPTIONAL

}

RRCConnectionSetupComplete-vexyext-IEs ::= SEQUENCE {

-- Other IEs

dcnIdentity							OCTET STRING (SIZE (2))			OPTIONAL

}