3GPP TSG-RAN WG2 Meeting #96	R2-168119

Reno, USA, 14 - 18 November 2016

Agenda item:	9.2.2.3

Source:	Nokia, Alcatel-Lucent Shanghai Bell

Title:	Secondary RLM aspects for LTE-NR tight interworking

WID/SID:	FS_ NR_newRAT - Release 14

Document for:	Discussion and Decision

# 1	Introduction

In this contribution, we discuss radio link management aspects for the LTE/NR tight interworking scenarios. We make a few observations and proposals to start discussions on this topic. A set of guidelines are also proposed for the running TR.

The three deployment scenarios that have been agreed to be studied as part of LTE-NR tight interworking are shown in Figure 1 below. These scenarios correspond to the non-standalone scenarios captured in RP-161266 [1] as scenarios 3, 4 and 7 respectively:

Figure 1: LTE-NR tight interworking deployment scenarios (RP-161266)

# 2	Radio link management aspects in LTE-DC

The radio link failure is dealt with the following behaviour in LTE.

Figure 1: The phases governing the behaviour associated with RLF ([1])

2 phases are defined in the specification; a 1st phase and a 2nd phase. Re-establishment of the RRC connection is triggered in 2nd phase so that a UE could reconnect to the network as soon as possible and resume services. In LTE- DC, the PCell supports above phases. In addition, the first phase of the radio link failure procedure is supported for PSCell. However, upon detecting RLF on the PSCell, the re-establishment procedure is not triggered at the end of the first phase. Instead, UE informs the radio link failure of PSCell to the MeNB.

A UE experiencing RLF failure of the PSCell triggers the SCG Failure Information procedure with the MeNB. It is then up to the MeNB to allocate a SeNB to the UE [2] and this involves X2 and Uu interface procedure similar to initial configuration of SeNB.

# 3	RLF handling principles in LTE/NR tight interworking

Based on the description in section 2, we make a few observations about RLF handling for LTE-NR tight interworking.

Observation 1: There is a need to understand how the radio link monitoring process (i.e. synchronization and quality determination) is performed in the NR.

Proposal 1: SCG radio link failure criteria and reporting will be described in the NR RRC specification.

Proposal 2: The MeNB needs to receive the indication of SCG RLF.

# 4	Conclusion

In this paper we have discussed RLF handling principles for LTE/NR tight interworking and made the following observation:

Observation 1: There is a need to understand how the radio link monitoring process (i.e. synchronization and quality determination) is performed in the NR.

We make the following proposals:

Proposal 1: SCG radio link failure criteria and reporting will be described in the NR RRC specification.

Proposal 2: The MeNB needs to receive the indication of SCG RLF.

# References

- 3GPP TS 36.300: Evolved Universal Terrestrial Radio Access (E-UTRA)  and Evolved Universal Terrestrial Radio Access Network (E-UTRAN); Overall description, Stage 2

- 3GPP TS 36.331: Evolved Universal Terrestrial Radio Access (E-UTRA); Radio Resource Control (RRC); Protocol specification

# Appendix: Text Proposal for TR 38.804

Beginning of the Text Proposal

## 4.2	Guidelines

For both control plane and user plane protocols:

-	NR Radio protocols and procedures should be designed to have as much commonality as possible between tight interworking with LTE and standalone operations.

-	Most essential functions (e.g., initial system access) should be future proof and designed to be common to various different use cases and services.

-	LTE layer 2 and RRC functions are taken as a baseline for NR.

In terms of intra-NR mobility:

-	Two types of UE states are taken as a baseline; one is network controlled mobility and the other is UE based mobility.

-	For typical inter-gNB network controlled mobility, the information provided in measurement configuration required for the UE to perform measurements should be minimised (e.g., avoid the need to provide detailed cell/beam level information). More detailed information may be provided to address some cases.

-	UE context transfer should be minimised as a consequence of UE based mobility.

In terms of URLLC:

-	Study will not focus on high availability as in node, hardware/software, transport link availability, and instead the focus should be on coverage, mobility, radio link features etc. related to providing low latency and/or high reliability.

In terms of system information delivery:

-	System information distribution should target a single technical framework, ensuring future proofness and smooth introduction of new services and features.

-	System information distribution should consider performance aspects like accessibility and state transition latency.

-	System information distribution should enable a high level of configurability enabling optimization of KPIs such as energy savings and accessibility.

-	System information distribution should include fast and efficient mechanisms for handling of system information change.

-	System information distribution should explore and leverage the fact that parts of the system information may be the same across a large area, such as the parts associated to system access (e.g. RACH configuration during state transitions).

-	System information distribution in NR should be designed such that UEs supporting less than the carrier bandwidth can determine at least the minimum system information.

-	System information broadcast should allow configurations that enable network energy efficiency (e.g. by long DTX duration).

In terms of LTE NR tight interworking:

- SCG radio link failure criteria and reporting will be described in the NR RRC specification.

- The master node needs to receive the indication of secondary node RLF.

End of the Text Proposa