3GPP TSG-RAN WG2 #97                                                                                   	          R2-1701706

Athens, Greece, 13 - 17 February 2017

Agenda Item:	10.2.1.4

Souce:	MediaTek Inc.

Title:	DRX enhancement for NR

Document for:		Discussion and decision

# 1 Introduction

This document is a resubmission of R2-1700228 (which is a resubmission of [1]).

In RAN2#95BIS, the topic of DRX enhancement for NR was discussed [2] and the following agreement was reached.

Agreements

1. 	DRX enhancement is studied in NR in order to support multiple services with different requirements and/or numerologies.

In this contribution, we provide our views on this topic.

# 2 Discussion

In [2], it is proposed that “service-specific DRX would be one possible approach, where DRX is configured per service while the UE monitors PDCCH in On Duration of any of the DRX pattern, i.e., “union approach”. Moreover, it is suggested in [2], that asymmetric activity in downlink and uplink also needs to be considered when considering DRX enhancement in NR.

It is true that different services and applications have different performance requirements (e.g., latency and data rate). However, it is not clear that service-specific DRX configuration is the solution. Since the gNB is aware of the services that a UE supports at any time, as well as the performance requirements of such services, a simpler approach would be for the gNB to determine the desired DRX cycle. For example, the gNB could use the “union approach” to On Duration as mentioned in [2].

Observation 1: The gNB can determine the desired DRX cycle and parameters based on the services and/or numerologies supported by the UE at any point of time.

Accordingly, we do not see the need to deviate from the LTE baseline for DRX configuration in NR. Of course, it is still possible to consider DRX enhancements (e.g., to support high frequency scenarios, different numerologies and services) but there does not seem to a strong reason to make the DRX configuration service specific.

Proposal 1: DRX configuration in NR should be UE-specific.

# 3 Conclusions

In this contribution, we discuss the issue of DRX configuration for NR. Our proposal and observation are summarized below.

Observation 1: The gNB can determine the desired DRX cycle and parameters based on the services and/or numerologies supported by the UE at any point of time.

Proposal 1: DRX configuration in NR should be UE-specific.

# 4 References

- R2-168590, DRX enhancement for NR, MediaTek Inc.

- R2-166647, DRX enhancement in NR, LG Electronics Inc.