3GPP TSG-RAN WG2 Meeting #97	R2-1701790

# Athens, Greece, 13th - 17th February 2017

| CR-Form-v11.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.304 | CR | 0355 | rev | - | Current version: | 13.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Clarifications on reselection for eMTC |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Sequans Communications |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R2 |  |  |  |  |  |  |  |  |  |
| Work item code: | LTE_MTCe2_L1-Core |  |  |  |  |  | Date: |  |  | 2017-02-03 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-13 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) |  |
| Reason for change: |  | - The expression “Ranking with cell selection criterion S for enhanced coverage is applied” in 5.2.4.6a is unclear and does not seem correct. It seems there are 2 possible interpretations A or B: A) Perform ranking as in 5.2.4.6 but use the “cell selection criterion S for enhanced coverage” instead of the “cell-ranking criterion Rs” - Issue A1: “cell selection criterion S for enhanced coverage” is a condition which can be fullfilled or not, and not a metric. - Issue A2: cells on frequency layers for which CE is not supported does not have Qrxlevmin_CE and “cell selection criterion S for enhanced coverage” does not exist for them. B) Perform ranking as in 5.2.4.6 but while doing so, only cells that fulfil cell selection criterion S for enhanced coverage are considered, i.e., the text “The UE shall perform ranking of all cells that fulfil the cell selection criterion S” should be read as “The UE shall perform ranking of all cells that fulfil the cell selection criterion S for enhanced coverage” - Issue B1: it could be considered that a cell may fulfil the cell selection criterion S for enhanced coverage only when it does not fulfil the cell selection criterion S, because in 5.2.3.2 the “cell selection criterion S for enhanced coverage” is only defined when “cell selection criterion S in normal coverage is not fulfilled”. - Issue B2: same as issue A2 above. In both cases A or B, there is a risk that some NC cells (cells on which cell selection criterion S in normal coverage is fulfilled) are not considered in the ranking process (particularly if they do not support CE), which would prevent EC->NC reselection. Our understanding is that the intention is to perform ranking according to 5.2.4.6 among all cells fullfiling “cell selection criterion S” (normal or enhanced coverage). In 5.2.4.6 the text is “The UE shall perform ranking of all cells that fulfil the cell selection criterion S” which in our view already encompasses both cases. - The condition “when the current serving cell can only be accessed using enhanced coverage” in 5.2.4.6a is unclear as it can be understood in 2 ways: A) The cell can only be accessed using CE mode (A or B). This means BL UEs would always use ranking based reselection. B) The cell can only be accessed while being in enhanced coverage (i.e. cell selection criterion S in normal coverage not fulfilled, but cell selection criterion S for enhanced coverage fulfilled, as defined in 5.2.3.2). This means BL UEs use priority based reselection when in NC and ranking based reselection while in EC. Our understanding from previous discussions is that the intended behaviour is B). This would also be in line with the RAN2#96 agreements related to SC-PTM mobility, which builds up on top of Rel-13 reselection:  For feMTC, SC-PTM should be considered in reselection priorities handling for BL UEs for normal suitability criterion is met.  For feMTC, Rel-13 mechanism of reselection priorities handling for SC-PTM should be reused for the above case.  For feMTC, for UE in enhanced coverage, an offset to SC-PTM cells in ranking based cell reselection is used if SC-PTM cell exists and UE is receiving or interested to receive an MBMS service.  For NB-IoT, an offset to SC-PTM cells in ranking based cell reselection is used if SC-PTM cell exists and UE is receiving or interested to receive an MBMS service. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Clarify how ranking is applied in 5.2.4.6a, by removing “with cell selection criterion S for enhanced coverage”, replacing by “as defined in sub-clause 5.2.4.6” (which is used at other places when referring to ranking procedure). - Clarify that the understanding B) is the intended one. Impact analysis Impacted functionality: Cell reselection under coverage enhancement. If the network is implemented according to the CR, but the UE is not: No inter-operability issue. This CR changes UE behaviour only. If the UE is implemented according to the CR, but the network is not: No inter-operability issue. This CR changes UE behaviour only. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Cell reselection may not be performed as expected. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.2.4.6a |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

#### 5.2.4.6a	Reselection for enhanced coverage

Ranking as defined in sub-clause 5.2.4.6is applied for intra-frequency and inter-frequency cell reselection when the current serving cell can only be accessed while being in enhanced coverage, as defined in sub-clause 5.2.3.2.