3GPP TSG-RAN WG2 #97bis	Tdoc R2-1702663

Spokane, USA, 3rd – 7th April 2017

Agenda Item:	10.3.1.1

Source:	Ericsson

Title:	Overview of MAC specification for NR

Document for:	Discussion, Decision

# Introduction

In this contribution we discuss what parts of the LTE MAC specification can be reused in the NR MAC specification. As LTE currently has a wider scope than NR some parts of the LTE MAC specification do not need to be included in the NR MAC specification, at least not from the start.

# Discussion

In the TR (3GPP TR 38.804) the following statement is included as a guideline:

-	LTE layer 2 and RRC functions are taken as a baseline for NR.

If we apply this to the NR MAC we think we can use the LTE MAC specification as the NR MAC specification with some modifications.

TS 38.321 can be based on 36.321 with some modifications.

The current scope of LTE is larger than the current scope for NR. Some functions in LTE will not be included in the first NR release. Hence, the corresponding functions in the LTE MAC specification do not need to be included in the NR MAC specification. These functions include:

-	MBMS (sections 5.12, 6.1.3.7, 6.1.3.7a)

-	Sidelink operation (sections 5.14, 5.15, 5.16, 6.1.3.1a, 6.1.6, 6.2.4)

There are agreed differences to LTE which will impact some sections:

-	Synchronous HARQ is no longer supported (section 5.4.2)

-	RLC concatenation is no longer supported (section 6.1.2)

Then there are some LTE features not carried over to NR which have impact on smaller level than whole sections, such as:

-	Network relays

-	eMTC and NB-IoT

-	LAA

This leads to the following proposals.

The sections in 36.321 which address MBMS (sections 5.12, 6.1.3.7, 6.1.3.7a) are not included in 38.321.

The sections in 36.321 which address Sidelink operation (sections 5.14, 5.15, 5.16, 6.1.3.1a, 6.1.6, 6.2.4) are not included in 38.321.

Section 5.4.2 and its subsections in 36.321 need to be updated before being included in 38.321 to consider that synchronous HARQ is no longer supported.

Section 6.1.2 in 36.321 needs to be updated before being included in 38.321 to consider that RLC concatenation is no longer supported.

Before adding any text from 36.321 to 38.321 it should be considered that Network relays, eMTC, NB-IoT, and LAA is no longer supported.

# Conclusion

Based on the discussion in section 2 we propose the following:

Proposal 1	TS 38.321 can be based on 36.321 with some modifications.

Proposal 2	The sections in 36.321 which address MBMS (sections 5.12, 6.1.3.7, 6.1.3.7a) are not included in 38.321.

Proposal 3	The sections in 36.321 which address Sidelink operation (sections 5.14, 5.15, 5.16, 6.1.3.1a, 6.1.6, 6.2.4) are not included in 38.321.

Proposal 4	Section 5.4.2 and its subsections in 36.321 need to be updated before being included in 38.321 to consider that synchronous HARQ is no longer supported.

Proposal 5	Section 6.1.2 in 36.321 needs to be updated before being included in 38.321 to consider that RLC concatenation is no longer supported.

Proposal 6	Before adding any text from 36.321 to 38.321 it should be considered that Network relays, eMTC, NB-IoT, and LAA is no longer supported.