3GPP TSG-RAN WG2 NR	Tdoc R2-1702665

Spokane, USA, 3rd – 7th April 2017

Agenda Item:	10.3.1.6

Source:	Ericsson

Title:	UL HARQ considerations for intra-UE punctured eMBB

Document for:	Discussion

# Introduction

URLLC (Ultra-Reliable Low Latency Communication) is a new data service with extremely strict error and latency requirements as described in [1].

In addition, RAN1 agreed to support puncturing eMBB data to transmit URLLC data in the punctured resources and the agreements are as below:

- Dynamic resource sharing between URLLC and eMBB is supported by transmitting URLLC scheduled traffic

  - URLLC transmission may occur in resources scheduled for ongoing eMBB traffic

The possibilities and impact of URLLC puncturing of eMBB UL data are discussed in [2],[3], and [4]. With UL puncturing, an ongoing (eMBB) transmission of slot length may be interrupted (punctured) by a shorter mini-slot (URLLC) transmission, which then overlaps in time. The punctured and puncturing transmission can be directed to the same or different UEs;

- Intra-UE puncturing: Both the shorter mini-slot and slot level tranmissions are transmitted to the same UE [4].

- Inter-UE puncturing: The data in shorter mini-slot transmission is transmitted to a different UE than the data transmitted in slot length transmission [5].

An ongoing transmission of slot length, e.g. eMBB, may be interrupted (punctured) by a shorter mini-slot transmission, e.g. URLLC.

In this paper we assume the scenario is that an eMBB transmission is punctured by a URLLC transmission, but the solutions apply to any service. The focus of this contribution is to clarify the HARQ procedures for the punctured eMBB data in UL.

# Discussion

## Intra-UE UL puncturing

Figure 1 depicts the intra-UE puncturing in the uplink, where the slot length data i.e eMBB might be punctured by a short mini-slot i.e. URLLC data.

The UE can store information about which parts of the eMBB transmission that were punctured. This could for example be a range of bits defined through the first bit and the last bit of the punctured range.

The UE can keep the information of punctured bits (part of the packet) of the eMBB transmission that was punctured.

Figure 1: Illustration of intra-UE puncturing i.e. URLLC transmission in punctured eMBB data in uplink

## HARQ Considerations for intra-UE UL puncturing

Right after the punctured transmission in the uplink, the UE is able to estimate the probability that the eMBB data cannot be successfully decoded by the gNB, due to the puncturing. If the probability of decoding failure due to puncturing is above a specified threshold, the UE should retransmit the eMBB data or only the corrupted part of the eMBB.

The UE could perform the retransmission of the corrupted punctured part of eMBB data in two different ways:

- UE receives a grant with UL resources from gNB (or a NACK )

- UE utilizes configured uplink resources (SPS UL) and retransmits the punctured part.

If there are no configured uplink resources available (alternative 1 above) for the UE, the gNB schedules uplink resources to allow the UE to retransmit the eMBB data (or optionally only the corrupted part of eMBB data), as illustrated in Figure 2.

If there are configured (SPS) uplink resources available for the UE that sent the eMBB and URLLC, the gNB can wait to receive the UE’s retransmission of the eMBB data (or only the corrupted part), instead of sending a NACK and transmitting a scheduling grant for the retransmission. This is shown in Figure 3. Note that at this point, the UE’s ID is known to the gNB and the gNB knows whether there are any configured UL resources available for the UE to perform SPS UL transmissions.

- Configuration of SPS resources for immediate HARQ retransmissions in the uplink (e.g. in case of puncturing) is supported without awaiting feedback.

Figure 2: UL HARQ retransmission where the gNB schedules UL resources only for retransmission of the corrupted punctured eMBB bits.

Figure 3:UL HARQ retransmission where semi-statically scheduled UL resources are utilized to retransmit the punctured eMBB data

Compared to LTE, the UL HARQ retransmission without NACK could save significant delay, illustrated with tsaved in Figure 3. Here, SPS UL resources are utilized to retransmit the punctured eMBB data.

# Conclusion

In section 2 we made the following observations:

Observation 1	An ongoing transmission of slot length, e.g. eMBB, may be interrupted (punctured) by a shorter mini-slot transmission, e.g. URLLC.

Observation 2	The UE can keep the information of punctured bits (part of the packet) of the eMBB transmission that was punctured.

Based on the discussion above, we propose the following:

Proposal 1	Configuration of SPS resources for immediate HARQ retransmissions in the uplink (e.g. in case of puncturing) is supported without awaiting feedback.

# References

R2-1700393, Overview of URLLC, Ericsson

R1-1701870, On eMBB/URLLC multiplexing for uplink, Ericsson

R1-1706056, On intra-node puncturing in UL, Ericsson

R1-1706057, On inter-node puncturing in UL, Ericsson