3GPP TSG-RAN WG2 #97bis	Tdoc R2-1702861

Spokane, USA, 3rd – 7th April 2017

Agenda Item:	10.4.1.4

Source:	Ericsson

Title:	Change of System information in NR

Document for:	Discussion, Decision

# Introduction

At the RAN2#97 meeting the following was agreed in relation to stored system information in NR:

Agreements

1: Broadcasting some kind of index/identifier in minimum SI to enable the UE to avoid re-acquisition of already stored SI-block(s)/SI message(s). The index/identifier and associated system information can be applicable in more than one cell. System information valid in one cell may be valid also in other cells.

FFS what the index/identifier is (e.g. single index or area plus value tag, etc)

In this paper we discuss and try to further address the possibility to introduce a valueTag similar to what is specified in LTE

# Discussion

In LTE, the SIB1 (or MIB for NB-IoT) includes a value tag (systemInfoValueTag) to allow UE’s to use stored system information. When reading system information, the UE may store and associate it with the value tag that is also broadcast. Later, e.g., upon return from out of coverage, the UE can check the value tag and if it has stored information related to the value tag broadcast, it can assume that no new reading of system information is necessary. The UE will however assume that stored system information becomes invalid after 3 hours.

In a similar manner like for LTE, in another contribution [1] we argue that a valueTag should be introduced in the NR-MIB to allow for a similar handling of stored SI. In case the UE receives an SS Block and decodes a valueTag (provided in the MIB) and it discovers that it already has a stored valid version of the SI corresponding to that valueTag, it does not have to read the system information again.

## How many value tags are needed?

The amount of value tags needed would be dependent on how many different sets of system information one can envision for the duration of a system information validity (in LTE, 3 hours as default).

There is a dependency between the system information validity time for valueTags and how frequent it is feasible to successfully update the system information while at the same time minimizing the risk that UE’s use old (invalid) stored information. As discussed in [1] the set of valueTags may also be applicable in more than one cell. Even though the system information may not be particularly dynamic today, it would be an unnecessary limitation to not allow for frequent updates due to a limitation in the valueTag range. At the same time, as the valueTags are assumed to be part of the MIB, it is important to not use more bits than necessary.

The valueTag range can limit how often it is feasible to update system information

It is not known today how frequent updates of system information that will be needed

To allow for a more frequent update of system information than what a limited set of equally valid valueTags would support, it makes sense to consider specifying at least a few valueTag values that have a much shorter validity time. This way, when such short-time-valid valueTags are broadcast in the MIB, system information associated with those valueTags will not be stored for long by the UE’s, allowing for more frequent updates. Then the valueTag range will not as quickly become a limitation, if, e.g., updates are required more times than what is allowed with a single and quite long validity time. We think that a limited set of valueTag values should be associated with such shorter validity times in addition to valueTag values with a longer validity time. It should be agreed how long such shorter validity times should be, for example, e.g., 1-10 minutes.

NR should support different validity durations for different value tag values to support both frequent changes as well as more static and preconfigured sets of system information.

It is for further discussion whether this variation in validity duration is realized through specific times per valueTag or with an additional parameter, similar to si-ValidityTime in LTE.

# Conclusion

In section 2 we made the following observations:

Observation 1	The valueTag range can limit how often it is feasible to update system information

Observation 2	It is not known today how frequent updates of system information that will be needed

Based on the discussion in section 2 we propose the following:

Proposal 1	NR should support different validity durations for different value tag values to support both frequent changes as well as more static and preconfigured sets of system information.

# References

R2-1702858, Stored system information, Ericsson, 3GPP RAN2#97bis, Spokane, USA, April 3-7, 2017