3GPP TSG-RAN WG2 Meeting #97bis	R2-1703485

Spokane, USA, 3rd – 7th April 2017	Revision of R2-1701540

Agenda Item	: 10.3.1.7 (NR_newRAT-Core)

Source	: LG Electronics Inc.

Title	: Numerology impact on DRX

Document for	: Discussion and Decision

1.	Introduction

In e-mail discussion [96#28], RAN2 discussed numerology impact on MAC in terms of numerology configuration, HARQ, LCP, DRX, BSR/SR, and PHR. The agreements as a result of the e-mail discussion is captured as below:

Agreements

1: a single logical channel can be mapped to one or more numerology/TTI duration.

2: ARQ can be performed on any numerologies/TTI lengths that the LCH is mapped to.

3: The RLC configuration is per logical channel without dependency on numerology/TTI length.

4: Logical channel to numerology/TTI length mapping can be reconfigured via RRC reconfiguration.

5: RAN2 will leave RAN1 to decide whether HARQ retransmission can be performed across different numerologies and/or TTI durations.

6: wait for more details from RAN1 to decide whether HARQ configuration, if any, needs to be numerology/TTI duration specific.

7: a single MAC entity can support one or more numerology/TTI durations.

8: LCP takes into account the mapping of logical channel to one or more numerology/TTI duration. Details of LCP will be discussed in the WI phase

In this contribution, we discuss some further aspects related to DRX supporting different numerologies.

2.	Discussion

There are several DRX timers, which are summarized in the Table below:

Table 1 DRX timers in MAC

| Timer Type | Value in number of… | Timers |
| --- | --- | --- |
| Type 1 | PDCCH-subframe | onDurationTimer drx-InactivityTimer drx-RetransmissionTimer drx-ULRetransmissionTime |
| Type 2 | subframe | HARQ RTT Timer UL HARQ RTT Timer ongDRX-CycleStartOffset shortDRX-Cycle |
| Type 3 | shortDRX-Cycle | drxShortCycleTimer |

In LTE, PDCCH-subframe refers to a subframe with PDCCH. Although PDCCH are spread within or over the subframe(s) when multiple numerologies/TTI durations are used, PDCCH-subframe can still refer to a subframe with PDCCH and be used for PDCCH monitoring.

With this understanding, the next question is how to calculate the DRX timers for differnet numerologies/TTI durations?

Type 1 DRX timers counting PDCCH-subframes

The intention of setting the DRX timers in number of PDCCH subframes is to offer a specific number of scheduling opportunity, i.e., number of PDCCH. Therefore, there could be two approaches in calculating these timers:

- Option 1. Count the number of PDCCH-subframe regardless of numerology/TTI durations.

- Option 2. Count the number of PDCCH opportunity by considering numerology/TTI duration.

For better understanding, let’s practice by assuming that DRX timer is set to psf4, and scheduling can be done based on 1 slot of 0.5ms. In Option 1, a DRX timer counts 4 PDCCH subframes, whereas in Option 2, a DRX timer counts 4 slots, i.e., 2 PDCCH subframes.

In Option 1, if the scheduling is based on 1 slot of 0.5ms TTI duration, the UE will have more scheduling opportunities during the Active Time, i.e., 8 PDCCH opportunities, compared to Option 2, i.e., 4 PDCCH opportunities. Considering that the intention of short TTI is to offer more scheduling opportunities, Option 1 would be acceptable even for the scheduling with short TTI.

On the other hand, the intention of short TTI is to reduce latency in scheduling, hence, it would also make sense only to offer the same scheduling opportunities by Option 2 if the scheduling is based on short TTI. However, if the scheduling is actually done based on 1 subframe of 1 ms, Option 2 may take scheduling opportunities.

As each DRX timer plays a different role in PDCCH monitoring, different option may need to be applied to each DRX timer.

For example, drx-InactivityTimer and onDurationTimer could keep counting the number of PDCCH subframes (Option 1) because those timers are defined per UE and should guarantee sufficient scheduling opportunities when multiple numerologies are used simultaneously.

Proposal 1. For drx-InactivityTimer and OnDurationTimer, the UE counts PDCCH-subframe.

In the meanwhile, drx-RetransmissionTimer and drx-ULRetransmissionTimer may need to count the number of PDCCH opportunities (Option 2) because those timers are defined per HARQ process and depend on the TTI length that is to be used for scheduling of retransmission.

On the other hand, in case TTI length switching between new transmission and retransmission is supported, the situation is similar to drx-InactivityTimer and onDurationTimer. In other words, the UE should be ready to receive scheduling with any TTI length. Thus, drx-RetransmissionTimer and drx-ULRetransmissionTimer need to count the number of PDCCH-subframe as in the legacy.

In RAN1, however, it is not yet decide how the scheduling could be for new transmission and retransmission with multiple numerologies. Thus, it seems that RAN2 cannot decide how to count the retransmission related timers now can discuss only after the progress in RAN1.

Proposal 2. For drx-RetransmissionTimer and drx-ULRetransmissionTimer, RAN2 discuss how to count the timer after RAN1 decides how to schedule new transmission and retransmission with multiple numerologies.

Type 2 DRX timers counting subframes

Among Type 2 DRX timers, HARQ RTT Timer and UL HARQ RTT Timer are to allow UE not to monitor the PDCCH by considering the processing time in network side, i.e., not TTI duration.

Given that processing time reduction and TTI reduction are separate issue, it can be considered that these timers can still count the same number of subframes as in LTE regardless of numerology/TTI duration. On the other hand, it would be reasonable to assume that processing time would be also reduced if short TTI duration is used. Then, these timer may need to count less number of subframes by considering the reduced processing time. Note that it doesn’t mean that the legnth of subframe is scaled down.

Given that processing time reduction is not necessarily linearly dependent to TTI duration reduction, there could be two options:

- Option 1. Count the same number of subframe as in LTE.

- Option 2. Define another value of UL HARQ RTT Timer and HARQ RTT Timer for reduced processing time.

Note that WI on Shortened TTI and processing time, it was decided in RAN2 that two values of HARQ RTT Timers in UL and DL is used for reduced processing time. However, this can also be discussed only after RAN1 decides more details about reduced processing time.

Proposal 2. For HARQ RTT Timers, RAN2 discuss how to count the timer after RAN1 decides processing time.

3.	Conclusion

In this contribution, we discuss the numerology impact on DRX, and propose the followings:

Proposal 1. RAN2 discuss whether Type 1 DRX timer counts PDCCH-subframes or PDCCH opportunities.

Proposal 2. RAN2 discuss whether to define another timer values for UL HARQ RTT Timer and HARQ RTT Timer by considering reduced processing time.