3GPP TSG-RAN WG2 #98	Tdoc R2-1704107

Hangzhou, P.R. of China, 15th – 19th May 2017

Agenda Item:	10.2.7

Source:	Ericsson

Title:	Neighbour relation establishment in NR

Document for:	Discussion, Decision

# Introduction

Since the first release of LTE, Automatic Neighbour Relation (ANR) functionality has been introduced to automatically generate the relations between radio network entities. Such relations are used to establish connections between entities, support mobility, load balancing, dual connectivity, etc., and ANR is seen as significantly reducing the planning and operation costs for operators [2]. Therefore, it is our understanding that automatic establishment of relations will continue to be an important feature in NR networks. One can expect that the challenging NR requirements will result in an even higher emphasis on the need for seamless mobility than in legacy RATs, which further increases the importance of automatic relation establishment in NR. In addition, an extensive use of beamforming, together with a lean design of NR will bring new challenges and additional scope to the relation management.

- Detection of neighbor cell for measurement is based on NR-SS.

In RAN1#88, the following was agreed:

In this paper, further details of how the UE could aid the neighbour relation establishment process is provided and scenarios wherein the UE fails to identify a neighbouring cell is provided.

# Discussion

## Usage of identifiers to support IDLE RS based RRC based mobility and neighbour relation establishment

In LTE, a UE detects cells based on their PSS/SSS, which encode a Physical Cell Identifier (PCI). An important property of these signals is that the UE autonomously detects the neighbour cell IDs from an acquired PSS/SSS, i.e., the network does not need to provide a neighbour cell list. UEs typically detect and measure neighbour cells by sampling a short time window (e.g. 5 ms) on the target frequency (which may be the same or different from serving) and search (possibly offline) for PSS/SSS occurrences within that sample. For each detected PSS/SSS, the UE can also perform a measurement using the CRS corresponding to the PCI. The result of that action is a list of neighbour cell IDs and corresponding measurement sample.

Once the UE perform measurements, mobility events can trigger the transmission of measurement reports. Based on these reports the serving eNB is able to identify that a given PCI belongs to a neighbour eNB in order to trigger a handover preparation procedure. In order to do that, the serving eNB needs to maintain a Neighbour Relation Table that maps locally unique identifiers, i.e. the PCIs in LTE, with globally unique identifiers, i.e. the E-UTRAN Cell Global identifier (E-CGI). E-CGI is defined in LTE as a combination of PLMN ID (Combination of country code and network code) and ECI (combination of eNB ID and cell ID).

The procedure is illustrated in Figure 1. In step 1, the UE sends a measurement report to the serving eNB, containing the PCI of cell B. The eNB then instructs the UE to read the ECGI, the TAC and all available PLMN ID(s) of cell B. The neighbour cell broadcasts its PLMN IDs, TAC etc. in the SIB1 message. After measuring these values, the UE reports the detected ECGI to the serving eNB, and the eNB decides to add this neighbour relation. The eNB can use the PCI and ECGI to lookup a transport layer address to the new eNB, update the Neighbour Relation list, and, if needed, setup a new X2 interface towards this eNB [1].

Figure 1. Automatic Neighbour Relation Function in LTE [1]

- In LTE, the UE reports to the serving eNB measurement reports associated to locally unique identifiers (e.g. PCIs).

- For handover decisions based on UE reporting of local IDs, a Neighbour Relation Table (NRT) mapping local IDs to global IDs (e.g. E-CGI) is necessary to identify the target eNB in order to send a handover preparation and/or a handover request.

- In LTE, a serving eNB can request the UE to acquire a global identifier (e.g. ECGI) associated to a previously reported local identifier (PCI).

- In LTE, a globally unique identifier (ECGI) is broadcasted in SIB1 and can be associated to a locally unique identifier (PCI).

In LTE, this Global ID, the ECGI, is not necessary as often as the PCI. Therefore, it is transmitted in SIB1 every 80 ms while the PCI is transmitted every 5ms.

- In LTE, a globally unique identifier (ECGI) is broadcasted with a larger periodicity than the locally unique identifier (PCI).

In RAN2#97 NR meeting in Athens, it was noted that an RRC_CONNECTED UE should be able to perform RRM measurements on IDLE RS..

An RRC_CONNECTED UE should be able to perform RRM measurements on always on IDLE RS (e.g. synchronization signal).

Though not already decided, the RRM measurements on IDLE RSs can be expected to be reported against the corresponding PCI values that the UE has measured. Based on such an RRM measurement report, if the serving cell do not find the reported PCI in its neighbour relation list, then the serving cell could reuse the ECGI measurement and reporting procedure for the UE, as listed in LTE, for NR neighbour relation establishment also.

- The RRM measurements on IDLE RSs are expected to be reported as per the measured PCI values by the UE.

- The neighbour relation establishment procedure based on the UE reporting the ECGI of the unknown cell as listed in LTE can be re-used for NR also.

It is our understanding that the information required to derive the ECGI (PLMN ID and the CellIdentity) are provided in NR as part of the SIB1 message as well. The system information will be broadcasted for the SA deployments of NR and hence the UE could read this information from the target cell.

- The information elements required to derive the ECGI (PLMN ID and the Cell Identity) are provided in NR as expected to be part of the SIB1 message.

As in LTE for the ECGI reading and reporting procedure, a UE in NR should be able to read and report the neighboring cell’s ECGI to aid neighbor relation establishment procedure in NR.

# Conclusion

In the previous section, the following has been observed:

- In LTE, the UE reports to the serving eNB measurement reports associated to locally unique identifiers (e.g. PCIs).

- For handover decisions based on UE reporting of local IDs, a Neighbour Relation Table (NRT) mapping local IDs to global IDs (e.g. E-CGI) is necessary to identify the target eNB in order to send a handover preparation and/or a handover request.

- In LTE, a serving eNB can request the UE to acquire a global identifier (e.g. ECGI) associated to a previously reported local identifier (PCI).

- In LTE, a globally unique identifier (ECGI) is broadcasted in SIB1 and can be associated to a locally unique identifier (PCI).

- In LTE, a globally unique identifier (ECGI) is broadcasted with a larger periodicity than the locally unique identifier (PCI).

- The RRM measurements on IDLE RSs are expected to be reported as per the measured PCI values by the UE.

- The neighbour relation establishment procedure based on the UE reporting the ECGI of the unknown cell as listed in LTE can be re-used for NR also.

- The information elements required to derive the ECGI (PLMN ID and the Cell Identity) are provided in NR as expected to be part of the SIB1 message.

Based on these observations, we have proposed the following:

- As in LTE for the ECGI reading and reporting procedure, a UE in NR should be able to read and report the neighboring cell’s ECGI to aid neighbor relation establishment procedure in NR.

# References

3GPP TS 36.300 V13.4.0 (2016-06), “Evolved Universal Terrestrial Radio Access (E-UTRA) and Evolved Universal Terrestrial Radio Access Network (E-UTRAN); Overall description”

A. Dahlén, A. Johansson, F. Gunnarsson, J. Moe, T. Rimhagen, H. Kallin, ”Evaluations of LTE Automatic Neighbor Relations”, in Proc. IWSON at VTC Spring, Budapest, Hungary, 2011.