3GPP TSG-RAN WG2 Meeting #97bis                                                                 R2-1704258

Hangzhou, China, 15th-19th May 2017

Source:	CATT

Title:	LCP procedure for NR

Agenda Item:	10.3.1.6

Document for:	Discussion and Decision

# Introduction

During RAN2#97bis, the LCP was discussed and the following agreements were achieved:

=>      Key question is if there cases in which a logical channel should not be mapped to a numerology even though it is the same TTI length as another numerology.

=>      Priority, PBR concept is used in NR as a baseline.

=>      For the purpose of LCP, the MAC entity learns the TTI duration/numerology from the PHY layer.  FFS on the details of how it is signalled.

=>      Logical channel priority is configured per UE as a baseline.  FFS is anything needs to be done to done to treat logical channels differently.

In this contribution, we will further discuss the open issues of LCP:

- LCP parameter configuration (priority/PBR/BSD)

- LCP procedure:

- Which logical channels are considered when performing LCP for one UL grant?

- If UE has multiple UL grants, by which the MAC CEs are transmitted?

- Whether special prioritization handling is needed for URLLC?

- How to perform LCP procedure?

- Assembly of MAC PDU based on resource allocation by LCP

# Discussion

## Parameter configuration for LCP

In RAN2#97bis, it has been agreed that priority and PBR concept are used in NR as a baseline. As to NR, for each UL grant, transmission resource of a single numerology is allocated during LCP. Since it was agreed during the study item that one logical channel can be mapped onto multiple numerologies, we need to further discuss whether the priority/PBR per logical channel is configured per UE or per numerology.

- PBR

PBR is used to guarantee the transmission rate of a logical channel. For GBR service, the PBR is configured based on the service’s GBR. There are two approaches to configure PBR needs to be considered for NR:

Option 1: PBR is configured per numerology for each logical channel

Option 2: PBR is configured per UE for each logical channel

In option 1, the sum of PBRs for the multiple numerologies should satisfy the transmission rate of the service. In other words, if NW does not allocate resource to one of the numerologies, the transmission rate of this logical channel would not be satisfied. This would limit the flexibility of scheduler in gNB.

In option 2, based on the global PBR, it does not matter on which numerology the data of a logical channel is transmitted, its transmission rate can be guaranteed.

Above all, since PBR is related to service’s QoS requirement, it should be always satisfied across the multiple numerologies. In this light, the option 2 is preferred.

- BSD

The bucket size equals to PBR*BSD, which is used to restrict the maximum amount of pending data for a logical channel. Therefore, BSD should follow the same configuration principle as PBR.

Proposal 1: The PBR/BSD per logical channel is configured per UE in NR.

## LCP procedure in UE side

In this part, we will discuss the LCP procedure from the following aspects:

- Issue 1: Which RBs can be considered during the LCP procedure for one UL grant?

- Issue 2: Where is the MAC CE transmitted if UE has multiple UL grants?

- Issue 3: Whether specific handling of URLLC is needed during LCP?

- Issue 4: Processing of the multiple grants

- Issue 5: The relative priority of MAC CEs

Issue 1: Which RBs are considered during the LCP procedure for one UL grant?

In LTE, for UL-SCH transmission, all logical channels which have available data can be considered during LCP for one UL grant. On the other hand, there is a restriction on SL-SCH transmission that only the logical channels belonging to one Prose Destination are considered during LCP for one sidelink grant, because data of different Prose Destinations cannot be multiplexed in one MAC PDU.

Referring to the LCP procedure in LTE, for NR LCP we should take into account that:

- In NR, a logical channel is mapped to one or multiple TTIs/numerologies. Therefore, data of one logical channel cannot be transmitted on a TTI/numerology not supported by this logical channel.

- For duplication, the data of the duplicated legs cannot be multiplexed in one MAC PDU.

Considering TTI/numerology mapping, the NR LCP for one UL grant should only consider the logical channels mapped to the TTI/numerology indicated by this UL grant.

Regarding duplication in CA, it was agreed in RAN2#97 that the duplicated legs would be mapped to different CCs. Since an UL grant is CC-specific in CA case, the corresponding LCP should not consider the duplicated logical channels which are not mapped to this CC.

Proposal 2: The NR LCP for one UL grant can only consider the logical channels which are mapped to the TTI/numerology indicated by this UL grant.

Proposal 3: For an UL grant on a CC, the corresponding LCP does not consider the duplicated logical channels not mapped onto this CC.

Issue 2: Where is the MAC CE transmitted if UE has multiple UL grants?

In LTE, if UE can transmit multiple MAC PDUs simultaneously, it is left to UE implementation in which MAC PDU to include the pending MAC CE. In NR, the multiple UL grants are on different TTIs/numerologies so that we should further consider:

- For UL grant on relative shorter TTI length, we can transmit latency critical MAC CE (e.g. BSR) on it. One the other hand, we can forbid any MAC CE transmission on it and leave more room for the latency critical service. Both approaches can bring some benefits.

- For non latency-critical MAC CE, it can be transmitted on any TTI/numerology

To this end, it is sufficient to handle this by UE implementation.

Proposal 4: If UE has multiple UL grants, in which MAC PDU the MAC CEs are transmitted is left to UE implementation.

Issue 3: Whether specific handling of URLLC is needed during LCP?

In legacy LCP procedure, the logical channels are treated in the decreasing priority order. In addition, the logical channel priority is not only related to service’s latency requirement according to the QCI framework. It cannot be excluded that some eMBB logical channels might have higher priority than those of URLLC. In RAN2#97bis, it was discussed whether optimization is needed to avoid higher priority eMBB logical channel hogging URLLC resource. And there are some solutions which suggest allocating resource for URLLC first, especially for the relative short TTI length. To our understanding, these kinds of optimization are not necessary because:

- Logical channel priority reflects the scheduling priority of the associated service. If all the URLLC logical channels are prioritized during LCP, the QoS of some eMBB services with higher priority would not be satisfied.

- For the 2-step LCP, appropriate PBR is configured to guarantee the transmission rate. As to GBR service, PBR is generally set to its GBR. . This means the GBR of URLLC can be generally satisfied under the LCP framework. It is not a severe problem that the transmission of non-GBR service is not guaranteed occasionally.

Based on above analysis, since the “hogging resource” problem is not severe, we prefer not to introduce optimization for this, considering pains versus gains.

Proposal 5: At the first stage of NR, there is no need to introduce special prioritization handling for URLLC in LCP procedure.

In LTE the priority of some MAC CEs is higher than the data from UL-SCH. In terms of latency, one might see it beneficial to prioritize URLLC over MAC CE. However, the resource occupied by MAC CEs is not large and there is not always MAC CE(s) triggered in every UL TTI. In this light, the corresponding optimization would not bring much benefit. We can keep the relative priority between MAC CEs and logical channels the same as that in LTE.

Proposal 6: The relative priority between MAC CEs and logical channels is kept the same as that in legacy LTE.

Issue 4: How to perform LCP?

In terms of detailed LCP procedure, the following aspects should be considered:

- Aspects 1: Whether the UE is able to process multiple UL grants on multiple TTIs/numerologies simultaneously?

- Aspects 2: The resource allocation procedure for each individual UL grant.

For Aspect 1, in NR if the UE has the capability to transmit/receive data simultaneously on multiple TTIs/numerologies, it may then receive UL grants on multiple TTIs/numerologies simultaneously. This capability could impact whether gNB would allocate resource on multiple TTIs/numerologies simultaneously. Therefore, a new UE capability signalling related to this should be introduced.

Proposal 7: A new capability signalling about whether UE can transmit/receive data on multiple TTIs/numerologies simultaneously should be introduced.

For those UEs which have this capability, we should expect RAN1 to define, similar to LTE, some UE processing delay between the UL grant reception on NR-PDCCH and the associated UL transmission on NR PUSCH (round-trip time, RTT). And it is likely that different TTIs/numerologies will result in different UE RTTs. Therefore, for consistency with L1 RTTs, UE should address multiple UL grants received simultaneously on different TTIs/numerologies in increasing order of their associated L1 RTTs.

On the other hand, if UE does not have the capability, the smart gNB would not allocate multiple UL grants simultaneously generally. If such UE still receives multiple UL grants simultaneously, it should select the one on TTIs/numerology with the shortest L1 RTTs.

Proposal 8: UE should address multiple UL grants received simultaneously on different TTIs/numerologies in increasing order of their associated L1 RTTs, if it has the capability to transmit/receive data on multiple TTI/numerology simultaneously.

Proposal 9: UE should select the received UL grant with the shortest associated L1 RTTs, if it does not have the capability to transmit/receive data on multiple TTI/numerology simultaneously.

For the Aspect 2, during the LCP for each individual UL grant, the considered logical channels should be allocated resource in decreasing order of priority. In addition, the starvation of the data from low-priority logical channel should be avoided. Therefore, the legacy 2-step LCP can be used as a baseline for this.

Proposal 10: The legacy 2-step LCP is used as baseline to process each individual UL grant.

Issue 5: Assembly of MAC PDU during LCP

Based on the 2-step LCP procedure, for each UL grant, the data transmitted for one logical channel in one TB is related to the resource allocated for it in each LCP step. Therefore, we should consider the assembly of data belonging to one logical channel during LCP.

Option 1: all RLC PDUs of the same logical channel allocated by LCP are consecutive in the MAC PDU, irrespevctive of the 2-step LCP allocation

Option 2: RLC PDUs of the same logical channel allocated by LCP can be located in two disjoint groups of consecutive PDUs in the MAC PDU, reflecting the 2-step LCP allocation.

In option 1, UE should first determine how much resource is allocated for one logical channel in the two-step LCP based on UL grant, respectively. Only then it can merge the two parts of data into one one group of consecutive RLC PDUs.. Doing so, UE cannot forward to L1 the set of MAC SDUs (RLC PDUs) already available from the 1st step of LCP until the 2nd set has been allocated and constructed.  Note that since it corresponds to the PBR, the 1st set of MAC SDUs can be fully pre-computed and ready to go upon green light from LCP, although construction of the 2nd set will depend on the granted allocation size. For these reasons, we prefer option 2 which best addresses the UE transmission optimization, which has been one of the key metrics influencing NR UP stack design so far.

Proposal 10: The data of one logical channel can be distributed in two separate groups of consecutive PDUs in the MAC PDU, thus meeting the resource allocation in the two-step LCP.

# Conclusion

Based on the analysis in section 2, it is proposed:

Proposal 1: The PBR/BSD per logical channel is configured per UE in NR.

Proposal 2: The NR LCP for one UL grant can only consider the logical channels which are mapped to the TTI/numerology indicated by this UL grant.

Proposal 3: For UL grant on a CC, the corresponding LCP does not consider the duplicated logical channels which are not mapped to this CC.

Proposal 4: If UE has multiple UL grants,  in which MAC PDU the MAC CEs are transmitted is left to UE implementation.

Proposal 5: At the first stage of NR, there is no need to introduce special prioritization handling for URLLC in LCP procedure.

Proposal 6: The relative priority between MAC CEs and logical channels is kept the same as that in legacy LTE.

Proposal 7: A new capability signalling about whether UE can transmit/receive data on multiple TTIs/numerologies simultaneously should be introduced.

Proposal 8: UE should address multiple UL grants received simultaneously on different TTIs/numerologies in increasing order of their associated L1 RTTs, if it has the capability to transmit/receive data on multiple TTI/numerology simultaneously.

Proposal 9: UE should select the received UL grant with the shortest associated L1 RTTs, if it does not have the capability to transmit/receive data on multiple TTI/numerology simultaneously.

Proposal 10: The data of one logical channel can be distributed in two separate groups of consecutive PDUs in the MAC PDU, thus meeting the resource allocation in the two-step LCP.