3GPP TSG-RAN WG2 Meeting #98	R2-1704320

Hangzhou, China, 15 - 19 May 2017

Agenda item:	9.4.5

Source:	Nokia, Alcatel-Lucent Shanghai Bell

Title:	Non-certified air-borne UE detection

WID/SID:	FS_LTE_Aerial - Release 15

Document for:	Discussion and Decision

# 1	Introduction

One of the objectives of the recently agreed “Study on enhanced Support for Aerial Vehicles”, description of which can be found in [1], is:

- Identification of an air-borne UE that does not have proper certification for connecting to the cellular network while air-borne [RAN2]

In this paper, we analyze this objective and provide our view about its impact on the work in other WGs as well as provide the principles and guidelines for the solution, which is to be developed to achieve the aforementioned goal.

# 2	Discussion

Before a solution for the problem mentioned in the introduction can be identified, it is important to clarify some aspects related to air-borne UEs certification. An extract from [2] can be helpful with that regards:

“In addition, two types of “drone UE” are observed in the field. One is a drone equipped with a cellular module. Such a drone UE has passed a certification test for aerial usage. On the other hand, there might be a drone carrying a smartphone. Since a smartphone has passed a certification test for terrestrial usage, such the usage is not permitted from the regulation standpoint. In that sense, UL signal from such the UE can be regarded as jamming.”

First of all, the provided fragment mentions regulations. However, it has to be noted that these are region and country specific, which can be seen e.g. from the table presented on Figure 1.

|  | LOS | Airport | Height | Any Person | Aglomeration and Congested Area | Road and Railway | Buildings and Industrial Zones | Others |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 5 km for public 8 km for military | 100 m |  | 150 m *From all urban areas | 150 m | 150 m* *Important Gvt Facilities | 150 m* *Military Zones / Royal Family Prop. |
|  |  | 1.5 km | 100 m *Restricted to 30m in some states |  | Avoid aglomerations and public gatherings |  | Not fly over areas that can cause danger |  |
|  |  | 10 km | 150 m |  | 50m (aglomerations) 100m (populated areas) |  |  |  |
|  |  | Avoid controlled airzones |  | 50 m (if filming) *30m during Take-Off or Landing | 150 m (if filming) |  | 50 m (if filming) |  |
|  |  | 8 km | 70 m | 50 m | 150 m |  | Not fly over industrial and infrastructure buildings | 200 m radius |
|  |  | Avoid controlled airzones | 120 m | 50 m | Not fly over areas that can cause danger |  |  |  |
|  |  | 15 km | 120 m |  | 150 m | 150 m (roads) 50 m (railways) | 150 m (buildings) 50 m (industrial buildings) |  |
|  |  | - Other rules may apply (distance from animal farms, power plants, etc). - Policies may vary regarding drone size/weight - Some countries require UAV register and/or pilot license. - Presented rules may be overriden by special licenses in some MS |  |  |  |  |  |  |

Figure 1: Overview of current regulations in some countries (June 2016)

Moreover, these are defined by proper regional and country institutions and are beyond 3GPP specification.

Observation 1: Drone UE certification is based on regional and country regulations, which are out of scope of 3GPP.

Based on this observation, we believe that certification as such should not be discussed by RAN2. What really matters from RAN2 point of view is whether a certain UE is authorized to work as an air-borne UE or not. Such authorization information should be provided from the Core Network. Core Network can decide about the authorization based on the subscription information or based on information from proper Application Servers or some 3rd party infrastructure. However, such aspects are beyond RAN2 and should be decided by SA2.

Proposal 1: RAN2 should request SA2 to discuss how air-borne UE certification can be checked for the UE. RAN2 should also ask both RAN3 and SA2 to discuss how authorization to act as an air-borne UE is provided from the Core Network to RAN.

However, authorization information itself will not be sufficient. The network should be also able to detect that a UE is air-borne and then compare this with authorization information. Therefore, we see an identification of the solution allowing to detect air-borne UEs as a main goal of “Identification of an air-borne UE” objective for RAN2.

Proposal 2: RAN2 should study the potential solutions for detecting that a certain UE is an air-borne UE.

However, even when such solution exists the question arises when it should be used. It would be unreasonable to require the network to try to identify each and every single UE, which is used in a non-legitimate way as an air-borne UE. The recalled fragment from [2] mentions also that traffic from non-certified UE can be regarded as jamming, since it is not allowed from regulation standpoint. This means that what really causes a problem is interference generated in the network. Based on this we believe that a first step taken by the network should be to detect excessive interference, potentially coming from an air-borne UE. As such, this objective is very closely related to another one mentioned in [1], i.e.:

- Solutions to detect whether UL signal from an air-borne UE increases interference in multiple neighbour cells and whether an air-borne UE incurs interference from multiple cells [RAN1, RAN2]

However, it could be argued that the difference is that in this objective, it is assumed already that we know that a UE is air-borne (e.g. based on authorization form the network) while for the detection of interference generated by a non-certified air-borne UE such knowledge is not available. In our opinion, it would be beneficial to develop a single solution for both cases. It is also important to note that the UEs, which are likely to be used improperly with drones are legacy UEs and the developed solution should allow for detection of interference originating from these UEs. We therefore propose the following requirement to be agreed and captured in the TR with regards to the interference detection solution:

Proposal 3: The solution developed for interference detection within this SI should allow for detection of potentially air-borne UEs, which cause excessive interference, including UEs non-certified for aerial usage and UEs not implementing Rel-15 enhancements.

If the above proposals would be agreed (and after identifying the proper solutions) the network would be able to perform the following tasks:

- Identify the potentially air-borne UE, which is a source of excessive interference

- Check whether this UE is an air-borne UE

- Verify whether this UE is authorized to act as an air-borne UE

We propose to capture these three elements as the guidelines for the solution to be developed within this work.

Proposal 4: Capture in the TR that the developed solution should allow for:

- Identifying the potentially air-borne UE, which is a source of excessive interference

- Checking whether this UE is an air-borne UE

- Verify whether this UE is authorized to act as an air-borne UE

One may wonder why two steps for air-borne UE identification are needed, i.e. detection of interference from potential air-borne UE and then verification whether the UE is indeed air-borne. In our opinion, it is a non-trivial task to develop a solution, which would undoubtedly tell that an interference is generated by an air-borne UE, e.g. an interfering UE may be the one placed on top of the skyscraper or in the passenger aerial vehicle and it may be used legitimately (even though it is a non-certified air-borne UE). At the same time, it is very important that network makes proper choices based on detected UE’s nature, e.g.

- If the UE causing interference is a terrestrial usage certified UE, which is used legitimately network may just modify its transmission parameters or apply interference mitigation technique.

- If the UE causing interference is a terrestrial usage certified UE, which used in a not legitimate way as an air-borne UE then network may terminate its transmission completely.

- If the UE causing interference is an air-borne certified UE used legitimately and having high priority traffic (e.g. firefighter’s drone) then the network may let it be and perform some action on other UEs.

The exact network behavior is obviously not to be specified, but the bottom line is that the detection solution needs to work accurately and cannot perform erroneous decisions. Otherwise, the users may experience deteriorated service quality due to improper network’s decisions, which in turn can lead to user complaints for operator’s service.

Proposal 5: A solution for air-borne UE identification needs to work with accuracy as close to 100% as possible.

# 3	Summary

This paper made an attempt to clarify the meaning of the objective concerning identification of a non-certified air-borne UE. Based on the presented considerations it is proposed to agree on the observations and proposals mentioned below and capture them in the Technical Report as the guidelines for the solution to be identified during the study.

Observation 1: Drone UE certification is based on regional and country regulations, which are out of scope of 3GPP.

Proposal 1: RAN2 should request SA2 to discuss how air-borne UE certification can be checked for the UE. RAN2 should also ask both RAN3 and SA2 to discuss how authorization to act as an air-borne UE is provided from the Core Network to RAN.

Proposal 2: RAN2 should study the potential solutions for detecting that a certain UE is an air-borne UE.

Proposal 3: The solution developed for interference detection within this SI should allow for detection of potentially air-borne UEs, which cause excessive interference, including UEs non-certified for aerial usage and UEs not implementing Rel-15 enhancements.

Proposal 4: Capture in the TR that the developed solution should allow for:

- Identifying the potentially air-borne UE, which is a source of excessive interference

- Checking whether this UE is an air-borne UE

- Verify whether this UE is authorized to act as an air-borne UE

Proposal 5: A solution for air-borne UE identification needs to work with accuracy as close to 100% as possible.

# References

- RP-170779, Study on enhanced Support for Aerial Vehicles – NTT DOCOMO INC, Ericsson

- R2-1701077, Potential challenges on emerging drone services – NTT DOCOMO INC