3GPP TSG-RAN WG2 #98	Tdoc R2-1704364

Hangzhou, P.R. of China, 15th – 19th May 2017

Agenda Item:	10.3.2.2

Source:	Ericsson

Title:	RLC Segmentation Configurability

Document for:	Discussion, Decision

# Introduction

In RAN2-97bis, the following was agreed:

Agreements RLC segmentation: - As a baseline, segmentation is always enabled for RLC-AM and RLC-UM. FFS if there are cases in which it is beneficial to disable segmentation - An RLC SDU for UM and AM can be associated with only one RLC SN, i.e., the byte segments from an RLC SDU can be associated with the same RLC SN. - Segmentation and re-segmentation is based on RLC SDU, i.e., SO field indicates byte position of the RLC SDU - RLC header is to be designed in following principles: - RLC header indicates if RLC PDU carries a complete RLC SDU or RLC SDU segments. - RLC header does not include SO field if RLC PDU carries a complete RLC SDU. - RLC header does not include SO field when the beginning of the RLC SDU is segmented. - RLC header includes SO field when the middle or end of the RLC SDU is segmented. - RLC header indicates whether the RLC PDU contains the end part of RLC SDU segment or not when the middle or end of the RLC SDU is segmented.

# Discussion

In RAN2-97bis email discussion [1], it was proposed that segmentation should be optional or configured dynamically for RLC UM and RLC AM mode. Such behaviour would not only add complexity to the UE and Network implementation but would also result waste of radio resources when an UL grant is too small for RLC PDUs to fit into transport block and the transport block must be filled with padding. Rather than filling the TB with padding, UE should fill the TB(s) with data and not send padding if there is more data to send. This option should not increase the UE processing significantly as the removal of the concatenation was removed from the RLC. As for the SO based segmentation, the processing overhead caused by the SO based segmentation is relatively small. Therefore, segmentation should be enabled as a baseline for RLC AM and RLC UM mode.

Email discussion [1] suggests that there could be a need for option to disable the segmentation in URLLC or high in data rate scenarios. However, in high data rate scenarios, it may be beneficial to use large PDU size. When large PDU size is used and grant is not enough to serve the whole PDU, disabling the segmentation may led to situation where over the air overhead caused by the padding is significant. In URLLC case the added complexity to disable the segmentation may not be justified because URLLC may already use SPS based scheduling and small packet size. By adjusting the packet and grant size, URLLC traffic may be configured so that the RLC segmentation becomes rare case.

RAN2 is kindly requested to confirm that RLC segmentation is always enabled for RLC AM and UM.

# Conclusion

Based on the discussion in section 2 we propose the following:

Proposal 1	RAN2 is kindly requested to confirm that RLC segmentation is always enabled for RLC AM and UM.

# References

- R2-1702646. Email discussion report on Segmentation. NTT DOCOMO, INC.

- R2-1702739. RLC PDU design, Ericsson