3GPP TSG-RAN WG2 #98	Tdoc R2-1704366

Hangzhou, P.R. of China, 15th – 19th May 2017

Agenda Item:	10.3.2.4

Source:	Ericsson

Title:	RLC Status Report Format

Document for:	Discussion, Decision

# Introduction

In RAN2-97bis, following agreements were made:

Agreements on RLC PDU format

-	NR RLC PDU and NR RLC PDU header should be byte-aligned.

-	NR RLC PDU will not include Length Indicator (LI) field.

-	NR RLC PDU will use 2-bits “FI-like” field to distinguish the complete RLC SDU, the first SDU segment, the middle SDU segment and the last SDU segment and SO field is needed just in the case of the middle SDU segment or the last SDU segment.

-	NR RLC TMD PDU only consists of a Data field and does not consist of any RLC headers.

-  FFS if NR RLC UMD SDU should not include SN field and only NR RLC UMD SDU segment should carry SN field

-	NR RLC AMD PDU includes a 1-bit D/C field, 1-bit P field

-	RLC status report format is byte-aligned

-	Introduce a new field(s) “NACK SN range” in the status report format. FFS the details of this field and how the status report is coded

Status report is sent by RLC AM to provide positive or negative acknowledgement of the PDUs. It consists of control PDU header and STATUS PDU payload.

In this contribution we discuss some of the stage-3 aspects of the RLC status report format.

# Discussion

Number of reserved bits used in the RLC status report is depending if the bit or byte alignment is used. Additionally, it is depending on the used Sequence Numbers. Some companies have proposed to reduce the SN size options [1]. We also think that reducing SN size options would be a beneficial [2][3] Having less SN size options results fewer header options for transmitter and receiver, which can be optimized to reduce the number of Reserved-bits in the header. Having fewer options for supported SN sizes also reduces the implementation complexity.

Figure 1 shows RLC status report format with examples of acknowledged and non-acknowledged header formats for 18 and 12 bit SNs. When only the cumulative ACK SN is added (1a, 1c) is used, the overhead caused by the byte-alignment is one bit. When status report has a list of “NACK_SNs” or “NACK_SNs with segmentation information”, the overhead is 3 bits (1b, 1d). We think that the overall overhead caused by byte-alignment in the RLC status report is not significant, when considering that the longer format with NACK_SNs is sent only when one or more SNs are missing. In 12-bit SN format, the overhead caused by the reserved fields is larger, but as the status reports are rarely sent, the overhead could be acceptable.

Figure 1: Bit aligned RLC status report formats including NACK_SN_RANGE with 18-bit SN (1.) and 12-bit SN (2.).

Overhead due to byte alignment is not significant compared to the size of the status report and taking into account that status reports are rare compared to the number of data PDUs.

Bit-shifting of fields (NACK_CN, ACK_SN and SO) should be minimized in the RLC status PDU.

Each status PDU sub-block should use only one bit-shifting pattern which is achieved by byte-aligning each status sub-block (ACK_SN, NACK_SN, NACK_SN+SO).

In RAN2 #97-AH, the working assumption of removal RLC concatenation was agreed. Consequently, there are as many RLC SNs in flight as there are PDCP PDUs (unless DC is used). If one MAC PDU including multiple RLC SNs gets lost, the RLC status report size is increased compared to LTE since many NACKED SNs are bundled to the status report. To overcome this limitation, RAN2 97bis agreed on introducing NACK SN_RANGE field to the status report. The NACK_SN_RANGE includes information about the adjacent non-acknowledged Sequence Numbers. When many consecutive SNs have been missed, the receiver may request those SNs by adding the SN range to the status report. As can be expected, using NACK_SN_RANGE would also reduce the over-the air overhead since less SNs with length of 12-bit or 18-bit are needed in the header.

It is not mandatory for NACK_SN_RANGE field to have same range as SN has. Having value, which is indicating continuous range from the first NACK_SN is sufficient. Static value, such as 10bits allows range of 1024 SNs, which should be sufficient. Figure 1 shows example of NACK_SN_RANGE

10-bit range for NACK_SN_RANGE parameter should be sufficient.

NACK_SN_RANGE value is static and does not change according to the SN range

With respect to the D/C and CPT field in the Figure1. As the D/C field creates distinction between data and control DPUs and CPT field is beneficial for allowing future extensions. We propose to keep both as it is in LTE.

Status report format has D/C field

Status report format includes CPT field

3 types of E-fields, or 3 bit NACK tuples can be observed from the figure 1. Purpose of the E-bits is to provide indication to the receiver parser of which type of NACK field will follow. The figure displays tuple of 3-bits, but the size of the NACK tuple may as well be 2-bits depending on number of NACK cases and possible extensions to status report format.

# Conclusion

In section 2 we made the following observations:

Observation 1	Overhead due to byte alignment is not significant compared to the size of the status report and taking into account that status reports are rare compared to the number of data PDUs.

Observation 2	10 bit range for NACK_SN_RANGE parameter should be sufficient.

Based on the discussion in section 2 we propose the following:

Proposal 1	Bit-shifting of fields (NACK_CN, ACK_SN and SO) should be minimized in the RLC status PDU.

Proposal 2	Each status PDU sub-block should use only one bit-shifting pattern which is achieved by byte-aligning each status sub-block (ACK_SN, NACK_SN, NACK_SN+SO).

Proposal 3	NACK_SN_RANGE value is static and does not change according to the SN range

Proposal 4	Status report format has D/C field

Proposal 5	Status report format includes CPT field

Proposal 6	Three E-bits are included in Status report format E1, E2 and E3.

# References

R2-1702740. RLC STATUS report format. Ericsson