3GPP TSG-RAN WG2 #98	R2-1704369

Hangzhou, P.R. of China, 15th – 19th May 2017

Agenda Item:	 10.3.3.3

Source: 	Ericsson

Title:  	Lossless PDCP SN reconfiguration at HO

Document for:	Discussion, Decision

# Introduction

This contribution introduces a solution to achieve lossless PDCP SN reconfigurations.

# Discussion

Normally there should be no reason for changing the SN length because the initial SN length should in most cases be sufficient. However, there may be a need for changing the SN length at handover, because one of the two nodes involved in the handover may support a different SN range set. In NR, two SN lengths have been agreed: 12 bits and 18 bits. Having only two values reduces the NR-NR handover cases in which the SN length may be reduced. In LTE, there is a larger number of SN length values. Yet, no solution was agreed to address lossless reconfiguration when the SN length is reduced. The main reason could be that the SN change is most frequent in inter-vendor handovers.

We do not think that SN length may be dynamically changed depending on the current bit rates, however. We consider that it is sufficient to change PDCP SN length only during handover.

PDCP SN length may be reconfigured only during a handover (as in LTE)

LTE is focused in mobile broadband which typically runs over TCP. TCP can cope with some losses. Since SN length change is not common and higher layers can recover, it is questionable that a solution for lossless handovers when the SN is reduced is needed.

For NR, more services are expected. There may be services such as URLLC which require high reliability, or there may be services running on other protocols than TCP. This can justify the introduction of a solution which can provide lossless SN length reconfigurations. However, due to the limited applicability, it is important that the solution is simple and can work in NR-NR and NR-LTE handovers.

## Lossless handovers when SN length is reduced

In LTE, the procedure for a lossless handover (SN length is maintained or increased) is as follows:

- The source cell forwards to the target cell the PDCP SDUs, COUNT (SN, and HFN) of those PDCP PDUs for which there is no confirmation that they have been received successfully

- The UE transmits a PDCP status report

- The target cell and the UE (re-)transmits the data which was not received.

In NR, step 1. and step 2. can be the same as in LTE (when the SN length value is not decreased).

Step 3. requires further analysis. The main question to answer in step 3. is the COUNT (i.e. HFN and SN) values to use in the target cell and, therefore, the SN to use for transmission.

The solution is quite simple. The UE and the NW should re-start the COUNT value (i.e. SN and HFN) to its initial value and re-assign new values. The procedure is exemplified in the figure below:

When the UE transmits the PDCP status report including the COUNT value, the target cell knows which PDCP SDUs need to retransmit, and the target cell will re-assign new COUNT value i.e. HFN and SN.

This solution provides a lossless reconfiguration. On the other hand, PDCP SDUs might not be delivered in sequence to higher layers. This is exemplified in the figure below.

When the reconfiguration happens, the UE may deliver to higher layers all received/buffered SDUs. The UE will deliver SN = {231, …, 279, 281, 282, … 499, 501, …}. This does not mean that missing PDCP PDUs are lost. It means that data will not be delivered in-sequence to higher layers. This behavior is already understood and possible in PDCP. This can happen, for instance, when T-reordering timer expires. Then, packets are not delivered in-sequence, as there are gaps. From a higher layer point of view, there will be no losses.

This solution is very simple and based on similar procedures already available in LTE for similar reconfigurations.

For lossless handover reconfigurations and when SN length is reduced, the procedure is:

  - Source cell forwards PDCP PDUs (HFN, SN, and user data) considered to not have been received to the target cell,

  - Reset HFN and SN,

  - Re-transmitted PDCP SDUs are assigned with new SN and HFN values, in the target cell

### Other solutions

We analyze some of the different options brought so far:

- [1] brings 3 potential problems at the SN length reconfiguration, and proposes some solutions. Problem 1 is a valid problem. However, the solution that is proposed for this problem forces the network/UE to get a PDCP status report (e.g. FMC) to resume transmissions. In NR, the goal is to have handovers with as close as possible to 0 ms interruption time. However, this solution can potentially delay the re-establishment of flow of user data. The network should be able to retransmit PDCP SDUs even if it does not get the status report.
Problem 2 is already resolved as RAN2 agreed to introduce a COUNT based PDCP status report.
Problem 3 relates to the expiration of certain timers. We do think this is a configuration issue i.e. setting the right value rather a timer issue. Of course, “problem 3” may happen if the PDCP transmitter does not get the status report. So “problem 3” is another reason why the PDCP transmitter should not be forced to wait for the status report.

- [2] proposes a number of solutions which are applied depending on whether the handover in within NR or between NR-LTE. It also introduces new requirements in the UE and the NW.

  - Moving RRC procedures to PDCP
The reconfiguration is not controlled any longer by RRC but by PDCP. Thus, it implies that PDCP will need to emulate the RRC introducing PDCP ACKs, and it will also require that PDCP handles error cases e.g. in case the PDCP indication fails.

  - The target cell must support and use the source configuration
After a reconfiguration, the target cell and UE will need to still operate using the source configuration and not the target configuration.

  - Mandatory support of all SN LTE and NR SN lengths in both UE and NW;

  - PDCP SN reconfiguration should guarantee continuous COUNT value to facilitate data deciphering for lossless data delivery.

    - Note that in RAN2#95bis, RAN2 agreed/acknowledged that NR specification should not prohibit out-of-order deciphering of PDCP PDUs.

  - RLC state must be passed from source to target

We think that these solutions have impact on several layers and are excessively complicated. The introduction of these solutions in LTE would be also rather complicated. It also brings new challenges and problems which must be solved. Considering that the limited cases in which the SN reconfiguration can occur, we think that these solutions are not justified and a much simpler solution is preferred.

# Proposals

We kindly request RAN2 to discuss and agree on:

Proposal 1	PDCP SN length may be reconfigured only during a handover (as in LTE)

Proposal 2	For lossless handover reconfigurations and when SN length is reduced, the procedure is:

a.	Source cell forwards PDCP PDUs (HFN, SN, and user data) considered to not have been received to the target cell,

b.	Reset HFN and SN,

c.	Re-transmitted PDCP SDUs are assigned with new SN and HFN values, in the target cell

# References

R2-1703557, Reconfiguration to Shorter SN, Nokia, Alcatel-Lucent Shanghai Bell

R2-1702587, PDCP SN Reconfiguration, Huawei, HiSilicon

# Annex for information (based on 36.323)

### 5.2.1	UL Data Transfer Procedures

#### 5.2.1.1	Procedures for DRBs mapped on RLC AM

When upper layers request a PDCP re-establishment, the UE shall:

-	reset the header compression protocol for uplink and start with an IR state in U-mode (if configured) [9] [11], except if upper layers indicate stored UE AS context is used and drb-ContinueROHC is configured [3];

-	if connected as an RN, apply the integrity protection algorithm and key provided by upper layers (if configured) during the re-establishment procedure;

-	if upper layers indicate stored UE AS context is used or if upper layers configure a PDCP SN length shorter than the last configured, set Next_PDCP_TX_SN, and TX_HFN to 0;

-	apply the ciphering algorithm and key provided by upper layers during the re-establishment procedure;

-	from the first PDCP SDU for which the successful delivery of the corresponding PDCP PDU has not been confirmed by lower layers, perform retransmission or transmission of all the PDCP SDUs already associated with PDCP SNs in ascending order of the COUNT values associated to the PDCP SDU prior to the PDCP re-establishment as specified below:

-	if upper layers configure a PDCP SN length shorter than the last configured, update the PDCP SN and HFN as specified in subclause 5.1.1

-	perform header compression of the PDCP SDU (if configured) as specified in the subclause 5.5.4;

-	if connected as an RN, perform integrity protection (if configured) of the PDCP SDU using the COUNT value associated with this PDCP SDU as specified in the subclause 5.7;

-	perform ciphering of the PDCP SDU using the COUNT value associated with this PDCP SDU as specified in the subclause 5.6;

-	submit the resulting PDCP Data PDU to lower layer.

#### 5.2.1.2	Procedures for DRBs mapped on RLC UM

When upper layers request a PDCP re-establishment, the UE shall:

-	reset the header compression protocol for uplink and start with an IR state in U-mode [9] [11] if the DRB is configured with the header compression pr

otocol and drb-ContinueROHC is not configured [3];

-	set Next_PDCP_TX_SN, and TX_HFN to 0;

-	apply the ciphering algorithm and key provided by upper layers during the re-establishment procedure;

-	if connected as an RN, apply the integrity protection algorithm and key provided by upper layers (if configured) during the re-establishment procedure;

-	for each PDCP SDU already associated with a PDCP SN but for which a corresponding PDU has not previously been submitted to lower layers:

-	consider the PDCP SDUs as received from upper layer;

-	if upper layers configure a PDCP SN length shorter than the last configured:

-	perform transmission of the PDCP SDUs in ascending order of the COUNT value associated to the PDCP SDU prior to the PDCP re-establishment, update the PDCP SN and HFN, as specified in the subclause 5.1.1 without restarting the discardTimer

-	else:

-	perform transmission of the PDCP SDUs in ascending order of the COUNT value associated to the PDCP SDU prior to the PDCP re-establishment, as specified in the subclause 5.1.1 without restarting the discardTimer.

#### 5.2.2.1a	Procedures for DRBs mapped on RLC AM while the reordering function is used

When upper layers request a PDCP re-establishment while the reordering function is used, the UE shall:

-	process the PDCP Data PDU(s) that are received from lower layers due to the re-establishment of the lower layers, as specified in the subclause 5.1.2.1.4;

-	if the PDCP entity is to be associated with one AM RLC entity after PDCP re-establishment:

-	stop and reset t-Reordering;

-	if upper layers configure a PDCP SN length shorter than the last configured, set Next_PDCP_RX_SN, RX_HFN to 0 and Last_submitted_PDCP_RX_SN to Maximum_PDCP_SN;

-	apply the ciphering algorithm and key provided by upper layers during the re-establishment procedure.