3GPP TSG-RAN WG2 #98	R2-1704374

Hangzhou, P.R. of China, 15th – 19th May 2017

Agenda Item:	10.3.3.3

Source:	Ericsson

Title:	Dynamic reconfiguration of split bearer or duplication

Document for:	Discussion, Decision

# Introduction

In this contribution, we discuss which aspects of the UL transmission behaviour of PDCP should be dynamically reconfigurable. For UL duplication in particular, the following agreements were reached in RAN2#97bis:

Agreements:

1: RRC configures PDCP for duplication and the radio protocols of the UE with separate RLC entities and logical channels to handle duplicates (referred to as “legs”)

2: only one additional leg is configured for PDCP duplicates.

3: the original PDCP PDU and the corresponding duplicate shall not be transmitted on the same transport block.

FFS whether in CA case to support PDCP duplicates on the same carrier with some restriction to prevent them from being transmitted on the same transport block. (Noting that we have already agreed that they can be sent on different carriers)

4:	PDCP duplication solution for CA requires only one MAC entity.

5	logical channel mapping restrictions need to be introduced to handle duplicates in within one MAC entity (CA).

# Discussion

In the email discussion [1] controlling of uplink duplication in NR was discussed. Companies were aligned on considering duplication configuration per DRB/SRB, and that beside RRC configuration of duplication, also L2 signaling should be used to adapt the duplication function.

Duplication on PDPC in NR is considered in both CA and DC. First, for simplicity, duplication control between CA and DC should be aligned. Under the assumption that PDCP data is pulled to lower layers when an UL grant is available (for which transmission is allowed acc. to logical channel restrictions as agreed in RAN2#87bis), we believe that this alignment between CA and DC operation is easily possible. Please note also our companion papers [2] discussing UL transmit operation on PDCP, and [3] discussing how PDCP data is considered for BSR.

Duplication control on PDCP is aligned among duplication in DC and CA.

For duplication, the transmitter should be configurable for three modes of transmission: using the first transmission leg, the second transmission leg, or both i.e. duplication. This way, beside duplication also link switches can be realized. Considering the potential dynamic changes in the UL channels of the respective transmission legs, aligned with [1], we believe beside RRC configured transmission direction(s), also dynamic reconfiguration should be enabled.

As duplication is applied on PDCP it seems straight forward to use PDCP control signalling to enable/disable the duplication and to steer the link direction if duplication is disabled.

Duplication activation/deactivation and link direction is configurable by RRC.

Additionally, duplication activation/deactivation and link direction can be dynamically reconfigured by signalling via PDCP control PDU.

Furthermore, considering uplink data split rather than duplication, and assuming the LTE split threshold approach is reused in NR (as discussed in [2]), it should be discussed to make also the UL split operation dynamically adaptable and aligned with the duplication configuration.

In LTE, the split operation is configured based on RRC reconfiguration. Since in NR however faster throughput changes are expected due to potential blocking and fading dips at higher carrier frequencies, it appears useful to make the split operation in NR more dynamically configurable. This way, the UL transmission direction can be quickly adapted towards the link with the highest throughput. Therefore, we propose to make also the prioritized link direction in the split operation configurable by PDCP control signaling.

The prioritized UL transmission direction (similar as ul-DataSplitDRB-ViaSCG), should be configurable by RRC and dynamically adaptable by PDCP control signaling.

UL direction configuration and duplication functionalities are both considered for DRBs e.g. for URLLC kind of traffic, as well as for SRBs for control diversity. For simplicity, we should consider aligning the reconfiguration functionalities among them.

Reconfiguring functionalities are aligned between DRB and SRB.

The above proposals can be realized by defining a new PDCP control PDU that carries information about whether first transmission leg and/or second transmission leg is used (2 bit). Therefore, we propose:

PDCP Control PDU introduced indicating whether first and/or second transmission leg is used for uplink transmissions (both meaning duplication operation).

# Conclusion

Based on the discussion in Section 2 we propose:

Proposal 1	Duplication control on PDCP is aligned among duplication in DC and CA.

Proposal 2	Duplication activation/deactivation and link direction is configurable by RRC.

Proposal 3	Additionally, duplication activation/deactivation and link direction can be dynamically reconfigured by signalling via PDCP control PDU.

Proposal 4	The prioritized UL transmission direction (similar as ul-DataSplitDRB-ViaSCG), should be configurable by RRC and dynamically adaptable by PDCP control signaling.

Proposal 5	Reconfiguring functionalities are aligned between DRB and SRB.

Proposal 6	PDCP Control PDU introduced indicating whether first and/or second transmission leg is used for uplink transmissions (both meaning duplication operation).

# References

R2-170xxxx, Email discussion summary on control of UL PDCP duplication (draft), Huawei, RAN2#98, Hangzhou, China, 15- 19, May2017

R2-1704681, PDCP Uplink transmit operation, Ericsson, RAN2#98, Hangzhou, China, 15- 19, May2017

R2-1704370, PDCP data volume reporting in duplication (BSR), Ericsson, RAN2#98, Hangzhou, China, 15- 19, May2017