3GPP TSG-RAN WG2 #98	Tdoc R2-1704375

Hangzhou, P.R. of China, 15th – 19th May 2017

Agenda Item:	10.3.1.5

Source:	Ericsson

Title:	SR signalling content in NR

Document for:	Discussion, Decision

# Introduction

It has been agreed in #RAN2 97bis that the SR should at least distinguish the “numerology/TTI type” of the logical channel that triggered the SR (how this is done is FFS). This contribution further analyses the possible options to enhance SR to achieve the above agreement.

# Discussion

The scheduling entity should be aware of the information including

- An indication that UE has data to transmit

- Buffer size for each logical channel

- Priority indication for each logical channel

- Indication of a set of the associated numerologies/TTI durations for each logical channel

To reduce the latency due to dynamic scheduling, and improve the resource utilization efficiency, the above information must be provided on time and accurately by UE via SR and BSR.

By knowing the type/priority of the logical channel, the gNB can provide grants for the traffic that should be scheduled. This enables a more correct priority handling. This is beneficial for RRC signalling or URLLC data.

More efficient priority handling is achievable by more detailed information provided for SR.

During e-mail discussion [97#62], there are two options that are under discussions for SR signalling content, i.e., how to convey more information than the indication of the data availability of new data.

- The first option is to introduce multiple SR configurations for single-bit SR, where different SR resources (in either time domain or frequency domain) are allocated to different LCHs/LCGs. If the UE triggers SR separately for different LCHs/LCGs, upon reception of a SR, gNB can identify the associated LCHs/LCGs. As another implementation, different SR transmission periodicities can be used for different SR configurations, in this way, the LCHs/(LCG)s with higher priority levels can be configured with shorter SR transmission intervals, while LCH (LCG)s with lower priority levels can be configured with longer SR transmission intervals.

- The second option is to support multi-bit SR. With more bits used in a SR it would be possible for a UE to provide more detailed information such as the type/priority of the logical channel which has data available. How many bits that a SR should be extended with, is a question of how to achieve a good trade-off between the increased L1 control channel issues (overhead, design complexity etc) and the possible achieved gain in terms of UP latency reduction.

The first option is an enhancement based on LTE SR format, therefore requires minimal standardization efforts. The second option aims to a new PUCCH format which requires bigger standardization efforts, and incurs higher control signaling overhead. From this perspective, we think we should stick to the first option. While at the same time, it is still unclear whether the second option is feasible to introduce in NR. Further study by RAN1 is necessary.

RAN2 should focus on single-bit SR with multiple SR configurations while leaving multi-bit SR discussion up to RAN1 progress.

Each logical channel may be configured with different TTI lengths/numerology characteristics by the network. So, the network has full knowledge on what TTI lengths/numerology characteristics are associated with each logical channel. Upon reception of a SR, the network identifies its associated LCH/LCG based on the SR configuration/pattern that it uses. Based on the mapping relation between TTI lengths/ numerology characteristics and the logical channels, the network can further identify the “numerology/TTI type” associated with the SR.

To limit the overhead due to multiple SR resources configured for SR, it may be sufficient to differentiate SRs for LCHs/LCGs with higher priority levels.

A UE may be configured with different SR resources to differentiate SR configurations. This will increase the resource consumption for control channel. To limit the overhead due to this, we should bear in mind that it may be sufficient to differentiate SRs for LCHs/LCGs with higher priority levels. Accurate scheduling information for LCH/LCG with lower priority levels can be signalled via BSR. We think that it should be possible to have up to four single-bit SRs, corresponding to the number of the logical channel groups in LTE.

Independently of if RAN1 concludes multi-bit SR is feasible, RAN2 should be able to conclude on configurable SR resources tied to the LCH to numerology/TTI.

The network can configure a UE with multiple SR configurations to distinguish the “numerology/TTI type” for each LCH/LCG.

When a SR is triggered by certain LCH/LCG, the UE uses the SR configuration associated with that LCH/LCG.

The UE uses the SR configuration associated with the LCH/LCG that triggers the SR.

As a correspondence of the distinguish of the “numerology/TTI type”, the grant should also indicate the “numerology/TTI type”. More detailed discussions on the grant indication are available in another contribution [1].

Assume a grant carries the information of the “numerology/TTI type”

# Conclusion

In section 2 we made the following observations:

Observation 1	More efficient priority handling is achievable by more detailed information provided for SR.

Observation 2	To limit the overhead due to multiple SR resources configured for SR, it may be sufficient to differentiate SRs for LCHs/LCGs with higher priority levels.

Based on the discussion in Section 2 we propose the following:

Proposal 1	RAN2 should focus on single-bit SR with multiple SR configurations while leaving multi-bit SR discussion up to RAN1 progress.

Proposal 2	The network can configure a UE with multiple SR configurations to distinguish the “numerology/TTI type” for each LCH/LCG.

Proposal 3	The UE uses the SR configuration associated with the LCH/LCG that triggers the SR.

Proposal 4	Assume a grant carries the information of the “numerology/TTI type”

# References

R2-1704397, Logical channel prioritisation and multiple numerologies, Ericsson