3GPP TSG-RAN WG2 #98	Tdoc R2-1704376

Hangzhou, P.R. of China, 15th – 19th May 2017

Agenda Item:	10.3.1.5

Source:	Ericsson

Title:	BSR triggering aspects

Document for:	Discussion, Decision

# Introduction

Uplink scheduling is a key functionality to meet a broad range of use cases including enhanced mobile broadband, massive MTC, critical MTC, and additional requirements defined during the RAN requirements study. Details to how this functionality can be designed, compared to the baseline framework of LTE is discussed in contributions. We have prepared two contributions focusing on different aspects. This contribution discusses when to trigger BSR. Another contribution discusses what information fields should be carried by SR and BSR [1].

# Discussion

## BSR triggers

In LTE, BSR is triggered in the below example cases.

- UE has UL data to transmit (Regular BSR)

  - UL data for a logical channel which belongs to a LCG becomes available for transmission.

  - Data becomes available for a logical channel with higher priority than current ones for which data is already available for transmission.

  - no data available for transmission for any of the logical channels which belong to a LCG

- UL Grant results in padding is added to the MAC PDU (Padding BSR).

- retxBSR-Timer is expired and UE has some data to transmit (Regular BSR)

- periodicBSR-Timer is expired (Periodic BSR)

We think some of the above triggers trigger BSR too seldom. For instance, new data with the same or lower priority compared to currently transmitted data will not trigger a BSR. This may result in a status mismatch between UE and eNB. The retxBSR-Timer is designed to compensate the loss of a BSR transmission.  The periodicBSR-Timer is used for a UE to periodically update its status to the network. To avoid redundant BSR transmissions, these timers are typically set with relatively long durations. This may lead to an additional latency for some UEs or redundant information transmitted if shorter timer values are configured.

There may be buffer status mismatch between UE and eNB based on current LTE BSR triggers.

BSR triggers should be enhanced to address inefficiencies.

The concept of a logical channel group (LCG) was included in LTE to reduce the BSR signalling overhead in case a UE have many radio bearers established simultaneously. Currently up to 4 LCGs can be supported per UE. The grouping of radio bearers (or logical channels) is established at radio bearer setup by the gNB based on QoS attributes of the radio bearers such as QoS Class Identifier (QCI). It has been agreed that NR reuses LTE BSR framework as the baseline. In other words, NR BSR is also triggered per LCG. Since each logical channel can be mapped to a separate numerology/TTI duration, we should aim to achieve a better granularity for BSR report under the framework of LCG, which was also agreed at RAN2#97bis. In order to do this, the maximum number of LCGs per UE must be increased accordingly. In this way, fewer LCHs can be included in a LCG. Therefore, more accurate buffer status report is achievable.

Increase the maximum number of LCGs per UE for better granularity of BSR report.

With more LCGs, a long BSR as like LTE (carries up to 4 LCGs) may not be able to accommodate all LCGs that trigger BSR. There are several alternatives to address this. The first option is to extend the maximum number of LCGs that can be included in a single BSR message. In this option, the long BSR has a fixed size. The second option is to introduce a variable-size BSR. Its actual size would depend on the actual number of LCGs that are reported. A length indicator may be necessary to indicate the total length of the BSR. The third option is to keep the same long BSR format as like LTE, however, the UE is allowed to send several BSRs at the same time.

Update the BSR format to accommodate more LCGs (e.g. introduce a variable-size BSR and/or a Long BSR longer than in LTE).

In current LTE, the Periodic BSR is used to periodically update the scheduler knowledge on the UE buffer status for durations where no regular BSR is triggered. For instace, it has the advantage to resolve some deadlock situations when new data arrive after a BSR has been transmitted to the eNB.

In NR, it would be beneficial if another trigger of a BSR could be defined, so that the buffer status update is more timely delivered, while not contributing to the transmission of redundant information when not needed.

In defining a new trigger, it would thus be of value if the trigger is based on events which relate to the amount of (in the BSR) reported data served by the gNB scheduler, and the inflow of new data to the UE buffer.

The triggered BSR will in this case result in that a BSR is not transmitted at a configured interval, but rather continously depending on the actual change in UE buffer. As such, the events and criteria for the BSR trigger should be configurable by RRC signalling.

Similar to the Periodic BSR, a BSR triggered by this event should not trigger an SR.

Below is one example on how this new BSR trigger could be defined:

- The UE maintains a variable of reported buffer status, “reported_volume”, when a BSR has been sent

- The UE also maintains a variable, “unreported_volume”, set to the amount of new data not considered in the reported BSR (reported_volume)

- The UE evaluates in each TTI the amount of remaining data of the “reported volume”, to a threshold configured by the NW

- When the remaining data of the reported volume is less than the threshold, and the amount of new data (unreported_volume) is not zero, a BSR is triggered.

Define a new BSR trigger which is related to the amount of reported data served by the gNB scheduler, and the inflow of new data to the UE buffer.

# Conclusion

In section 2 we made the following observations:

Observation 1	There may be buffer status mismatch between UE and eNB based on current LTE BSR triggers.

Based on the discussion in Section 2 we propose the following:

Proposal 1	BSR triggers should be enhanced to address inefficiencies.

Proposal 2	Increase the maximum number of LCGs per UE for better granularity of BSR report.

Proposal 3	Update the BSR format to accommodate more LCGs (e.g. introduce a variable-size BSR and/or a Long BSR longer than in LTE).

Proposal 4	Define a new BSR trigger which is related to the amount of reported data served by the gNB scheduler, and the inflow of new data to the UE buffer.

# References

R2-1704375, SR signaling content in NR Ericsson